3GPP TSG-SA4#112e Meeting 	S4aM200631

March 17th  2021

Source: 	Tencent

Title: 	Draft CR updates on Multiple Overlay Configuration

Document for	Agreement

Agenda item: 	4.5

# Introduction:

This document proposes additional text for the draft CR. The below updates are proposed to be added in section X.6.4.3.3 of the draft CR.

First Change

X.6.4.3.4 Multiple Overlay Configuration

An ITT4RT client supporting the 3gpp_overlay attribute to configure a sphere-relative overlay or viewport-relative overlay shall additionally include the following parameter for indicating the support for multiple overlays:

- multiple_overlay_flag: Indicates if the ITT4RT-Rx client is allowed to use multiple overlays. When set to 1, the client may stream more than one overlay to be displayed on the 360-degree video. The default value is 1.

- overlay_overlapp_flag: Indicates if the ITT4RT-Rx client is allowed to overlap overlays from other ITT4RT-Rx clients onto the overlays shared by the ITT4RT-Tx client of the 360-degree video. If set to 1, the Rx client may overlap overlays. The  default value is 1.

X.6.4.3.5 Overlay info parameter

The parameter overlay_info is a bit field consisting of flags describing the type of overlay interactivity recommended:

overlay_info= ‘overlay_info=’b4b3b2b1b0

-	b0 = ‘0’: changing the position of the overlay is not recommended, overlay position is fixed
b0 = ‘1’: changing the position of the overlay is allowed, ITT4RT-Rx client may change position

-	b1 = ‘0’: switching the overlay on/off is not recommended
b1 = ‘1’: switching the overlay on/off is allowed, ITT4RT-Rx client may switch overlay off

-	b2 = ‘0’: rotating the overlay is not recommended 
b2 = ‘1’: rotating the overlay is allowed

- 	b3 = ‘0’: resizing the overlay is not recommended 
b3 = ‘1’: resizing the overlay is allowed

- 	b4 = ‘0’: changing the opacity of the overlay is not recommended
b4 = ‘1’: changing the opacity of the overlay is allowed

End of Changes

# Proposal

The proposal is to move the agreed changes to the draft CR