3GPP TSG-SA4 Meeting #-RTC SWG AH	S4aR250138

# Paris, France, 3rd Sep 2025 - 5th Sep 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.822 | CR | 0006 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Evaluation of Scenarios for handling N6-unmarked PDU's |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5G_RTP_Ph2, 5G_RTP_Ph2 |  |  |  |  |  | Date: |  |  | 2025-08-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Evaluation of different scenarios in TR 26.822 is missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adds a clause evaluating the different scenarios for N6 unmarked/lone PDU handling |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing insights on the different cases for lone/unmarked PDU handling |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.Y (new clause), 7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**CHANGE**

## 6.Y	Solution #Y: Evaluation of Scenarios for handling N6-unmarked (lone) PDU's

### 6.2.1	Key Issue mapping

This solution intends to give an evaluation of scenarios on the KI#2: QoS handling requirements for lone PDUs. It looks more at the bandwidth and reliability requirements in typical service scenarios.

### 6.2.2	Description

Clause 6.2 (solution #2) provides a gap analysis of cases with N6 unmarked/lone PDU’s. It details how and when such cases can occur in practice.

A common case is multiplexing of RTP with RTCP, audio and/or video, multiplexing re-transmissions.

Another case is the multiplexing of specific control packets such as STUN packet on the same port for a specific purpose of NAT traversal and obtaining public IP address.

This solution looks at typical scenarios involving lone/unmarked N6 PDU’s and the respective bandwidth and reliability requirements. The discussion of the multiplexing solution is handled in key issue #9 and the related solutions which focuses more on how to apply the marking in such cases.

In this solution we evaluate common unmarked/lone PDU scenarios and the corresponding bandwidth and reliability requirements.

Common identified scenarios are the following:

1.  RTP/RTCP multiplex with unmarked RTCP packets. In this case packets are typically limited up to 5 percent of the total bandwidth consumed in the session. RTCP packets are generally important for the service to transmit critical control information and RTCP packets are preferably not discarded at NG-RAN.

2.	RTP video/audio multiplex in a typical video XR call or IPTV stream: generally audio only comprises 10-20 percent of the total bandwidth, and reports often show that for the overall quality of experience audio is quite important, therefore, it make sense to assign a higher priority to audio packets.

3.	Unmarked video and multiplexed video: in some cases redundant video streams may be sent (e.g. as a backup) to enabling fast tune in and other use cases (channel zapping). In some cases the multiplexed video is unmarked. However, the guidelines in TS 26.522 in clause 4.2.6.2.2 and 4.2.6.2.3 provide ways of deriving the marking, and such packets can be marked in a meaningful way at the UPF.

4.	STUN packets (Session traversal utilities for NAT) are packets that can be sent on the same IP tuple, but are used to identify the public IP address of a sender behind the NAT. Generally the STUN packets are sparse compared to the other packets but critical for the service.

5.  A multiplexed re-transmission streams (4588 [31]). RTP retransmission can be supported on clients in a separate RTP stream. Typically RTP re-transmission are sparse compared to the main or original stream but they can be critical for the service quality and should be handled with priority.

6.	AL-FEC stream multiplex. An RTP session may include an AL-FEC RTP stream containing repair packets. Currently there are no guidelines for PDU set marking and assigning PDU Set importance to AL-FEC packets, but there is no real reason to leave AL-FEC packets unmarked, as dropping some AL-FEC packets may improve the overall service performance and network performance.

### 6.2.3	Conclusion

Based on the evaluation in the above, it is proposed to make the following conclusions.

-	Many scenarios for unmarked PDU’s exist

-	Typically the unmarked PDU data is sparse compared to the marked data up to 20 percent maximum

-	In most observed scenarios the reliability is rather important and NG-RAN discards should be avoided

-     An unmarked AL-FEC RTP stream could be an exception, on the other hand there is no reason not to also mark an AL-FEC RTP stream

-	Normative work on guidelines for PDU Set marking should take these aspects into account and prioritize the more common cases.

**CHANGE**

## 7.1	Key issues progress

Table 7.1-1: Summary of key issue progress

| Key Issue # | Short Description | Progress | Objectives met | Normative work proposed | Convergence possible? | Dependencies |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | PSSize accuracy | Solutions #4, #7 and #23 | Partially | Yes #4, Yes #7, Yes #23 | #4 and 7 are complementary solutions #23 is not complementary | #7 depends on SA2 #4 is fully contained in SA4 #23 has dependency on RAN feedback |
| 2 | Lone PDU | Solutions #2, #15, #20 and #21 #Y | Partially | No #2, Yes #15 Yes #20 Yes #Y | #2 provides gap analysis. Complementary solutions in #15 and #20 | SA2 |
| 3 | FEC support | Solutions #5, #17 and #18 | Yes | Yes #5, Yes #17, Yes #18 | Complementary solutions FEC schemes, congestion control and FEC awareness | None |
| 4 | FEC awareness | Solutions #8, #10, #17, #18, #19, #27 | Yes | Yes #8 Yes #10 Yes #17 Yes #18 Yes #19 | Complementary solutions | RAN2, SA2 |
| 5 | RTP transport for XR | Not progressed | - | - | - | - |
| 6 | Encryption | Solution #3 | Yes | Yes (but limited) | Single solution | None |
| 7 | RTCP messages for 5G | Solution #14 | Yes | No | Single solution | None |
| 8 | RTP retransmission | Solutions #9 and #11 | Yes | Yes #9, Yes #11 | Complementary solutions | SA2, RAN2 |
| 9 | RTP Multiplexing | Solutions #12 #22 | Yes | Yes #12 and #22 | Complementary solutions | SA2 |
| 10 | Use cases | Not progressed | - | - | - | - |
| 11 | Enhancements to PDU Set HE | Not progressed | - | - | - | - |
| 12 | Data Burst marking | Solutions #6, #13, #16, #24, #25, #26, #28 | Yes | Yes #6, Yes #16, #13: Reinforces the normative work proposed in #6 and #16, #24: No #25: Yes #26: Yes #28: No | Some solutions are complementary and convergence needed for others, possible way forward proposed in clause 7.13. | SA2, RAN2 |
| 13 | Applicability of PDU Set HE | Solution #1 | Partially | Yes #1 | Single solution | SA2, RAN2 |
| 14 | QoS flow multiplex | Solution #12 | Partially | Yes | Single solution | SA2 |
| 15 | Media and metadata | Not progressed | - | - | - | - |

** END OF CHANGES**