Agenda item: 	7

Source: 	Qualcomm Inc.

Title: 	[AvCall-MED] Avatar Negotiation and Signaling

Document for	Discussion and Agreement

# Introduction

In this contribution, we provide specification text for the signaling and negotiation of Avatars using the SDP for establishing an Avatar application data channel, and a simple negotiation protocol over the established Avatar application DC.

# Proposed Changes

## 7.4 	SDP and DC Negotiation and Signaling of Avatars

### 7.4.1	General

The SDP is used for establishing the Avatar negotiation application data channel (ADC) between AR-MTSI and the remote peer (MF or AR-MTSI client). The Avatar negotiation itself is performed over that ADC using UTF-8 JSON messages carried in the TS 26.264 metadata envelope and URN naming model. The procedures apply to two-party and MF-assisted sessions and preserve the media and data channel architecture of IBACS.

### 7.4.2 	SDP signalling

### 7.4.2.1 Establishment of ADC for avatar negotiation

---For avatar negotiation over an established IMS application data channel, the data channel sub-protocol shall be identified as “3gpp-avatar-negotiation”--.

The channel shall be ordered and reliable-.

### 7.4.3	Avatar negotiation over ADC

### 7.4.3.1 Avatar negotiation protocol

Messages sent over the ADC for avatar negotiation shall use the AR metadata message format defined in clause 6.2.

The message type for avatar negotiation messages shall be set to "urn:3gpp:avatar:v1:negotiation".

Table X.1 payload format of the avatar negotiation message type

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| subtype | number | 1..1 | An identifier of the subtype of the message payload. The allowed values are listed in Table X.2 |
| content | object | 1..1 | A URN that identifies the message type. |

Table X.2 defines all supported sub-types of the avatar negotiation message type:

| Value | Message Name |
| --- | --- |
| 1 | REQUEST |
| 2 | ACCEPT |
| 3 | REJECT |
| 4 | ACK |

The REQUEST is sent by the UE or by the MF when acting as the negotiation coordinator.

The ACCEPT and REJECT messages are sent by the UE or MF to indicate that the received REQUEST is accepted or rejected.

The ACK is issued by the DC-AS or MF when it decides to perform animation and rendering on behalf of the receiver.---

-

### 7.4.3.2 Message Formats

-

The following tables describe the message formats, exchanged for avatar negotiation.

Table 1 — REQUEST payload

| Name | Type |  |  | Cardinality |  |  | Description |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| avatarProfile | string |  |  | 1..1 |  |  | Indicates the avatar profile as defined in clause 5.6. [Allowed values: "2d" or "3d". The value shall match the ARF profile of the selected Avatar representation. ] |  |  |
| arfContainer |  | object |  |  |  | 1..1 |  |  | Reference to ARF container of the Avatar representation that is offered for the call. The container shall be retrievable from the BAR. |
| animationSourceData | array(URI) |  |  | 1..1 |  |  | Provides a list of supported animation data for the avatar. The following values are allowed: - A subset of the AnimationFrameworks as declared in the ARF container of the base avatar - Audio with the URN "urn:3gpp:avatar:v1:animation:audio" - Video with the URN "urn:3gpp:avatar:v1:animation:video" At least one animation source data shall be included. |  |  |
| renderingLocation | array(string) |  |  | 1..1 |  |  | Indicates the possible location(s) for animating and rendering of the offered avatar. The possible values are: “sender”, “receiver” or “mf”. An MF may add “mf” in the offered list if it is capable of animating and rendering the offered avatar. |  |  |

Table 1 — Offer payload

Table 2 — ACCEPT payload

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| renderingLocation | string | 1..1 | Indicates the selected location for the rendering and animation of the offered avatar. If the location is set to “mf”, it has to be confirmed by the transmission of an ACK message from the MF to the receiver. |
| animationSourceData | array(URI) | 1..1 | Includes the selected subset of the animation frameworks that the receiver prefers to receive for performing the avatar animation. If an audio or a video source is used, the receiver or MF shall use one of the audio or video stream from the IMS session to animate the avatar. |

Table 3 — ACK payload

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| selectedRenderingLocation | string | 1..1 | Indicates the selected rendering location as a final decision. |

|  |  |  | -- |
| --- | --- | --- | --- |

Table 5 — REJECT payload

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| code | string | 1..1 | Machine-readable error code. NOTE: If the REJECT message needs to be associated with a particular REQUEST is FFS. NOTE: Definition of Error codes is FFS. |
| detail | string | 0..1 | Human-readable diagnostic text. |

Table 6 — Component: arfContainer

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| uri | string | 1..1 | Authorization-bound reference to the base avatar in BAR or an operator repository. Typical schemes: bar://… or https://…. |
| auth | object | 0..1 | Ephemeral authorization for dereferencing URI (e.g., bearer token); present when required by BAR policy. |

Table 7 — Sub-component: arfContainer.auth

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| scheme | string | 1..1 | Authentication scheme; "bearer" is supported. NOTE: Other HTTP defined authentication schemes can be added and is FFS. |
| token | string | 1..1 | Opaque credential used to access and download the ARF container. |
| expiresAt | number | 0..1 | Expiry time in milliseconds since epoch for the token. |

|  |  |  | - |
| --- | --- | --- | --- |

|  |  |  | -- |
| --- | --- | --- | --- |

### [

-

-

### 7.4.3.2 JSON Schemas

The following schemas are authoritative. They follow the envelope format and constrain the negotiation payloads. An implementation shall validate outgoing and incoming messages against these schemas prior to state transitions.

The following table provides the JSON schema for the REQUEST payload.

{ "$schema": "https://json-schema.org/draft/2020-12/schema", "$id": "urn:3gpp:avatar:negotiation", "type": "object", "properties": { "avatarProfile": { "type": "string", "enum": ["2d", "3d"], "description": "Indicates the avatar profile as defined in clause 5.6. The value shall match the ARF profile of the referenced base avatar." }, "arfContainer": { "type": "object", "description": "Reference to the ARF representation that is offered for the call. The container shall be retrievable from the BAR or an operator URL." }, "animationSourceFramework": { "type": "array", "items": { "type": "string", "format": "uri" }, "minItems": 1, "description": "Provides a list of supported Animation Frameworks that may be used by the sender to generate the animation streams. The list shall contain a subset of the supportedAnimationFrameworks as declared in the ARF container of the base avatar. Additional 3GPP-defined animation source frameworks include: Audio (urn:3gpp:avatar:animation:audio) and Video (urn:3gpp:avatar:animation:video) which indicate that the receiver/MF shall use a referenced audio or video stream from the IMS session to animate the avatar." } }, "required": ["avatarProfile", "arfContainer", "animationSourceFramework"] }

The JSON schema for the payload of the messages is provided in the following table.

The schema for the message bindings is defined as follows:

# ]

-

--

-

# Proposal

We propose to agree the contents of section 2 into the base CR to define the avatar offer and negotiation procedures.