3GPP SA4 Video SWG Telco on FS_5GVideo	S4aV200548

29th September 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.955 | CR | <CR#> | rev | - | Current version: | 0.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SDR Test Material Selection for 4KTV Scenario |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ATEME, InterDigital, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5GVideo |  |  |  |  |  | Date: |  |  | 2020-04-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | No test condtions are currently selected for 4KTV material |  |  |  |  |  |  |  |  |
| Summary of change: |  | This contribution introduces SDR test material selection for 4K-TV scenario. There are three changes: - Changes in Section 2 to add missing reference to ITU-T P.910 recommandation. - Changes in Section 6.3.7 referencing the abovementioned selected video sequences. - Changes in Annex C to indicate the candidate test material and how the selection was performed for 4K-TV scenario. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No test material for 4KTV scenario |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

===== CHANGE 1 =====

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia telephony; Media handling and interaction".

[3]	3GPP TS 26.116: "Television (TV) over 3GPP services; Video profiles".

[4]	3GPP TS 26.118: "3GPP Virtual reality profiles for streaming applications".

[5]	3GPP TR 26.925: "Typical traffic characteristics of media services on 3GPP networks".

[6]	3GPP TR 26.928: "Extended Reality over 5G".

[7]	Recommendation ITU-T H.264 (06/2017): "Advanced video coding for generic audiovisual services" | ISO/IEC 14496-10:2014/Amd.3 2016: "Information technology - Coding of audio-visual objects - Part 10: Advanced Video Coding".

[8]	Recommendation ITU-T H.265 (11/2019): "High efficiency video coding" | ISO/IEC 23008-2:2020: "High Efficiency Coding and Media Delivery in Heterogeneous Environments - Part 2: High Efficiency Video Coding".

[9]	2020 Mobile Internet Phenomena Report, accessible here: https://www.sandvine.com/download-report-mobile-internet-phenomena-report-2020-sandvine, February 2020.

[10]	2019 Ericsson Mobility Report, accessible here: https://www.ericsson.com/4acd7e/assets/local/mobility-report/documents/2019/emr-november-2019.pdf, November 2019.

[11]	T. Fautier, "New Codecs for 5G", DASH-IF Workshop on "Media Streaming meets 5G", December 2019, accessible here: https://dashif.org/docs/workshop-2019/04-thierry%20fautier%20-%20Harmonic%20Codec%20Comparison%205G%20Media%20Workshop_Final%20v3.pdf

[12]	Bitmovin Video Developer Report, accessible here: https://go.bitmovin.com/video-developer-report-2019, September 2019.

[13]	3GPP TS 26.511: "5G Media Streaming (5GMS); Profiles, codecs and formats".

[14]	Recommendation ITU-R BT.709-6 (06/2015): "Parameter values for the HDTV standards for production and international programme exchange".

[15]	Recommendation ITU-R BT.2020-2 (10/2015): "Parameter values for ultra-high definition television systems for production and international programme exchange".

[16]	Recommendation ITU-R BT.2100-1 (06/2017): "Image parameter values for high dynamic range television for use in production and international programme exchange".

[17]	“You can watch Netflix on any screen you want, but you’re probably watching it on a TV”, https://www.vox.com/2018/3/7/17094610/netflix-70-percent-tv-viewing-statistics

[18]	YouTube Revenue and Usage Statistics (2020), https://www.businessofapps.com/data/youtube-statistics/

[19]	Ultra HD Forum service tracker B2C, https://ultrahdforum.org/uhd-service-tracker/“

[20]	HIS Market “4K-TV and UHD: the whole picture”, https://cdn.ihs.com/www/pdf/4ktv-uhd-ebook.pdf

[21]	A. Mercat, M. Viitanen, and J. Vanne, “UVG dataset: 50/120fps 4K sequences for video codec analysis and development,” Accepted to ACM Multimedia Syst. Conf., Istanbul, Turkey, June 2020.

[21]	R. Jullian, Y. Chen, F. Galpin, E. François, M. Kerdranvat (InterDigital), “Extra results to JVET-N605 “Comparative study of video coding solutions VVC, AV1 and EVC versus HEVC””, document JVET-O0898, 15th Meeting: Gothenburg, SE, 3–12 July 2019.

[22]	R. Jullian, Y. Chen, F. Galpin, E. François, M. Kerdranvat (InterDigital), “Comparative study of video coding solutions VVC, AV1 and EVC versus HEVC”, document JVET-N0605, 14th Meeting: Geneva, CH, 19–27 March 2019.

[23]	S. Iwamura, S. Nemoto, A. Ichigaya, “[AHG13] Compression performance analysis for 4K and 8K HLG test sequences”, document JVET-N0828, 14th Meeting: Geneva, CH, 19–27 March 2019.

[24]	A. Segall, E. François, W. Husak, S. Iwamura, D. Rusanovskyy, “JVET common test conditions and evaluation procedures for HDR/WCG video,” document JVET-P2011, 16th JVET meeting: Geneva, CH, 1–11 Oct. 2019.

[25]	Nick Zarzycki, "The Complete Guide to Social Media Video Specs in 2018", July 23, 2018, https://blog.hootsuite.com/social-media-video-specs/

[26]		YouTube Help, "Recommended upload encoding settings", https://support.google.com/youtube/answer/1722171?hl=en

[27]	Facebook Help Center, "What are the video format guidelines for live streaming on Facebook?", https://www.facebook.com/help/1534561009906955

[28]	Deepak Kumar, "All about TikTok video size [ Full Guide]-2020", February 2020, https://tiktoktip.com/tiktok-size/

[29]	Snapchat business center, "Single Image or Video Specifications", https://businesshelp.snapchat.com/en-US/article/top-snap-specs

[30]	ISO/IEC 23000-19: "Information Technology Multimedia Application Format (MPEG-A) – Part 19: Common Media Application Format (CMAF) for segmented media".

[31]	GSMA White Paper, "Chinese operators make major RCS commitment: Whitepaper", April 9, 2020, https://www.gsma.com/futurenetworks/latest-news/china-operators-make-major-rcs-commitment-whitepaper/

[32]	3GPP TS 26.140, "Multimedia Messaging Service (MMS); Media formats and codecs".

[33]	GSMA RCC.71, "RCS Universal Profile Service Definition Document", version 2.4, October 2019.

[34]	GSMA NG.114, "IMS Profile for Voice, Video and Messaging over 5GS", February 2020.

[35]		3GPP TS 26.223, "Telepresence using the IP Multimedia Subsystem (IMS); Media handling and interaction"

[36]		"NVIDIA VIDEO CODEC SDK", https://developer.nvidia.com/nvidia-video-codec-sdk

[37]		Microsoft Online Documentation, "Real-time media calls and meetings with Microsoft Teams", https://docs.microsoft.com/en-us/microsoftteams/platform/bots/calls-and-meetings/real-time-media-concepts

[38]	Recommendation ITU-T H.266 (07/2020): "Versatile Video Coding" | ISO/IEC 23090-3:2020: "Information technology — Coded representation of immersive media — Part 3: Versatile video coding".

[39]	Recommendation ITU-T H.274 (07/2020): "Versatile supplemental enhancement information for coded video bitstreams" | ISO/IEC 23002-7:2020 "Information technology — MPEG video technologies — Part 7: Supplemental enhancement information messages for coded video bitstreams".

[40]	Recommendation ITU-T P.910 (04/2008): "Subjective video quality assessment methods for multimedia applications".

[41]	Ultra HD Forum Guidelines, https://ultrahdforum.org/wp-content/uploads/UHD-Guidelines-V2.3-final.pdf

===== End of CHANGE 1 =====

===== CHANGE 2 =====

### 6.3.4	Encoding and Decoding Constraints

Table 6.3-2 provides an overview of encoding and decoding constraints for 4K-TV category using legacy codec HEVC. This will support the definition of detailed test conditions. It is noted that no relevant profiles exist in TS26.116 and TS26.511 for HFR 4K-TV content.

Table 6.3-2 Encoding and Decoding Configurations for 4K-TV with legacy HEVC codec

| Encoding and Decoding Constraints | H.265/HEVC 4K-TV | H.265/HEVC 4K-TV HFR |
| --- | --- | --- |
| Relevant Codec and Codec Profile/Levels according to TS26.116 and TS26.511. | H.265/HEVC Main-10 Profile Level 5.1 [8] | No relevant 3GPP profiles, should be aligned with H.265/HEVC Main-10 Profile Level 5.2 [8] |
| RAP period | 3.84sec, 1sec | 3.84sec, 1sec |
| Bit rate parameters (CBR, VBR, CAE, HRD parameters) | B = {10,20,30,40} Mbps [41] CBR and capped-VBR | B = {10,20,30,40} Mbps [41] CBR and capped-VBR |
| Latency requirements and specific encoding settings | No latency requirements beyond RAP so picture reordering allowed | No latency requirements beyond RAP so picture reordering allowed |
| Encoding complexity context | real-time encoding, cloud-based encoding, offline encoding, etc. | real-time encoding, cloud-based encoding, offline encoding, etc. |
| Required decoding capabilities | H.265/HEVC Main-10 Profile Level 5.1 [8] | H.265/HEVC Main-10 Profile Level 5.2 [8] |

### 6.3.5	Performance Metrics

Performance is assessed using BD-Rate computation, with PSNR, SSIM and VMAF metrics as objective quality criterion. Regarding complexity considerations, encoding/decoding runtime is provided.

### 6.3.6	Interoperability Considerations

In order to use a codec in the context of 5G Media Streaming services in 3GPP TS 26.511 [13] and for TV Video profiles in 3GPP TS 26.116 [3], the same considerations for interoperability as for FullHD according to clause 6.2.6 apply.

For additional details, please refer to 3GPP TS 26.116 [3] and 3GPP TS 26.511 [13].

### 6.3.7	Test Sequences

The Annex C.3 describes in details the selection process conducted to build the test sequences considered for this scenario. Two subsets are considered for the purpose of codec characterization:

- Class 4K-SDR-1: challenging from an encoding perspective

- Class 4K-SDR-2: easy from an encoding perspective

The two subsets are detailed in the Table below.

Table 6.3-3 : Selected test material for 4K-TV scenario

| Class | Name | Resolution | Frame-rate | ColorGamut | Number of Frames / Duration | Thumbnail |
| --- | --- | --- | --- | --- | --- | --- |
| 4K-SDR-1 | 4EVER_Brest_Sedof | 3840 x 2160 | 60 | BT.2020 | 600 | tbd |
|  | 4EVER_Paris_Montmartre | 3840 x 2160 | 60 | BT.2020 | 600 | tbd |
|  | Netflix_WindAndNature | 4096 x 2160 | 59.94 | BT.709 | 1199 | tbd |
|  | FlowerKids | 3840 x 2160 | 50 | BT.709 | 12s | tbd |
|  | ParkJoy | 3840 x 2160 | 50 | BT.709 | 500 | tbd |
|  | Netflix_TunnelFlag | 4096 x 2160 | 59.94 | BT.709 | 600 | tbd |
| 4K-SDR-2 | Netflix_TunnelFlag | 4096 x 2160 | 59.94 | BT.709 | 600 | tbd |
|  | DucksTakeOff | 3840 x 2160 | 50 | BT.709 | 500 | tbd |
|  | Daylightroad2 | 3840 x 2160 | 60 | BT.709 | 300 | tbd |
|  | FlowerFocus | 3840 x 2160 | 50 | BT.709 | 12s | tbd |
|  | Netflix_DinnerScene | 4096 x 2160 | 59.94 | BT.709 | 1199 | tbd |
|  | CityAlley | 3840 x 2160 | 50 | BT.709 | 12s | tbd |

### 6.3.8	Detailed Test Conditions

#### 6.3.8.1	Overview

First, the legacy codec HEVC is tested to assess the relevance of what’s already in the 3GPP specification for this particular 4K-TV scenario. For this first test, two HEVC implementations are compared (x265 and reference software HM), according to encoding constraints derived from Table 6.3-2. For x265, encoding presets are selected to cover the desired encoding complexity contexts, for live and offline encoding. The tested rate-control modes are CBR and capped-VBR to fit with the possible delivery methods (single or multiple services inside a resource). In this first test, the HEVC reference implementation HM is evaluated with CBR rate-control on, at coding-tree-unit (CTU) granularity. In addition, fixed QP encoding is also carried out to provide additional anchor points aligned with formal MPEG/JVET common test conditions (potentially for future comparison with other codecs, if needed).

===== End of CHANGE 2 =====

===== CHANGE 3 =====

## C.3		4K-TV Test Sequences

### C.3.1 	SDR Category

#### C.3.1.1	Candidate Test Sequences

#### In this Annex, a wide range of  SDR test material is reported. The candidate sequences are extracted from test material used in video processing community. The following sections describe the test material in terms of format characteristics and copyright.

#### C.3.1.1.1	Ultra Video Group Sequences

The Ultra Video Group (UVG) sequences are publicly available, and can be used under CC BY-NC 3.0 license. The sequences are available for download at https://github.com/ultravideo/UVG-4K-Dataset and an overview is provided in the table below.

Table C.3-1 : UVG test material description

| Name | Resolution |  | Frame-rate |  | ColorGamut |  | duration |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CityAlley | 3840 x 2160 |  | 50 |  | BT.709 |  | 12s |  |
| FlowerFocus | 3840 x 2160 |  | 50 |  | BT.709 |  | 12s |  |
| FlowerKids | 3840 x 2160 |  | 50 |  | BT.709 |  | 12s |  |
| FlowerPan | 3840 x 2160 |  | 50 |  | BT.709 |  | 12s |  |
| RaceNight | 3840 x 2160 |  | 50 |  | BT.709 |  | 12s |  |
| RiverBank | 3840 x 2160 | 50 | BT.709 | 12s |
| SunBath | 3840 x 2160 |  | 50 |  | BT.709 |  | 6s |  |
| Twilight | 3840 x 2160 |  | 50 |  | BT.709 |  | 12s |  |

#### C.3.1.1.2	JVET Sequences

The JVET test material for UHD SDR sequences is composed by six sequences described in the table below. The licensing information referenced in the table is described below:

- CC BY-NC-ND 4.0

- Our dataset is available for free and academic research only, no commercial use. Please cite our paper (L. Song, X. Tang, W. Zhang, X. Yang, P. Xia, The SJTU 4K Video Sequence Dataset, the Fifth International Workshop on Quality of Multimedia Experience (QoMEX2013), Klagenfurt, Austria, July 3rd-5th, 2013) in any published work if you use those video sequences.

- Please note that this test material remains the property of the Institute of Research & Technology b<>com and is solely provided “as is" to ITU-T SG16 Q6 and ISO/IEC JTC1/SC29/WG11, on a non-transferable and non-exclusive basis, and solely for the purpose of development, testing and promulgation of video coding standards. Any disclosure to third parties, exhibition, broadcast, distribution, public display or other exploitation of the test material is strictly prohibited.

- The proposed test sequences in this document and all intellectual property rights therein remain the property of Huawei Technologies Co., Ltd., and National Engineering Research Center.

Table C.3-2 : JVET test material description

| Name | Resolution | Frame-rate | ColorGamut | Number of frames | License |
| --- | --- | --- | --- | --- | --- |
| Tango2 | 3840 x 2160 | 60 | BT.709 | 294 | See 1. |
| FoodMarket4 | 3840 x 2160 | 60 | BT.709 | 300 | See 1. |
| Campfire | 3840 x 2160 | 30 | BT.709 | 300 | See 2. |
| Catrobots1 | 3840 x 2160 | 60 | BT.709 | 300 | See 3. |
| Daylightroad2 | 3840 x 2160 | 60 | BT.709 | 300 | See 4. |
| Parkrunning3 | 3840 x 2160 | 50 | BT.709 | 300 | See 4. |

#### C.3.1.1.3	Netflix Sequences

The Netflix dataset is composed by eighteen 4K sequences described in the table below. The sequences can be used under the CC BY-NC-ND 4.0 license restriction.

Table C.3-3 : Netflix test material description

| Name |  | Resolution |  | Frame-rate |  | ColorGamut |  | Number of frames |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Netflix_Aerial |  | 4096 x 2160 |  | 59.94 |  | BT.709 |  | 1199 |
| Netflix_DinnerScene |  | 4096 x 2160 |  | 59.94 |  | BT.709 |  | 1199 |
| Netflix_RollerCoaster |  | 4096 x 2160 |  | 59.94 |  | BT.709 |  | 1199 |
| Netflix_DrivingPOV |  | 4096 x 2160 |  | 59.94 |  | BT.709 |  | 1199 |
| Netflix_BarScene |  | 4096 x 2160 |  | 59.94 |  | BT.709 |  | 1199 |
| Netflix_ToddlerFountain |  | 4096 x 2160 |  | 59.94 |  | BT.709 |  | 1199 |
| Netflix_Dancers |  | 4096 x 2160 |  | 59.94 |  | BT.709 |  | 1199 |
| Netflix_PierSeaside | 4096 x 2160 |  | 59.94 |  | BT.709 |  | 1199 |  |
| Netflix_WindAndNature | 4096 x 2160 |  | 59.94 |  | BT.709 |  | 1199 |  |
| Netflix_Boat | 4096 x 2160 |  | 59.94 |  | BT.709 |  | 300 |  |
| Netflix_FoodMarket | 4096 x 2160 |  | 59.94 |  | BT.709 |  | 600 |  |
| Netflix_Tango | 4096 x 2160 |  | 59.94 |  | BT.709 |  | 294 |  |
| Netflix_BoxingPractice | 4096 x 2160 |  | 59.94 |  | BT.709 |  | 254 |  |
| Netflix_Narrator | 4096 x 2160 |  | 59.94 |  | BT.709 |  | 300 |  |
| Netflix_TunnelFlag | 4096 x 2160 |  | 59.94 |  | BT.709 |  | 600 |  |
| Netflix_RitualDance | 4096 x 2160 |  | 59.94 |  | BT.709 |  | 600 |  |
| Netflix_FoodMarket2 | 4096 x 2160 |  | 59.94 |  | BT.709 |  | 300 |  |
| Netflix_SquareAndTimelapse | 4096 x 2160 |  | 59.94 |  | BT.709 |  | 600 |  |
| Netflix_Aerial | 4096 x 2160 |  | 59.94 |  | BT.709 |  | 1199 |  |
| Netflix_DinnerScene | 4096 x 2160 |  | 59.94 |  | BT.709 |  | 1199 |  |
| Netflix_RollerCoaster | 4096 x 2160 |  | 59.94 |  | BT.709 |  | 1199 |  |

#### C.3.1.1.4	EBU Sequences

The EBU dataset is composed by two sequences described in the table below. The sequences can be used under the CC BY-NC-ND 4.0 license restriction.

Table C.3-4 : EBU test material description

| Name | Resolution | Frame-rate | ColorGamut | Number of frames |
| --- | --- | --- | --- | --- |
| Rain_fruits | 3840 x 2160 | 50 | BT.709 | 600 |
| Lupo_candlelight | 3840 x 2160 | 50 | BT.709 | 600 |

#### C.3.1.1.5	SVT Sequences

The SVT dataset is composed by five sequences described in the table below. The sequences can be used under the license available at https://tech.ebu.ch/webdav/site/tech/shared/hdtv/svt-multiformat-conditions-v10.pdf .

Table C.3-5 : SVT test material description

| Name | Resolution | Frame-rate | ColorGamut | Number of frames |
| --- | --- | --- | --- | --- |
| CrowdRun | 3840 x 2160 | 50 | BT.709 | 500 |
| ParkJoy | 3840 x 2160 | 50 | BT.709 | 500 |
| DucksTakeOff | 3840 x 2160 | 50 | BT.709 | 500 |
| IntoTree | 3840 x 2160 | 50 | BT.709 | 500 |
| OldTownCross | 3840 x 2160 | 50 | BT.709 | 500 |

#### C.3.1.1.6	4ever Sequences

The 4ever dataset is composed by two sequences described in the table below. The sequences can be used under the license described below:

The proposed test sequences Brest_Sedof_3840x2160p60_10b.yuv and Paris_Montmartre_3840x2160p60_10b, and all intellectual property rights therein remain the property of 4EVER consortium partners: Orange, AMP Visual TV, ATEME, France Télévisions, GlobeCast, InterDigital, Highlands Technologies Solutions, INSA Rennes, TeamCast and Télécom ParisTech

  - The following uses are allowed for the proposed sequences:

    - Can be published in technical papers, played at technology research and development events.

    - Can be used in 3GPP SA4.

  - The following uses are not allowed for the proposed sequences:

    - Do not publish snapshots in product brochures.

    - Do not redistribute video with a commercial product.

Table C.3-6 : 4ever test material description

| Name | Resolution | Frame-rate | ColorGamut | Number of frames |
| --- | --- | --- | --- | --- |
| 4EVER_Brest_Sedof | 3840 x 2160 | 60 | BT.2020 | 600 |
| 4EVER_Paris_Montmartre | 3840 x 2160 | 60 | BT.2020 | 600 |

#### C.3.1.1.7	CableLabs Sequences

The CableLabs dataset is composed by eight sequences described in the table below. The sequences can be used under the CC BY-NC-ND 3.0 license restriction.

Table C.3-7 : CableLabs test material description

| Name | Resolution | Frame-rate | ColorGamut | duration | Comment on selected intervals |
| --- | --- | --- | --- | --- | --- |
| Lifting Off | 3840 x 2160 | 23.98 | BT.709 | 3:21 | Nice colored pictures, som burned in text, scene cuts (2) |
| Seconds that count | 3840 x 2160 | 23.98 | BT.709 | 5:22 | Not critical |
| Unspoken Friend | 3840 x 2160 | 23.98 | BT.709 | 4:17 | Not critical |
| Indoor Soccer | 3840 x 2160 | 23.98 | BT.709 | 4:12 | Fast motion, scene cuts (4) |
| Skateboarding | 3840 x 2160 | 23.98 | BT.709 | 4:36 | Not critical |
| Eldorado | 3840 x 2160 | 23.98 | BT.709 | 3:02 | Panning up/down on a tree (oak), no scene cut |
| Moment of Intensity | 3840 x 2160 | 59.94 | BT.709 | 3:10 | Skate board, no scene cuts |
| Ancient Thought | 3840 x 2160 | 23.98 | BT.709 | 3:08 | Not critical |

All of the sequences are not of interest for compression evaluation. A selection of some critical time intervals have been conducted based on experience on compression. Especially intervals with fast or complex motion, some including scene cuts, representative of TV contents have been selected. Static pictures, or pictures with artistic intent with few spatial details have been discarded. To remain in the scope of the scenario, the following sub sequences have been selected as final candidates:

- Lifting off : from 00:05 to 00:22

- Indoor Soccer : from 01:06 to 01:23

- Eldorado : from 01:33 to 01:50

- Life Untouched : from 00:00 to 00:22

The extraction of subpart from original ProRes sequences is realized with FFmpeg release 4.3.1 using the following commandline:

./ffmpeg.exe -i $IN.mov -vcodec rawvideo -ss $START_TIME -t $DURATION -pix_fmt yuv420p10le $OUT.yuv

Where $IN and $OUT are respectively the input and output sequences, $START_TIME and $DURATION respectively the start time and duration in hh:mm:ss format.

#### C.3.1.2	Selected Test Sequences

In order to decide wich sequences should be used in this specification, the following process has been used:

- Run informative encoding using x265 on the candidate sequences

- Run SI-TI metrics computation on the candidate sequences

- Select two diverse datasets based on R-D and SI-TI information.

#### C.3.1.2.1	x265 Encoding

To represent candidate sequences in the R-D plane, fixed QP encodings are carried out for QPs [12,	17,22,27,32,37,42] using ffmpeg release 4.3.1, with the following command line:

./ffmpeg.exe -pix_fmt yuv420p10le -s $Wx$H -framerate $FR -i $IN -vcodec libx265 -qp $QP -preset fast -x265-params “psnr=1:ssim=1” $OUT.mp4

Where $W and $W are respectively the sequence width and height, $FR the sequence frame-rate, $QP the selected quantization parameter. $IN and $OUT are respectively the input and output files.

From the obtained log, the overall bitrate and PSNR are extracted to be used for plotting. The encoding result are presented below, all sequences being represented in the targeted [10-40] Mbps range (red box).

Figure C.3-1 : R-D curves for the UVG dataset

Figure C.3-2 : R-D curves for the JVET dataset

Figure C.3-3 : R-D curves for the Netflix dataset

Figure C.3-4 : R-D curves for the EBU, SVT and 4ever datasets

Figure C.3-5 : R-D curves for the CableLabs dataset

#### C.3.1.2.2	SI-TI Metrics

The analysis is based on spatial perceptual information (SI) and temporal perceptual information (TI) metrics calculated as defined in ITU-P.910 recommendation [40]. In order to reduce the bias introduced by scene-cuts in TI metric computation, the TI at scene-cut boundaries are removed from computation.

Figure C.3-6 : SI-TI for the UVG dataset

Figure C.3-7 : SI-TI for the JVET dataset

Figure C.3-8 : SI-TI for the Netflix dataset

Figure C.3-9 : SI-TI for the EBU, SVT, 4ever and CableLabs dataset

#### To be added later: reference to the SI-TI scripts will be available (ETSI git)

#### C.3.1.2.3	Sequence Selection

The first step consists in classifying the sequences in two categories : easy and challenging. This reflects the complexity of the sequences from an encoding perspective. Based on the R-D data, the QP points covering the application range [10-40]Mbps are identified. For the lower (10Mbps) and higher (40Mbps) boundaries, a linear interpolation is achieved between the two closest QP points to estimate the distortion at the boundaries: D_10 and D_40. If the distortion difference D_DIFF=(D_40-D_10) is less than or equal to a threshold Thr=1.5, then the sequence is classified as easy, otherwise it is classified as challenging. The results of this first classification step is reported in the table below.

Table C.3-8 : Sequence classification based on R-D information in the [10-40] Mbps range, Thr=1.5

|  | Name | D_10 | D_40 | D_DIFF | Complexity |
| --- | --- | --- | --- | --- | --- |
| JVET | Tango2 | 40,33 | 41,44 | 1,12 | Easy |
|  | FoodMarket4 | 40,28 | 42,37 | 2,09 | Challenging |
|  | Campfire | 35,06 | 37,29 | 2,23 | Challenging |
|  | Catrobots1 | 38,36 | 39,85 | 1,50 | Easy |
|  | Daylightroad2 | 37,42 | 38,61 | 1,19 | Easy |
|  | Parkrunning3 | 31,17 | 33,65 | 2,48 | Challenging |
| UVG | CityAlley | 43,37 | 43,49 | 0,12 | Easy |
|  | FlowerFocus | 41,13 | 41,33 | 0,20 | Easy |
|  | FlowerKids | 40,92 | 42,78 | 1,86 | Challenging |
|  | FlowerPan | 39,60 | 39,85 | 0,25 | Easy |
|  | RaceNight | 37,82 | 38,62 | 0,79 | Easy |
|  | RiverBank | 36,62 | 38,84 | 2,22 | Challenging |
|  | SunBath | 43,85 | 46,98 | 3,13 | Challenging |
|  | Twilight | 42,92 | 43,24 | 0,33 | Easy |
| Netflix | Netflix_Aerial | 38,62 | 41,00 | 2,37 | Challenging |
|  | Netflix_DinnerScene | 42,85 | 43,08 | 0,23 | Easy |
|  | Netflix_RollerCoaster | 42,61 | 44,73 | 2,11 | Challenging |
|  | Netflix_DrivingPOV | 37,27 | 39,54 | 2,27 | Challenging |
|  | Netflix_BarScene | 41,39 | 41,62 | 0,23 | Easy |
|  | Netflix_ToddlerFountain | 33,56 | 36,01 | 2,45 | Challenging |
|  | Netflix_Dancers | 41,96 | 42,14 | 0,18 | Easy |
|  | Netflix_PierSeaside | 41,05 | 43,10 | 2,06 | Challenging |
|  | Netflix_WindAndNature | 43,35 | 45,09 | 1,75 | Challenging |
|  | Netflix_Boat | 35,27 | 38,64 | 3,37 | Challenging |
|  | Netflix_FoodMarket | 37,62 | 38,95 | 1,32 | Easy |
|  | Netflix_Tango | 40,31 | 41,27 | 0,97 | Easy |
|  | Netflix_BoxingPractice | 40,97 | 42,14 | 1,17 | Easy |
|  | Netflix_Narrator | 45,33 | 46,36 | 1,03 | Easy |
|  | Netflix_TunnelFlag | 39,47 | 42,06 | 2,59 | Challenging |
|  | Netflix_RitualDance | 40,25 | 43,46 | 3,21 | Challenging |
|  | Netflix_FoodMarket2 | 39,11 | 41,27 | 2,16 | Challenging |
|  | Netflix_SquareAndTimelapse | 39,43 | 42,15 | 2,72 | Challenging |
| EBU | Rain_fruits | 40,04 | 41,69 | 1,65 | Challenging |
|  | Lupo_candlelight | 39,81 | 40,01 | 0,20 | Easy |
| SVT | CrowdRun | 31,41 | 33,94 | 2,53 | Challenging |
|  | ParkJoy | 29,86 | 32,13 | 2,27 | Challenging |
|  | DucksTakeOff | 29,31 | 30,75 | 1,44 | Easy |
|  | IntoTree | 35,06 | 35,45 | 0,39 | Easy |
|  | OldTownCross | 33,93 | 34,06 | 0,13 | Easy |
| 4ever | 4EVER_Brest_Sedof | 28,66 | 31,42 | 2,76 | Challenging |
|  | 4EVER_Paris_Montmartre | 30,33 | 33,19 | 2,86 | Challenging |
| CableLabs | LiftingOff | 45,01 | 48,31 | 3,30 | Challenging |
|  | Soccer | 43,31 | 44,89 | 1,58 | Challenging |
|  | ElDorado | 39,41 | 44,29 | 4,88 | Challenging |
|  | MomentsOfIntensity | 43,09 | 44,66 | 1,57 | Challenging |

It is observed that 26 sequences are classified as “Challenging” while 19 are “Easy”. In order to reduce further the dataset to a reasonable but diverse set of sequences, the next step consists in removing sequences having some similarities, using SI-TI information computed in Section C.3.1.2.2. The sequences are represented below in the SI-TI plane, with highlighted in red the “similar” sequences from an SI-TI perspective.

Figure C.3-9 : SI-TI for the Challenging sequences

Figure C.3-10 : SI-TI for the Easy sequences

Based on the informative clustering illustrated in Figures C.3-9 and C.3-10, the selected sequences for the characterization are the following:

Table C.3-9 : Result of the selection process

| Class |  |  | Name |  |  | Resolution |  |  | Frame-rate |  |  | ColorGamut |  |  | Number of Frames / Duration |  |  | Thumbnail |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Challenging |  | 4EVER_Brest_Sedof |  |  | 3840 x 2160 |  |  | 60 |  |  | BT.2020 |  |  | 600 |  | tbd |  |  |
|  |  | 4EVER_Paris_Montmartre |  |  | 3840 x 2160 |  |  | 60 |  |  | BT.2020 |  |  | 600 |  | tbd |  |  |
|  |  | Netflix_WindAndNature |  |  | 4096 x 2160 |  |  | 59.94 |  |  | BT.709 |  |  | 1199 |  | tbd |  |  |
|  |  |  | FlowerKids |  |  | 3840 x 2160 |  |  | 50 |  |  | BT.709 |  |  | 12s |  |  | tbd |  |
|  | ParkJoy |  |  | 3840 x 2160 |  |  | 50 |  |  | BT.709 |  |  | 500 |  |  |  | tbd |  |
|  |  |  | Netflix_TunnelFlag |  |  | 4096 x 2160 |  |  | 59.94 |  |  | BT.709 |  |  | 600 |  |  | tbd |  |
| Easy |  |  | Netflix_TunnelFlag |  |  | 4096 x 2160 |  |  | 59.94 |  |  | BT.709 |  |  | 600 |  |  | tbd |  |
|  |  |  | DucksTakeOff |  |  | 3840 x 2160 |  |  | 50 |  |  | BT.709 |  |  | 500 |  |  | tbd |  |
|  |  |  | Daylightroad2 |  |  | 3840 x 2160 |  |  | 60 |  |  | BT.709 |  |  | 300 |  |  | tbd |  |
|  |  |  | FlowerFocus |  |  | 3840 x 2160 |  |  | 50 |  |  | BT.709 |  |  | 12s |  |  | tbd |  |
|  |  |  | Netflix_DinnerScene |  |  | 4096 x 2160 |  |  | 59.94 |  |  | BT.709 |  |  | 1199 |  |  | tbd |  |
|  |  |  | CityAlley |  |  | 3840 x 2160 |  |  | 50 |  |  | BT.709 |  |  | 12s |  |  | tbd |  |

### C.3.2 	HDR Category

tbd

### C.3.3 	HFR Category

tbd

===== End of CHANGE 3 =====