SA4-e (AH) Video SWG post 110-e	S4aV200553

# 6th October 2020	revision of AHVIC-289

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.955 | CR | <CR#> | rev | 1 | Current version: | 0.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Test and Characterization Framework for Video Codecs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5GVideo |  |  |  |  |  | Date: |  |  | 2020-10-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5, Annex B |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| 56 |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

=====  CHANGE  =====

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia telephony; Media handling and interaction".

[3]	3GPP TS 26.116: "Television (TV) over 3GPP services; Video profiles".

[4]	3GPP TS 26.118: "3GPP Virtual reality profiles for streaming applications".

[5]	3GPP TR 26.925: "Typical traffic characteristics of media services on 3GPP networks".

[6]	3GPP TR 26.928: "Extended Reality over 5G".

[7]	Recommendation ITU-T H.264 (06/2017): "Advanced video coding for generic audiovisual services" | ISO/IEC 14496-10:2014/Amd.3 2016: "Information technology - Coding of audio-visual objects - Part 10: Advanced Video Coding".

[8]	Recommendation ITU-T H.265 (11/2019): "High efficiency video coding" | ISO/IEC 23008-2:2020: "High Efficiency Coding and Media Delivery in Heterogeneous Environments - Part 2: High Efficiency Video Coding".

[9]	2020 Mobile Internet Phenomena Report, accessible here: https://www.sandvine.com/download-report-mobile-internet-phenomena-report-2020-sandvine, February 2020.

[10]	2019 Ericsson Mobility Report, accessible here: https://www.ericsson.com/4acd7e/assets/local/mobility-report/documents/2019/emr-november-2019.pdf, November 2019.

[11]	T. Fautier, "New Codecs for 5G", DASH-IF Workshop on "Media Streaming meets 5G", December 2019, accessible here: https://dashif.org/docs/workshop-2019/04-thierry%20fautier%20-%20Harmonic%20Codec%20Comparison%205G%20Media%20Workshop_Final%20v3.pdf

[12]	Bitmovin Video Developer Report, accessible here: https://go.bitmovin.com/video-developer-report-2019, September 2019.

[13]	3GPP TS 26.511: "5G Media Streaming (5GMS); Profiles, codecs and formats".

[14]	Recommendation ITU-R BT.709-6 (06/2015): "Parameter values for the HDTV standards for production and international programme exchange".

[15]	Recommendation ITU-R BT.2020-2 (10/2015): "Parameter values for ultra-high definition television systems for production and international programme exchange".

[16]	Recommendation ITU-R BT.2100-1 (06/2017): "Image parameter values for high dynamic range television for use in production and international programme exchange".

[17]	“You can watch Netflix on any screen you want, but you’re probably watching it on a TV”, https://www.vox.com/2018/3/7/17094610/netflix-70-percent-tv-viewing-statistics

[18]	YouTube Revenue and Usage Statistics (2020), https://www.businessofapps.com/data/youtube-statistics/

[19]	Ultra HD Forum service tracker B2C, https://ultrahdforum.org/uhd-service-tracker/“

[20]	HIS Market “4K-TV and UHD: the whole picture”, https://cdn.ihs.com/www/pdf/4ktv-uhd-ebook.pdf

[21]	A. Mercat, M. Viitanen, and J. Vanne, “UVG dataset: 50/120fps 4K sequences for video codec analysis and development,” Accepted to ACM Multimedia Syst. Conf., Istanbul, Turkey, June 2020.

[21]	R. Jullian, Y. Chen, F. Galpin, E. François, M. Kerdranvat (InterDigital), “Extra results to JVET-N605 “Comparative study of video coding solutions VVC, AV1 and EVC versus HEVC””, document JVET-O0898, 15th Meeting: Gothenburg, SE, 3–12 July 2019.

[22]	R. Jullian, Y. Chen, F. Galpin, E. François, M. Kerdranvat (InterDigital), “Comparative study of video coding solutions VVC, AV1 and EVC versus HEVC”, document JVET-N0605, 14th Meeting: Geneva, CH, 19–27 March 2019.

[23]	S. Iwamura, S. Nemoto, A. Ichigaya, “[AHG13] Compression performance analysis for 4K and 8K HLG test sequences”, document JVET-N0828, 14th Meeting: Geneva, CH, 19–27 March 2019.

[24]	A. Segall, E. François, W. Husak, S. Iwamura, D. Rusanovskyy, “JVET common test conditions and evaluation procedures for HDR/WCG video,” document JVET-P2011, 16th JVET meeting: Geneva, CH, 1–11 Oct. 2019.

[25]	Nick Zarzycki, "The Complete Guide to Social Media Video Specs in 2018", July 23, 2018, https://blog.hootsuite.com/social-media-video-specs/

[26]		YouTube Help, "Recommended upload encoding settings", https://support.google.com/youtube/answer/1722171?hl=en

[27]	Facebook Help Center, "What are the video format guidelines for live streaming on Facebook?", https://www.facebook.com/help/1534561009906955

[28]	Deepak Kumar, "All about TikTok video size [ Full Guide]-2020", February 2020, https://tiktoktip.com/tiktok-size/

[29]	Snapchat business center, "Single Image or Video Specifications", https://businesshelp.snapchat.com/en-US/article/top-snap-specs

[30]	ISO/IEC 23000-19: "Information Technology Multimedia Application Format (MPEG-A) – Part 19: Common Media Application Format (CMAF) for segmented media".

[31]	GSMA White Paper, "Chinese operators make major RCS commitment: Whitepaper", April 9, 2020, https://www.gsma.com/futurenetworks/latest-news/china-operators-make-major-rcs-commitment-whitepaper/

[32]	3GPP TS 26.140, "Multimedia Messaging Service (MMS); Media formats and codecs".

[33]	GSMA RCC.71, "RCS Universal Profile Service Definition Document", version 2.4, October 2019.

[34]	GSMA NG.114, "IMS Profile for Voice, Video and Messaging over 5GS", February 2020.

[35]		3GPP TS 26.223, "Telepresence using the IP Multimedia Subsystem (IMS); Media handling and interaction"

[36]		"NVIDIA VIDEO CODEC SDK", https://developer.nvidia.com/nvidia-video-codec-sdk

[37]		Microsoft Online Documentation, "Real-time media calls and meetings with Microsoft Teams", https://docs.microsoft.com/en-us/microsoftteams/platform/bots/calls-and-meetings/real-time-media-concepts

[38]	Recommendation ITU-T H.266 (07/2020): "Versatile Video Coding" | ISO/IEC 23090-3:2020: "Information technology — Coded representation of immersive media — Part 3: Versatile video coding".

[39]	Recommendation ITU-T H.274 (07/2020): "Versatile supplemental enhancement information for coded video bitstreams" | ISO/IEC 23002-7:2020 "Information technology — MPEG video technologies — Part 7: Supplemental enhancement information messages for coded video bitstreams".

[XX]	Technical Paper ITU-T HSTP-VID-WPOM (07/2020): "Working practices using objective metrics for evaluation of video coding efficiency experiments".

[XX]	3GPP TR 26.902, "Video codec performance".

[XX]	3GPP TS 26.244, "Transparent end-to-end packet switched streaming service (PSS); 3GPP file format (3GP)"

=====  CHANGE  =====

# 5	Test and Characterization Framework for Video Codecs

## 5.1	Overview

This clause defines the characterization framework for video codecs for relevant 3GPP scenarios. For this purpose, the following is applied:

- A set of relevant scenarios are defined. The scenarios reflect a typical application for video codecs in 5G Systems and networks. The scenarios are introduced in clause 6 and may be extended in future versions of this document.

- For each scenario, one or several test sequences are

- For each test scenario, one or several anchors are defined and generated. For details on anchors, please refer to clause 5.3. For anchor generation, reference software tools are used as introduced in clause 5.4.

- For each of the anchors, metrics are provided. Metrics are documented in detail in clause 5.5.

- Tests for new codecs can be developed and generated. They are equivalent to anchor generation, but possibly for other codecs. Tests are introduced in clause 5.6.

- Codecs can be characterized against anchors. Characterization is documented in terms of expected bitrate savings for a codec, and may include additional comparison parameters such as complexity increase, etc. The basic characterization is introduced in clause 5.7.

An overview of the anchor generation framework and the anchor metrics is provided in Figure 5.1-1. An integral part of this Technical Report is:

-	Test and anchor sequence formats to store raw video data

-	One or multiple Test Sequences for the different scenarios

-	Reference Software encoders

-	Anchor Configuration files

-	Anchor bitstreams in a well-defined anchor bitstream format

-	A anchor metric computation based on test sequence, anchor bitstream and an anchor sequence.

-	Conforming decoders to generate anchor sequences from an anchor bitstream

-	Anchor metrics

NOTE: as the anchor sequences can be generated by conforming decoders, anchor sequences are not included in this technical report

Figure 5.1-1 Anchor Generation Framework and Anchor Metrics Generation

## 5.2	Test Sequences

<define test sequence principles and format>

We need to agree on raw file formats. I am currently checking with Dave Singer and Jean-Le Feuvre to a raw format based on mp4. We expect to reuse what is in TR26.902 and/or extend. Contribution is planned for next MPEG meeting – no action needed.

[GT] I assume there’s a common agreement in principle to rely on mp4 file format.

[GT] I think it is also possible with ffmpeg to convert a YUV file into an mp4 file using AVC lossless. In this case, quality metrics implemented in ffmpeg could be used easily.

[[TS]] AVC lossless? But does it support 10 bit, HDR and so on?

## [TB] I would prefer to stick to the raw format to avoid any additional encoding/decoding steps in the scripts we will develop. A raw format based on mp4 is fine.

## 5.3	Anchors

Anchors provide a baseline that a tested method can be compared against. Anchors use a codec that exists in an existing 3GPP specification as defined in clause 4.

Anchors are collected in tuples to address different quality and bitrates that can then be used for evaluation over a larger set of operation points.

The following principle apply to anchor definitions

- Each scenario typically has several well-defined anchors

- An anchor is a combination of:

  - Explanation what this anchor is important for

  - Test Sequence

  - Reference Encoder

  - Encoder configuration

  - Encoding complexity estimation, if possible

  - Variable encoder configuration to create multiple quality/bitrate variants (using for example QP variations).

  - Anchor tuples creating multiple variants, each including

    - Anchor bitstream

    - Anchor Metrics

- Anchors and anchor tuples are an integral part of the Technical Report

The generation of anchor tuples is shown in Figure 5.3-1.

## Figure 5.3-1: Anchor Tuple Generation Framework and Anchor Tuple Metrics Generation

## 5.4	Reference Software Tools

### 5.4.1	General

Anchors are generated with a dedicated reference software. The reference software is documented in order to permit repeatability of the anchor generation.

### 5.4.2	H.264/AVC

JM: https://vcgit.hhi.fraunhofer.de/jct-vc/JM

### 5.4.3	H.265/HEVC

### HM: https://vcgit.hhi.fraunhofer.de/jct-vc/HM

## 5.5	Metrics

### 5.5.1	General

### 5.5.2	SDR Metrics

### 5.5.2.1	Peak-Signal-to-Noise Ratio

- Luminance PSNR

### Weighted PSNR (6 times luminance)

### 5.5.2.2	SSIM

#### 5.5.2.3	VMAF

### 5.5.3	HDR Metrics

### tbd

### 5.5.4	Metrics Computation

#### Metrics are defined based on a script.

We develop a script that provide based on the original sequence (preferably in raw Mp4) and the reconstructed sequence (after decoding) a set of metrics including different variants of PSNR, SSIM, VMAF and so on. As I do not wanna write all details in the spec, my preference is that we use scripts and the these scripts are part of the 3GPP report.

For this script:

  - Nils would do all the scripting (C, python) and packaging (docker?)

  - We wanna rely on your experience and input. Can we use FFMPEG? And so on

    - [TB] we’ll need to freeze and fork a given FFmpeg release in the ETSI git for the sake of easy reproducibility of the results (same apply for encoding/decoding).

  - An important issue is that we also add HDR metrics. Dmytro also in CC is willing to help here.

## The script should also include the original sequence to generate the bitrate (to be done based on the track run).

## 5.4	Reference Software Tools5.6	Tests

- A test is developed against an anchor:

  - Test Sequence

  - Codec

  - Reference Encoder

  - Static encoder configuration matching the anchor settings

  - Encoding complexity estimation

  - Variable encoder configuration to create multiple quality/bitrate variants (using for example QP variations)

  - Multiple variants, each including test bitstream, bitrate

    - Several subjective quality metrics: PSNR, SSIM, VMAF, etc.

  - Tests are an integral part of the Technical Report

## Figure 5.6-1: Test Tuple Generation Framework and Test Tuple Metrics Generation

## 5.7	Characterization

- Use BD-rate information for codec characterization

- Document for all relevant metrics for the scenario

  - The BD-rate for each anchor

  - The average BD-rate for all anchors against one codec

=====  CHANGE  =====

Annex B
Details on Performance Metrics

## B.1	Introduction

This Annex provides a detailed overview on performance metrics that are potentially relevant for the evaluation of video codecs as considered and used in this Technical Report.

## B.2	Objective metrics for evaluation of video coding efficiency experiments

The ITU-T Technical Paper [40] provides a description of Bjøntegaard Delta rate (BD-rate) measurement practices for video coding experiments. The document provides a concept-level overview of recent practices and provides references to technical papers that describe further details. In addition, the document provides comments on why some of the choices were made and identifies situations where caution must be taken when interpreting the results.

To calculate the Bjøntegaard Delta bit rate, a distortion metric needs to be used. For standard-dynamic range video, the distortion metric primarily used has been the Peak Signal to Noise Ratio (PSNR). There are certainly some weaknesses to the PSNR-based BD-rate measure in terms of its correspondence with human perception of fidelity, but the use of PSNR-based BD-rate measures is the most prevalent in the video coding standardization community.

For high-dynamic range (HDR) material additional aspects that influence the usability of BD-rate calculations. For screen content material (i.e., material that has not been captured by a camera), no need for a special metric has been seen and PSNR-based BD-rate is used for this category.

According to [40], there are several steps in the BD-rate calculation process, where the result in each step is calculated from the result obtained in the previous step:

- Calculation of PSNR for individual frames, see clause B.4 for details.

- Calculation of per-sequence PSNR and bit rate values for each quantization parameter (QP) value. The QP value influences the resulting bit rate. Hence, compressing the sequence several times with different QPs ensures that the final BD-rate measurement will reflect the performance at many different bit rates. See clause B.4 for details.

- Calculation of BD-rate values, for details see clause B.5.

In clause B.3, a reference file format to store raw elementary streams are introduced. These formats can be used together with the sequence quality evaluation tool defined in clause B.4.

An explanation to the collected tools are provided in Annex B.6.

## <Editor’s Note: Additional metrics such as VMAF or SSIM may be calculated and also used for BD-rate evaluation>

## B.3	Raw video elementary streams and sample definitions

### B.3.1	Overview

Raw video formats follow the concepts developed in TR 26.902 [41]. The raw video sequences are stored in a format such that display timing is maintained. The 3GP file format as defined in TS 26.244 [41] was viewed as most appropriate.

Attached to this document is a package ISOFileFormatConverter_711.zip which allows conversion of yuv video files in different formats to and from 3GPP/ISO BMFF file format. This package contains the relevant library as well as sample applications.

Raw video is stored as samples in a video track in ISO Base Media Format family files (such as MP4, 3GP and so on). That is, it uses the same video handler, video media header, etc., as a video track containing, for example, H.264/AVC. Raw video can take various formats - based on choice of colour model, sub-sampling, and so on. As is usual in ISO files, the format of the video (the 'decoder' needed) is declared by the sample entry 4-character-code.

This clause defines a raw video format based on the ISO file format.

This video formats adds boxes to the sample entry to parameterize the video.

Based on the requirements in this Technical Report, the following aspects of the raw data need to be attached with the raw data format:

1)	Spatial Resolution width

2)	Spatial Resolution height

3)	Chroma Format

4)	Chroma Sub-Sampling

5)	Picture Aspect ratio

6)	Frame rate

7) 	Bit depth

8) 	Color space format

9)	Transfer characteristics

10) Duration of the sequence

Preferably code points as defined in CICP should be used.

In addition, a sample needs to be defined along with the storage format.

### B.3.2.	Raw Video elementary streams and sample definitions

<tbd>

## B.4	Sequence Quality Evaluation Tool

## B.4.1	Overview

The quality evaluation tool is an extension to the tool provided TR 26.902 [41].

For quality evaluation, an archive QualityEvaluation_711.zip is attached to this document. Note that some libraries in the package ISOFileFormatConverter_711.zip are required.

### B.4.2	Usage of tool

The usage of the tool is as follows:

QualEval [OrigSeq in ISOBMFF format] [ReconSeq in ISOBMFF format] [EncodedSeq in CMAF format]

whereby

-	OrigSeq corresponds to one the original sequence in 3G format.

-	ReconSeq corresponds to the reconstructed sequence without transmission.

-	EncodedSeq corresponds to the encoded sequence after compression.

The output of the program are the following six values:

-	The number of video frames in OrigSeq.

-	The number of video frames in ReconSeq.

-	The average PSNR_Y, PSNR_U and PSNR_V as well as the weighted PSNR in dB.

-	The PSNR of the average MSE_Y, of the average MSE_U and the average MSE_V in dB as well as the PSNR of the weighted average MSE.

-	The bitrate value.

-	<other values should be added, SSIM and VMAF>

### B.4.3	What does the tool

Following the description in [40], the tool does the following for Standard-Dynamic Range (SDR) and Hybrid Log-Gamma (HLG):

-	For each sample of the original sequence

-	checks for the presented frame in the reconstructed sequence.

-	If a frame with the same presentation time exists in the reconstructed sequence, this one is taken for comparison.

-	If no frame with the same presentation time exists in the reconstructed sequence, the one which is closest in the past is taken.

-	If the chosen frame in the reconstructed frame does not have the same resolution as the original one, the reconstructed frame is up-sampled to match the original frame size.

-	The mean square error, MSE_Y, is calculated between the luma channel 𝑑𝑒𝑐𝑌 of the reconstructed output image and the luma channel 𝑜𝑟𝑖𝑔𝑌 of the original image according to [40], equation (7-1) with H the height and W the width.

-	The luma PSNR value, PSNR_Y, for the frame is then calculated according to [40], equation (7-2) with bitDepth identical to the signal bit depth.

-	In addition, the MSE numbers for the two chroma, MSE_U and MSE_V, are calculated accordingly.

-	In addition, the PSNR numbers for the two chroma, PSNR_U and PSNR_V, are calculated accordingly.

-	For each sample of the encoded entire sequence, the following values are computed

-	the sample size according to the sample table box for encoded value is extracted.

-	For the entire sequence, the following values are computed

-	The aggregate PSNR_Y, PSNR_U and PSNR_Y for a sequence are calculated as the average of the PSNR values for the individual frames according to [40], equation (7-3).

-	The aggregate PSNR as a weighted average for each frame according to [40], equation (7-5) is calculated.

-	The aggregate MSE_Y, MSE_U and MSE_Y for a sequence is calculated as the average of the MSE values. The resulting PSNRs of the aggregated MSE_Y, MSE_U and MSE_Y is calculated using the equation in [40], equation (7-2).

-	The PSNR of the averaged MSE is calculated.

-	The aggregate the sample size in bytes is used and divided by the duration of the sequence.

For HDR sequences with PQ transfer characteristics, the following additional metrics are created as defined in [40], clause 8: PSNRL100, wPSNR and DE100.

## B.5	BD-rate numbers

### B.5.1	Introduction

The previous sections provide PSNR values and bit rate values for a single sequence encoding (using also a single encoder configuration). For the BD-rate computation, two streams are needed

1)	Anchor: the anchor refers to the baseline that is used to check the improvements for a tested configuration.

2)	Test: test is the tested method under investigation.

BD-Delta provides the aggregated bitrate savings over a range of bitrates at the same quality. Any quality measure Q may be used as defined in B.4.

The following assumptions are taken:

-	Several experiments have been carried out (for example for a different QP) for the anchor and the test, each resulting in a different bitrate results Ra[i], i=1,…, Na and Rt[j], j=1,…, Nt, respectively, as well as quality results Qa[i], i=1,…, Na and Qt[j], j=1,…, Nt,, respectively. Such results can be mapped to a single bitrate saving value.

-	The results may be carried out with the same resolution for all encodings or for different resolutions (applying a bitrate ladder). The tool in B.4.3 takes care of appropriate scaling to the highest resolution.

### B.5.2	Per-sequence BD-rate

The quality-rate functions for anchor and test results in a curve. The idea for a sequence is to estimate the area between these two curves to compute an average bit rate savings for equal measured quality. A detailed explanation of the how to do this is provided in [40], clause 7.3.

def BD_Q(RA, QA, RT, QT, piecewise=0):

lRA = np.log(RA)

lRT = np.log(RT)

QA = np.array(QA)

QT = np.array(QT)

p1 = np.polyfit(lRA, QA, 3)

p2 = np.polyfit(lRT, QT, 3)

# integration interval

min_int = max(min(lRA), min(lRT))

max_int = min(max(lRA), max(lRT))

# find integral

if piecewise == 0:

p_int1 = np.polyint(p1)

p_int2 = np.polyint(p2)

int1 = np.polyval(p_int1, max_int) - np.polyval(p_int1, min_int)

int2 = np.polyval(p_int2, max_int) - np.polyval(p_int2, min_int)

else:

# See https://chromium.googlesource.com/webm/contributor-guide/+/master/scripts/visual_metrics.py

lin = np.linspace(min_int, max_int, num=100, retstep=True)

interval = lin[1]

samples = lin[0]

v1 = scipy.interpolate.pchip_interpolate(np.sort(lRA), QA[np.argsort(lRA)], samples)

v2 = scipy.interpolate.pchip_interpolate(np.sort(lRT), QT[np.argsort(lRT)], samples)

# Calculate the integral using the trapezoid method on the samples.

int1 = np.trapz(v1, dx=interval)

int2 = np.trapz(v2, dx=interval)

# find avg diff

avg_diff = (int2-int1)/(max_int-min_int)

### return avg_diff

### B.5.2	BD-Rate overall

## Once the BD-rate values for a set of test sequences have been determined, they are typically combined using an arithmetic average. For details refer to [40], clause 7.4.

## B.6	Scripts and Tools

## [Some baseline scripts are provided here: https://bitmovin.com/vp9-vs-hevc-h265/. A full tool will be made available here on github https://github.com/haudiobe/5GVideo and the ported to 3GPP repository]