Source:	Samsung Electronics Co., Ltd.

Title:	Use case categorization

Agenda Item:	6.3

Document for:	Discussion and Agreement

- Introduction

Currently, there are 15 existing use cases agreed from TR26.928 and 4 more new cases collected during 5GSTAR discussions, tentatively. With those, this contribution proposes a baseline categorization for further detailed discussion of requirements and device functional structures.

Note that new use cases still require confirmation as to whether they can provide new features, and the proposed categorization here may need to be revised accordingly.

- Categorization of use cases

Based on the media to be transmitted through uplink/downlink, and the delay requirements for each scenario, the collected use cases can be categorized as follows:

- AR/MR conversational (real-time media service with capturing): A user captures objects or surroundings using a media camera and transmits the captured media via uplink in a real-time manner. The captured media may be avatars, an actual 3D face/body, 2D/360 video of surroundings (or even 3D), or 3D audio. All media are possibly processed at the edge, delivered to another user having AR/MR glasses, and finally represented. The total end-to-end delay should be kept sufficiently low in order not to cause any annoying delay, depending on the service. A receiver device may exchange pose information to the edge for media processing (e.g., split rendering).

- AR/MR messaging (non real-time media sharing with capturing): A user captures objects or surroundings using a media camera and shares them with other user(s). The delay for capturing and uplink/downlink transmission do not need to be as strict as that of the AR/MR conversational use cases. The captured media is primarily static (no change over time).

- AR/MR download (non real-time media download): The pre-captured/generated and stored media in the application server is downloaded and represented in the AR/MR glasses. The uplink for media transmission is not required and the downlink delay does not need to be as strict as that of AR/MR conversational use cases. The captured media is primarily static.

- AR/MR streaming (real-time media streaming): The pre-captured/generated and stored media in the application server is streamed in a real-time manner. The captured media is primarily dynamic (changes over time) and the downlink delay should be sufficiently low such that is comparable to current streaming services. The AR/MR glasses may send its pose information to the edge for media processing (e.g., split rendering).

Table X-1 details the required features for each use case and its mapping to the proposed category.

[Table X-1] List of categorized use cases

| Cat | No1) | Use Case Title | Media |  |  | Delay |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Visual | Audio | UL/DL | Capturing | Delivery |
| Conver- sational | 7 | Real-time 3D Communication | 2D/3D Dynamic | 2D | UL/DL | Sensitive | Sensitive |
|  | 8 | AR guided assistant at remote location (industrial services) | 2D/3D Dynamic | 2D | UL/DL | Sensitive | Sensitive |
|  | 9 | Police Critical Mission with AR | 2D/3D Dynamic | 2D/3D | UL/DL | Sensitive | Sensitive |
|  | 12 | 360-degree conference meeting | 2D/3D Dynamic | 2D | UL/DL | Sensitive | Sensitive |
|  | 15 | XR Meeting | 2D/3D Dynamic 360 Video | 2D/3D | UL/DL | Sensitive | Sensitive |
|  | 16 | Convention / Poster Session | 2D/3D Dynamic 360 Video | 2D/3D | UL/DL | Sensitive | Sensitive |
|  | 17 | AR animated avatar calls | 2D/3D Dynamic | 2D/3D | UL/DL | Sensitive | Sensitive |
|  | 18 | AR avatar multi-party calls | 2D/3D Dynamic | 2D/3D | UL/DL | Sensitive | Sensitive |
|  | 19 | Front-facing camera video multi-party calls | 2D/3D Dynamic | 2D/3D | UL/DL | Sensitive | Sensitive |
|  | 2.2 | AR remote cooperation | 2D/3D Dynamic | 2D | UL/DL | Sensitive | Sensitive |
|  | 2.3 2) | AR remote advertising | 2D/3D Dynamic | 2D | UL/DL | Sensitive | Sensitive |
|  | 2.4 | AR Conferencing | 2D/3D Dynamic | 2D | UL/DL | Sensitive | Sensitive |
| Messag-ing | 1 | 3D Image Messaging | 3D Static | - | UL/DL | Tolerant | Tolerant |
|  | 11 | Real-time communication with the shop assistant | 2D/3D Static | - | UL/DL | Tolerant | Tolerant |
|  | 2.3 2) | AR remote advertising | 2D/3D Static | - | UL/DL | Tolerant | Tolerant |
| Down-load | 2 | AR Sharing | 2D/3D Static | - | DL | Pre-captured | Tolerant |
|  | 10 | Online shopping from a catalogue – downloading | 2D/3D Static | - | DL | Pre-captured | Tolerant |
|  | 23 | Spatial Shared Data | 2D/3D Static | - | DL | Pre-captured | Tolerant |
| Stream- ing | 20 | AR Streaming with Localization Registry | 2D/3D Dynamic | 2D | DL | Pre-captured | Sensitive |
|  | 2.1 | Streaming volumetric video for glass-type MR devices | 2D/3D Dynamic | 2D | DL | Pre-captured | Sensitive |

Note 1) For existing use cases in TR26.928, the same use case number is used. For those in 5GSTAR, the section number in the permanent document is used (tentatively). Re-numbering may be needed after confirmation.

Note 2) Needs further clarification.

- Proposal

We propose to include the text and table in section 2 of this document into the Permanent Document as a baseline for detailed discussions on device functional structures and the way forward.