Source:	Qualcomm, Xiaomi

Title:	FS_5GSTAR: Updates to Device functional structures

Agenda Item:	4.1

Document for:	Discussion and Agreement

# Introduction

The text is a result of offline discussions between the sources related to the input contributions listed below.  A brief summary (by the rapporteur) of the contributions and discussions during the telcos are also included.

[List of input contributions]

- S4aV200586, FS_5GSTAR: Updates to Device functional structures, Samsung

- S4aV200589, [FS_5GSTAR] More on Device Architectures, Qualcomm

- S4aV200583, FS_5GSTAR: Generic reference device functional structures pCR, Xiaomi

- 583 looks a good starting point and it was to be revised (Dec 1st). No revision available yet.

- 586 and 589 are to be merged into 594 (Dec 8th)

[Summary of proposed device types]  (*color code represents the same/similar criteria)

Samsung: 2 UE Types * 2 Glass Modes

- UE Types: depending on the presence of 5G Modem in AR Glass

- Glass Modes: depending on the presence of AR processing functions in AR Glass

Qualcomm: 3 Device types

- Smart Glass wireless tethering: No 5G Uu Modem, External XR Engine

[[TS]] I want to clarify that there is an AR Engine in the device. We need to define AR. The functionalities are quite limited due to power restrictions

- Smart Glass Standalone (or 5G Smart Glass): 5G Uu Modem inside, External or Split XR Engine, Less power consumption

[[TS]] Same as above, AR Engine. I wanna make sure that it is not power consumption, it is about power availability.

- AR HMD Standalone: 5G Uu Modem inside, External or Split XR Engine, High power consumption

[[TS]] For me this one is XR Engine included. You can still defer some high-end rendering to the network. Power consumption is not “high”, but power availability is not the main concern.

Xiaomi: 3 Device types

- Standalone AR glasses or Head-Mounted Displays: 5G Modem, Local AR/MR Processing, High power consumption

- Smart Tethered AR glasses: No 5G Modem, (Mostly) Local AR/MR Processing, Medium power consumption, low weight

- Lightweight tethered AR glasses: No 5G Modem, Little (or No) Local AR/MR Processing, Low power consumption, lightest weight

This document provides further updates to S4aV200604 addressing additional agreements. Also attached some slides to support the discussions.

# Proposed updates to Device Functional Structures

## *** Start change***

## 4.2	Device Functional Structure

### 4.2.1	Device Functions

AR glasses contain various functions that are used to support a variety of different AR services as highlight by the different use cases in clause X.

The various functions that are essential for enabling AR glass-related services within an AR device functional structure include:

a)	Tracking and sensing

-	Inside-out tracking for 6DoF user position

-	Eye tracking

-	Hand tracking

-	Sensors

b)	Capturing

-	Vision camera: capturing (in addition to tracking and sensing) of the user’s surroundings for vision related functions

-	Media camera: capturing of scenes or objects for media data generation where required

-	Microphones: capturing of audio sources including environmental audio sources as well as users’ voice.

NOTE: vision and media camera logical functions may be mapped to the same physical camera, or to separate cameras. Camera devices may also be attached to other device hardware (AR glasses or smartphone), or exist as a separate external device.

-	Audio microphone: capturing of audio may be used for media capture purposes or control purposes.

c)	Basic AR functions

-	2D Decoders: Media decoders to decode 2D media to be displayed

-	2D Encoders: Encoders providing compressed versions of cameras and microphones and other image sensors.

-	Vision engine: engine which performs processing for AR/MR related localisation, mapping, 6DoF pose generation, object detection etc., i.e. SLAM, objecting tracking, and media data objects. The main purpose of the vision engine is to “register” the device, i.e. the different sets of data from real and virtual world are transformed into the single world coordinate system.

-	Late stage reprojection: feature that helps stabilizing holograms when the user moves for pose correction. Typically, this is done by asynchronous time warping.

-	Audio renderer: AR content may require audio media playback; audio playback may also be used for user control.

d)	XR functions

-	All basic AR functions

-	Immersive Media decoders: Media decoders to provide input to rendering stages

-	AR/MR media modelling: 3D modelling of AR/MR media

-	Compositor: displaying layers of images at different levels of depth in a display

-	Display renderer: the generation of one (monoscopic displays) or two (stereoscopic displays) eye buffers from the video content typically using GPUs.  Rendering operations may be different depending on the rendering pipeline of the media, e.g. AR/MR rendering on-device, or on a remote processor.

-	3D Reconstruction: process of capturing the shape and appearance of real objects.

-	Semantic Perception: conversion of sensory observations to contextual and semantical information.

e)	Tethering and network interfaces for AR/MR content delivery

-	The AR glasses may be tethered through non-5G connectivity (wired, WiFi)

-	The AR glasses may be tethered through 5G connectivity

-	The AR glasses may be connected through different flavours for 5G connectivity

-	The requirements for such a connectivity are for further study, in particular for different glass-types. (see for example https://docs.microsoft.com/en-us/azure/remote-rendering/reference/network-requirements)

f)	Physical Rendering

-	Display: Optical see-through displays allow the user to see the real world “directly” (through a set of optical elements though). AR displays add virtual content by adding additional light on top of the light coming in from the real-world.

Some good reads: https://www.linkedin.com/pulse/why-making-good-ar-displays-so-hard-daniel-wagner/

- 	Loudspeakers: integrated loudspeakers in device or headphones

g)	XR Application

-	   An application that makes use of the XR and AR capabilities to provide a user experience.

### 4.2.2	Generic reference device functional structure device types

In TR 26.928, different AR and VR device types had been introduced in clause 4.8. This clause provides and update and refinement in particular for AR glasses.

A summary of the different device types consider the context of this report is provided in Table X. The table also covers

- how the devices are connected to get access to information an

- where the 5G Uu modem is expected to be placed

- where the basic AR functions are placed

- where the core XR functions are placed

- where the XR application is running

- where the power supply/battery is placed.

In all glass device types, the sensors, cameras and microphones are on the device.

This technical report only considers the first three device types as the latter one is considered a special case of the first one when addressing media processing needs, etc.

The following definitions for the AR Basic functions and XR engine are used:

- Basic AR: only the basic AR functions are supported.

- Split XR: the tethered device or external entity (cloud/edge) does a pre-rendering of the viewport based on sensor and pose information, and the AR/MR device performs the final rendering considering the latest sensor information (e.g. applying pose correction). Different degrees of split exist, between different devices and entities. Similarly, AR/MR related localisation (e.g. SLAM), mapping, 6DoF pose generation, object recognition, 3D modelling, encoding and decoding of 3D AR/MR media can be subject to split computation.

- AR/MR device: the AR/MR device performs the full rendering of the viewport in the device, sensor/pose information is only processed locally. Any AR/MR localisation (e.g. SLAM), mapping and other AR/MR engine functions, if applicable, is performed on the device.

Table X: 5GSTAR device types

| Device Type Name | Tethering | 5G Uu Modem | Basic AR | XR (Rendering, etc.) | XR Application | Power Supply |
| --- | --- | --- | --- | --- | --- | --- |
| 5G AR HMD | n/a | device | device | Glass/Split | Device | Device |
| 5G Smart AR Glasses | n/a | device | device | Cloud/Edge | Device/split | Device |
| Wireless Tethered AR Glasses | 802.11ad/y, 5G sidelink. etc. | phone/puck | device | Phone/puck/Split | Phone | device |
| Wired Tethered AR Glass | USB-C | phone | phone | phone | Phone | phone |

Typical architectures for the three main different device types are provided below:

Figure 4.2.1: Functional structure for 5G AR HMD

Main characteristics of Standalone AR Glasses Type 1:

Editor’s Note : This section is updated from S4aV200583 (Xiaomi); further updates required

- As a standalone device, 5G connectivity is provided through an embedded 5G modem

- User control is local and is obtained from sensors, audio inputs or video inputs

- Both AR/MR engine and its associated AR/MR media processing functions is either on the AR/MR device, or  split

- Media processing is local, device needs to embed all required media codecs.

- AR/MR processing is local, device needs to embed AR/MR renderer and compositor.

- Local vision engine shall be powerful enough to provide real-time processing of its inputs.

- (Due to the amount of processing required, such devices are likely to present a significantly higher power consumption in comparison to the other device types., hence the need for heavier batteries, a significantly low battery life (2-4h) and a heavier weight.)

- Support of correction glasses or lenses may be possible but likely at a higher cost.

Standalone AR Glasses Type 2

Figure 4.2.2: Functional structure for 5G Smart AR Glasses

Main characteristics of Standalone AR Glasses Type 2:

Editor’s Note : This section is updated from S4aV200583 (Xiaomi); further updates required

- As a standalone device, 5G connectivity is provided through an embedded 5G modem

- User control is local and is obtained from sensors, audio inputs or video inputs.

- Media processing is local, the device needs to embed all media codecs required for decoding pre-rendered viewports

- The Basic AR engine is local, the XR functions are on the 5G cloud/edge

- Power consumption on such glasses must be low enough to fit the form factors. Heat dissipation is essential.

- Support of correction glasses or lenses may be possible but likely at a higher cost.

Wireless Tethered AR Glasses

Figure 4.2.3: Functional structure for Wireless Tethered AR Glasses

Main characteristics of Wireless Tethered AR Glasses:

Editor’s Note : This section is updated from S4aV200583 (Xiaomi); further updates required

- 5G connectivity is provided through a tethered device which embeds the 5G modem. Wireless tethered connectivity is through WiFi or 5G sidelink. BLE connectivity may be used for audio.

- User control is mostly provided locally to the AR/MR device; some remote user interactions may be initiated from the tethered device as well.

- The AR/MR engine and its associated AR/MR media processing functions (including SLAM/registration, and pose correction) are either in the AR/MR tethered device, or split

- While media processing (for 2D media) can be done locally to the AR glasses, heavy processing may be done on the AR/MR tethered device, or split.

- (While such devices are likely to use significantly less processing than Standalone AR Glasses Type 1 devices by making use of the processing capabilities of the tethered device, they can still support a lot of local media and AR/MR processing. Such devices are expected to provide 8-10h of battery life while keeping a significantly low weight.)

### [4.2.3	Interfaces

Editor’s Note : This section needs to be updated to take into account agreed changes from the previous section

The following interfaces between the different functions of the device functional structure for device type #1 are defined:

Figure 4.2.4: Interfaces for device type #1 functional structure

1)	Tracker/sensor output interface

-	Raw outputs from various tracking and sensor device components, namely IMUs (inertial measurement unit).  These outputs are typically sent as inputs to the vision engine, which performs operations such as 6DoF pose generation.

-	AR/MR devices might contain the following related components:

a)	Accelerometer – used by the system to determine linear acceleration along the X, Y and Z axes and gravity

b)	Gyro – used by the system to determine rotations

c)	Magnetometer – used by the system to estimate absolute orientation

2)	Cameras output to Vision engine interface

-	Depending on the device hardware, different types of camera outputs may be available as inputs into the vision engine or the camera outputs may be sent to the network. Possible camera signals include:,

a)	Visible light environment tracking cameras – typically gray-scale cameras used by a system for head tracking and map building

b)	Depth cameras (ToF)– these may be short-throw (near-depth) or long-throw (far-depth), depending on the application desired (e.g. hand tracking, or spatial mapping)

c)	Infrared (IR cameras) – used for eye tracking and hand tracking

d)	World facing RGB camera – used for image processing tasks and locating the camera’s position in and perspective on the scene

e)	Media camera for capturing persons or objects in the scene for media data generation and consumption

3)	Vision engine to AR/MR renderer interface

-	The vision engine provides all the information required for the AR/MR renderer to adapt the rendering for a consistent combination of virtual content with the real world [4.3.0].  This information may be the output of vision engine processes such as spatial mapping, scene understanding, and room scan visualization.  Vision engine processes are typically SLAM related, differing depending on the specific device and/or platform implementation.

-	One typical output of the vision engine is the device/user pose information, which may include estimation properties depending on the service and application.

-	Additional output can be 3D objects or representations for media consumption.

Editor’s note: in the case of other (tethered) device types, the split of functions between entities are FFS.

4)	Vision engine to AR/MR media processor interface

-	The vision engine provides all the information required for the AR/MR media processor to perform processes such as 3D modelling.  Information from the vision engine as used by the AR/MR renderer may also be sent to and used by the AR/MR media processor for relevant media processing.

5)	AR/MR renderer to Compositor interface

-	The AR/MR rendering typically outputs a rendered 2D frame (per eye) for a given time instance according to the device/user’s current pose in his/her surrounding environment.  This rendered 2D frame is sent as an input to the compositor.  In the future, it can be predicted that non-2D displays will require a different output from the AR/MR renderer in order to support 3D displays e.g., Light Field 3D display.

6)	Compositor to Display interface

-	Rendered 2D frames are composited by the compositor before being passed onto the device display.  The compositor may perform certain pose correction functions depending on the overall rendering pipeline used.

7)	Cameras output to Media processor interface

-	RGB and depth cameras are be used to capture RGB/depth images and videos, which can be consumed as regular 2D or 3D images and videos, or may be used as inputs to a media processor for further media processing.

8)		Media processor to AR/MR media codec interface

-	A media processor performs processes such as 3D modelling, in order to output uncompressed media data into the AR/MR media codec for encoding.  One example of the media data at this interface is raw point cloud media data.

9)		AR/MR media codec to AR/MR renderer interface

-	Compressed AR/MR media content intended for rendering, composition and display is decoded by the AR/MR media codec, and fed into the AR/MR renderer.   Media data through this interface may be different depending on the rendering pipeline.

10)		AR/MR media codec to Network delivery interface

-	Media contents that are captured or generated by the device (from interface 5 and 6) are encoded by the AR/MR media codec before being passed onto the network delivery entity for packetization and delivery over the 5G network.  For 2D AR/MR media contents, this is typically a compressed video bitstream that is conformant to the video codec used by the AR/MR media codec.

-	Media contents that are received through the network delivery interface over the 5G network are depacketized by the network delivery entity and fed into the AR/MR media codec.  The subsequent decoded bitstream is handled through interfaces 7 and 4.

11)		Vision engine to Network delivery interface

-	Certain vision engine outputs may be sent to a remote processor which exists outside the device.  Such data is passed from the vision engine to the network delivery entity for packetization and delivery over the 5G network.

12)		Network delivery interface

-	Network interface for AR/MR content delivery over the 5G network.  In device type #1 for which interfaces are defined in this clause, the 5G modem exists inside the standalone AR glasses device.

]

## *** End change***

# Proposal

We propose to include the changes as presented in this contribution as a pCR for TR 26.998.