Source:	Samsung, Qualcomm, Xiaomi

Title:	FS_5GSTAR: Updates to Device functional structures

Agenda Item:	4.1

Document for:	Discussion and Agreement

# Introduction

During the previous telco on 15th December 2020, it was agreed to merge S4aV200604 and S4aV200605 based on discussions for better clarification. This contribution is to provide an updated pCR which is the outcome from the offline discussion between the source companies.

# Proposed updates to Device Functional Structures

## *** Start change***

## 4.2	Device Functional Structure

### 4.2.1	Device Functions

AR glasses contain various functions that are used to support a variety of different AR services as highlight by the different use cases in clause X.

The various functions that are essential for enabling AR glass-related services within an AR device functional structure include:

a)	Tracking and sensing

-	Inside-out tracking for 6DoF user position

-	Eye tracking

-	Hand tracking

-	Sensors

b)	Capturing

-	Vision camera: capturing (in addition to tracking and sensing) of the user’s surroundings for vision related functions

-	Media camera: capturing of scenes or objects for media data generation where required

NOTE: vision and media camera logical functions may be mapped to the same physical camera, or to separate cameras. Camera devices may also be attached to other device hardware (AR glasses or smartphone), or exist as a separate external device.

-	Microphones: capturing of audio sources including environmental audio sources as well as users’ voice.

c)	Basic AR functions

-	2D media decoders: media decoders to decode visual/audio 2D media to be rendered and presented

-	2D media encoders: encoders providing compressed versions of camera visual data, microphone audio data and/or other sensor data.

-	Vision engine: engine which performs processing for AR/MR related localisation, mapping, 6DoF pose generation, object detection etc., i.e. SLAM, objecting tracking, and media data objects. The main purpose of the vision engine is to “register” the device, i.e. the different sets of data from real and virtual world are transformed into the single world coordinate system.

-	Pose corrector: function for pose correction that helps stabilise AR media when the user. Typically, this is done by asynchronous time warping (ATW) or late stage reprojection (LSR).

d)	AR/MR functions

-	Immersive media decoders: media decoders to decode compressed immersive media as inputs to the immersive media renderer.  Immersive media decoders include both 2D and 3D visual/audio media decoder functionalities.

-	Immersive media encoders: encoders providing compressed versions of visual/audio immersive media data.

-	Compositor: compositing layers of images at different levels of depth for presentation

-	Immersive media renderer: the generation of one (monoscopic displays) or two (stereoscopic displays) eye buffers from the visual content, typically using GPUs.  Rendering operations may be different depending on the rendering pipeline of the media, and may include 2D or 3D visual/audio rendering, as well as pose correction functionalities.

-	Immersive media reconstruction: process of capturing the shape and appearance of real objects.

-	Semantic perception: process of converting signals captured on the AR glass into semantical concept. Typically uses some sort of AI/ML. Examples include object recognition, object classification, etc.

e)	Tethering and network interfaces for AR/MR immersive content delivery

-	The AR glasses may be tethered through non-5G connectivity (wired, WiFi)

-	The AR glasses may be tethered through 5G connectivity

-	The AR glasses may be connected through different flavours for 5G connectivity

-	The requirements for such a connectivity are for further study, in particular for different glass-types. (see for example https://docs.microsoft.com/en-us/azure/remote-rendering/reference/network-requirements)

f)	Physical Rendering

-	Display: Optical see-through displays allow the user to see the real world “directly” (through a set of optical elements though). AR displays add virtual content by adding additional light on top of the light coming in from the real-world. Some good reads: https://www.linkedin.com/pulse/why-making-good-ar-displays-so-hard-daniel-wagner/

- 	Speakers: integrated speakers in device or headphones

g)	AR/MR Application

-	   An application that makes use of the AR and MR capabilities to provide a user experience.

### 4.2.2	Generic reference device functional structure and device types

#### 4.2.2.1	Overview

In TR 26.928, different AR and VR device types had been introduced in clause 4.8. This clause provides an update and refinement in particular for AR glasses.

A summary of the different device types is provided in Table X.  The Wired Tethered AR Glass device type is for reference purposes only, and not considered in this document as it is not included as part of the study item objectives.  The table also covers:

- how the devices are connected to get access to information an

- where the 5G Uu modem is expected to be placed

- where the basic AR functions (as specified in 4.2.1) are placed

- where the AR/MR functions (as specified in 4.2.1) are placed

- where the AR/MR application is running

- where the power supply/battery is placed.

In all glass device types, the sensors, cameras and microphones are on the device.

The definition for Split AR/MR in Table X is as follows:

- Split: the tethered device or external entity (cloud/edge) does a pre-rendering of the viewport based on sensor and pose information, and the AR/MR device and/or tethered device performs a rendering considering the latest sensor information (e.g. applying pose correction). Different degrees of split exist, between different devices and entities. Similarly, vision engine functionalities and other AR/MR functions (such as AR/MR media reconstruction, encoding and decoding) can be subject to split computation.

Table X: 5GSTAR device types

| Device Type Name | Tethering | 5G Uu Modem | Basic AR Functions | AR/MR Functions | AR/MR Application | Power Supply |
| --- | --- | --- | --- | --- | --- | --- |
| Type 1 | N/A | Device | Device | Device/Split 1) | Device | Device |
| Type 2 | N/A | Device | Device | Split 1) | Cloud/Edge | Device |
| Type 3 | 802.11ad, 5G sidelink, etc. | Tethered device (phone/puck) | Device | Split 2) | Tethered device | Device |
| Wired 3) | USB-C | Tethered device (phone/puck) | Tethered device | Split 2) | Tethered device | Tethered device |
| 1) Cloud/Edge 2) Phone/Puck and/or Cloud/Edge 3) Not considered in this document |  |  |  |  |  |  |

The fourth device type "Wired " is considered to provide no difference in terms of architectural and processing requirements compared to the one in the first row as the glass only acts as a sensor and display.

Based on this, the focus is on three main different device types in the remainder of this document following the rows 1 to 3 in Table X.

#### 4.2.2.2	Type 1

Figure 4.2.2-1 provides a functional structure for Type 1

Figure 4.2.2-1: Functional structure for Type 1

Main characteristics of Type 1:

- As a standalone device, 5G connectivity is provided through an embedded 5G modem

- User control is local and is obtained from sensors, audio inputs or video inputs

- AR/MR function are either on the AR/MR device, or split

- The AR/MR application is resident on the device

- Due to the amount of processing required, such devices are likely to require a higher power consumption in comparison to the other device types.

- Functionality is more important than design

#### 4.2.2.3	Type 2

Figure 4.2.2-2 provides a functional structure for Type 2.

Figure 4.2.2-2: Functional structure for Type 2

Main characteristics Type 2:

- As a standalone device, 5G connectivity is provided through an embedded 5G modem

- User control is local and is obtained from sensors, audio inputs or video inputs.

- Media processing is local, the device needs to embed all media codecs required for decoding pre-rendered viewports

- The Basic AR Functions are local to the AR/MR device, and the AR/MR functions are on the 5G cloud/edge

- The AR/MR application resides on the Cloud/Edge.

- Power consumption on such glasses must be low enough to fit the form factors. Heat dissipation is essential.

- Design is typically more important than functionality.

#### 4.2.2.4	Type 3

Figure 4.2.2-3 provides a functional structure for Type 3.

Figure 4.2.3: Functional structure for Type 3

Main characteristics of Type 3:

- 5G connectivity is provided through a tethered device which embeds the 5G modem. Wireless tethered connectivity is through WiFi or 5G sidelink. BLE (Bluetooth Low Energy) connectivity may be used for audio.

- User control is mostly provided locally to the AR/MR device; some remote user interactions may be initiated from the tethered device as well.

- AR/MR functions (including SLAM/registration, and pose correction) are either in the AR/MR device, or split.

While media processing (for 2D media) can be done locally to the AR glasses, heavy AR/MR media processing may be done on the AR/MR tethered device, or split.

- While such devices are likely to use significantly less processing than Type 1 devices by making use of the processing capabilities of the tethered device, they can still support a lot of local media and AR/MR processing. Such devices are expected to provide 8-10h of battery life while keeping a significantly low weight.

## *** End change***

# Proposal

We propose to include the changes as presented in this contribution as a pCR for TR 26.998.