Source:	Samsung Electronics

Title:	[FS_5GSTAR] 7 Devices form-factor related Issues

Agenda Item:	4.8

Document for:	Discussion and Agreement

- Introduction

This contribution proposes draft text for AR glasses device form-factor related issues.

- Proposed Text Change

*** Change #1 ***

2	References

[7.1] H. Chen, Y. Dai, H. Meng, Y. Chen and T. Li, "Understanding the Characteristics of Mobile Augmented Reality Applications," 2018 IEEE International Symposium on Performance Analysis of Systems and Software (ISPASS), 2018, pp. 128-138.

[7.2] S. Kang, H. Choi, “Fire in Your Hands: Understanding Thermal Behavior of Smartphones”, The 25th Annual International Conference on Mobile Computing and Networking (MobiCom '19)

[7.3] T. Chihara, A. Seo, “Evaluation of physical workload affected by mass and center of mass of head-mounted display”, Applied Ergonomics, Volume 68, pp. 204-212, 2018

*** End of Change #1 ***

*** Change #2 ***

7	Devices Form-factor related Issues

7.1	General

The components of AR glasses are same or similar with those of mobile phones which can launch and execute AR applications. However, AR glasses have rather different requirements and limitations compared with mobile phones.

From a form factor perspective, AR glasses have several different design considerations. For example, AR glasses have two separate see-through displays for each eye. They also usually include more than two vision cameras which are spatially separated in order to achieve better disparity for depth estimation. In addition, AR glasses are worn and closely attached to a user’s face and contain IMU sensors to estimate where the user’s focal point is. Most of the included components are designed and placed in order to meet requirements which differ to those for mobile phones.

From a media processing perspective, AR/MR applications consume far more energy than non-AR applications [7.1]. Multiple, as well as different types of cameras are always turned on to track the features detected in 2D and 3D video every second. In the case when AR/MR objects are augmented into the real world, the objects should be rendered frame by frame with different view frustum positions and directions. In the case when the AR/MR objects are rendered in a server, the AR/MR device should upload the user's pose in a millisecond frequency, then download, decode, correct, and composite the pre-rendered image sequences streamed from the server.

Besides, from an ergonomics perspective, restrictions should be considered to place the components of the AR glasses in a limited space and under the manageable range of user neck joint torque.

This clause addresses form-factor related issues from the components of AR glasses, such as battery/power consumption, camera, display, heat dissipation, and weight.

7.2	Battery/Power consumption

The run time of a typical battery is proportional to its physical size, capacity, and weight, while they are proportional to user discomfort and neck torque. A study on the characteristics of AR applications [7.1] measured battery consumption of commercially available applications on AR, streaming, and social networking which shows that AR applications consume around at least 46% more energy than non-AR applications. The capacity of the battery should be designed to support a fair amount of running time for the everyday use of AR/MR applications. However, as capacity is typically proportional to weight, and as the AR glasses should be worn and equipped under the consideration of human ergonomics such as neck strain, there are clear limitations on extending the capacity of the battery.

It is recommended to study the followings to overcome the battery and power consumption related issues:

- How to allow AR/MR applications to offload energy intensive computations from AR glasses to server

- How to allow AR/MR applications to adapt its processing complexity based on the power consumption from the relevant components (E.g., Power consumption from cameras and CPU for SLAM processing)

7.3	Camera

Augmented reality can be realized by SLAM. To understand the physical world through SLAM, various types of multiple cameras need to be continuously turned on and always need to be acquiring image sequences.

Among the various components contributing to heat, such as CPU, GPU, camera and display, it is measured that the cameras are one of major sources of heat dissipation for AR applications [7.1]. AR/MR applications may need to be aware of the available run time remaining, and the amount of heat dissipation felt by the user.

In addition, as multiple cameras may be equipped in AR glasses for various purposes, they need to be designed and placed optimally to process the required functions. Camera related parameters, such as for calibration, pose estimation, SLAM etc. are expected to have a big impact on the quality of service for AR glasses. AR/MR applications may need to be aware of intrinsic and extrinsic parameters for the cameras to properly process the required functions. Such parameters may be delivered to the server whenever there is any change in camera configurations.

7.4	Display

There is at least one display for each eye on a pair of immersive AR glasses. The AR glasses estimates the position of each eye then presents pixels of the rendered AR objects on the display in order to combine the ray of light reflected from the surface of real-world objects with each pixel. A renderer in the AR scene manager may take into consideration the shape and optical distortion characteristics of the displays, pixel arrangements, and the estimated position of each eye of the user. At least one of the view frustum models that represents either an AR glasses, each display, or each eye, with a 3D map of the surroundings can be provided to the AR scene manager in order to minimize the post processing of customizing a generic rendered image to fit to a certain pair of AR glasses.

7.5	Heat dissipation

It has been studied that AR applications generate 4-5 degrees Celsius higher heat than non-AR applications on the same device [7.1]. Another study shows that a user’s heat sensation and discomfort increase with temperature. Overheated components have not only degraded performance but also power leakage through thermal throttling [7.2].

7.6	Electromagnetic field exposure

AR glasses introduce a difference in usage pattern compared with mobile phones. Usually, a user uses a mobile phone whist holding it in his/her hands, keeping the phone close to the ear when he/she is calling to others, or carrying the phone close to the body. On the contrary, AR glasses is a wearable device typically worn on the head, and a user is expected to wear the AR glasses for a relatively longer time in a fixed position. Thus, the electromagnetic field (EMF) exposure to the human head needs to be very carefully examined.

Specific Absorption Rate (SAR) is a measure of the rate of how much energy is absorbed by the human body when exposed to a radio frequency EMF and is basic restriction for frequencies below 6 GHz. For above 6 GHz the relevant metrics are the incident or absorbed power densities (PD). Standardized methodologies for SAR and PD testing have been developed by IEC TC106 and IEEE ICES TC34. These standards specify test procedures for held on the ear and body-worn exposure conditions but lack dedicated procedures for head worn devices like AR glasses.

As AR glasses have distinct features in its device type and usage pattern, it is recommended to study whether there are any safety implications on the user due to these features. It is recommended to establish liaisons with IEC TC106 and IEEE ICES TC34 and request their Joint Working Groups (JWG11, JWG12 and JWG13) to develop standardized evaluation procedures, including guidance on test positions and test phantoms, for head-worn devices.

7.7	Weight

AR glasses consists of displays, sensors, cameras, batteries and so on. The weight of AR glasses puts constant pressure on a user’s skin and changes the amount of torque applied to the neck joints and muscles in a neutral posture.

A study shows that a user’s posture can be changed from a neutral to a look-up posture, a look-down posture, or a body-bending posture because of the relative placement of virtual objects [7.3]. Those different postures increase the moment arm between the Centre of Mass (COM) of the wearable device and the neck joint.

There are different characteristics between HMD type and glasses type devices, as the COM of glasses type devices is biased towards the front of the device, by design. As a result, AR/MR applications should consider the issues due to the differences in the ergonomics between the two different types of wearable devices.

*** End of Change #2 ***