Agenda item: 	5

Source: 	InterDigital,

Title: 	Aligning Video codec/profile/levels across specs

Document for	Discussion and agreement

# Introduction

TS 26.116, Television (TV) over 3GPP services;Video profiles proposes the following video codec requirements in section 3 and operating points mappoing to codec profile and tier in table 4.2

The following video codecs and associated Profiles and Levels should be used:

-	H.264/AVC Progressive High Profile Level 3.1 [5] for 720p HD services

-	H.264/AVC Progressive High Profile Level 4.2 [5] for Full HD services

-	H.265/HEVC Main Profile Main Tier Level 3.1 [6] for 720p HD services

-	H.265/HEVC Main-10 Profile Main Tier Level 4.1 [6] for Full HD services

-	H.265/HEVC Main-10 Profile Main Tier Profile Level 5.1 [6] for UHD services

-	H.265/HEVC Main-10 Profile Main Tier Profile Level 4.1 [6] for Full HD HDR services

-	H.265/HEVC Main-10 Profile Main Tier Profile Level 5.1 [6] for UHD HDR services

The Table 4.2 presents the mapping of the operation points with the codec type, profile and level.

Table 4.2: Video codec parameters

| Operation Point name | Resolution Format | Codec type, profile and level |
| --- | --- | --- |
| H.264/AVC 720p HD | 1280 × 720 | AVC/H.264 Progressive High Profile Level 3.1 |
| HEVC/H.265 720p HD | 1280 × 720 | HEVC/H.265 Main Profile Main Tier Level 3.1 |
| H.264/AVC Full HD | 1920 × 1080 | AVC/H.264 Progressive High Profile Level 4.2 |
| HEVC/H.265 Full HD | 1920 × 1080 | HEVC/H.265 Main-10 Profile Main Tier Level 4.1 |
| HEVC/H.265 UHD | 3840 × 2160 | HEVC/H.265 Main-10 Profile Main Tier Level 5.1 |
| HEVC/H.265 Full HD HDR | 1920 x 1080 | HEVC/H.265 Main-10 Profile Main Tier Level 4.1 |
| HEVC/H.265 UHD HDR | 3840 x 2160 | HEVC/H.265 Main-10 Profile Main Tier Level 5.1 |
| HEVC/H.265 Full HD HDR HLG | 1920 x 1080 | HEVC/H.265 Main-10 Profile Main Tier Level 4.1 |
| HEVC/H.265 UHD HDR HLG | 3840 x 2160 | HEVC/H.265 Main-10 Profile Main Tier Level 5.1 |

In the introduction section, TS.26.116 states that:

The video profile Operation Points defined in this clause are primarily defined in order to apply to the content format being delivered to 3GPP UE PSS/DASH,MBMS and 5GMS clients over the 3GPP system. Parameters for video encoder/decoder, content format and transport are defined.

TS. 26.116 points to the PSS specifications:

- TS 26.244 (3GP file format)

- TS 26.247 (3GPP DASH)

TS 26.247 states: 3GP-DASH clients supporting a particular continuous media type, media decoders are specified in TS 26.234, while TS 26.244 points to TS 26.234 and 26.346.

However 26.234 Transparent end-to-end Packet-switched Streaming Service (PSS); Protocols and codecs; and 26. 346 Multimedia Broadcast/Multicast Service (MBMS);Protocols and codecs;

only provides for the support of:

- H.264 (AVC) Progressive High Profile Level 3.1 decoder

- H.265 (HEVC) Main Profile, Main Tier, Level 3.1 decoder

It may be beneficial to align the SA4 specification so that there are no confusion on which profiles/levels are supported for each codecs.

Note: TS 26.511, 5G Media Streaming (5GMS); Profiles, codecs and formats, is aligned with 26.116 for TV services but provides support for yet additional profile/levels for other services, possibly using TS.26.247. It may be worthwhile to verify if alignment is needed between 26.511 and 26.247 /26.234 too.

# Proposal

It is proposed to add support for the profile/levels defined in 26.112 in 26.234 and 26.346.

If agreed pCR will be generated.