Agenda item: 	4.6

Source: 	Qualcomm Incorporated

Title: 	[FS_5GVideo] Characterization Updates

Document for	Discussion and Agreement

# Introduction

This document provides a first set of characterization results and in particular, it also addresses the issues observed that need discussion.

The contribution includes also a script that is expected to be used for consistent and robust generation of characterization results. This addresses the Editor’s Note in clause 5.7.

Editor’s Note: At this stage the BD-Rate is defined as result of the above excel sheets. In an updated version of the TR, an independent metric computation tool may be provided that allows to generate the BD-Rate values based on the metrics results. The reference for an extended version with 5 points is still tbd.

# Current Text in TR 26.955

## 5.7	Characterization

Characterization is the comparison of a codec under test with an anchor based on the framework introduced in this clause. Characterization in this report is based on Bjöntegard-Delta (BD)-rate information according to [44].

Characterization is expected to provide a summary of the expected gains a codec under test would provide, compared to a reference codec. For characterization, the metric results in this Technical Report are used to derive summary numbers. It is important for a codec to understand the performance for individual scenarios, for individual configurations within a scenario, but also for individual reference sequences. At the same time, a summary comparison is beneficial to provide an overview of the overall performance. A summary based on averages of selected sequences as an example can only provide indication of the performance if the reference sequences would be fully representative. However, it is also of interest to understand maximum and minimum gains.

Figure 5.7-1: Characterization Framework

Based on this, a full characterization of a codec for a scenario against a 3GPP codec is expected to provide at least the following metrics

-   The BD-rate gain (for a given metric and a given anchor codec) for each reference sequence in the test configuration.

-   The minimum, maximum and average BD-rate gain (for a given metric and a given anchor codec) across all reference sequences in the test configuration.

Note that this results typically in the following results for a codec under test. For each scenario and for each configuration, a table is provided to compare against each anchor codec

-    where the row header documents the reference sequence for the test, e.g. S5-R<i> to the specific configuration.

-    where the column header documents the key of the metric

-    that documents in the cell with BD-Rate gain

-    the above is extended with three summary rows, where the column header documents average, minimum and maximum gain

An example is provided in Table 5.7-1.

Table 5.7-1 BD-Rate gain example table for a given codec under test, a given anchor codec and given test configuration

| Reference sequence | y_psnr | u_psnr | v_psnr | psnr | ms-ssim | vmaf |
| --- | --- | --- | --- | --- | --- | --- |
| S5-R01 |  |  |  |  |  |  |
| S5-R02 |  |  |  |  |  |  |
| S5-R03 |  |  |  |  |  |  |
| S5-R04 |  |  |  |  |  |  |
| S5-R05 |  |  |  |  |  |  |
| S5-R06 |  |  |  |  |  |  |
| S5-R07 |  |  |  |  |  |  |
| S5-R08 |  |  |  |  |  |  |
| S5-R09 |  |  |  |  |  |  |
| S5-R10 |  |  |  |  |  |  |
| S5-R11 |  |  |  |  |  |  |
| S5-R12 |  |  |  |  |  |  |
| S5-R13 |  |  |  |  |  |  |
| Average |  |  |  |  |  |  |
| Minimum |  |  |  |  |  |  |
| maximum |  |  |  |  |  |  |

BD-Rate is computed according to the CTC method used in JVET and specified in [44] from the tools publicly available: Reference codec software, Excel file available in [57] for SDR and in [58] for HDR.

The Excel files include the VBS script bdrate( ) to compute the BD-Rate performance between a test codec and a reference from four or five rate-distortion points.

These excel files have been extended in the Random-Access and low delay tabs to contain new columns for the new metrics: VMAF and MS-SSIM, in the SDR case only. The “SA4 extended excel files” for SDR and HDR are attached as S4-template-HDR.xlsx and S4-template-SDR.xlsx.

Editor’s Note: Need to decide what to do if a metric does not have a monotonic behavior for a particular sequence.

Editor’s Note: At this stage the BD-Rate is defined as result of the above excel sheets. In an updated version of the TR, an independent metric computation tool may be provided that allows to generate the BD-Rate values based on the metrics results. The reference for an extended version with 5 points is still tbd.

# Test Data

In order to test the characterization, we used existing data, namely comparing 265 with VTM. We have test data with 4 entries and 5 entries.

Test Data with 4 entries:

- Anchor Data: 	https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/Bitstreams/Scenario-5-Gaming/265/Metrics/

- Test Data: 	https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/Bitstreams/Scenario-5-Gaming/VTM/Metrics/

Test Data with 5 entries:

- Anchor Data: 	https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/Bitstreams/Scenario-3-Screen/265/Metrics/

- Test Data: 	https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/Bitstreams/Scenario-3-Screen/VTM/Metrics/

# Script

As promised above, script is provided to generate the characterization results. The script is hosted at https://github.com/haudiobe/5GVideo. The script can be used to compare an anchor and test.

The generic configuration usage is:

python3 compare.py ANCHOR TEST [OUTPUT]

- ANCHOR: the reference data,

- TEST: test data to be compared against ANCHOR

- OUTPUT: optional output csv path

The scripts make use of the numpy and scipy python libraries.

The script permits two configuration options

comparing 2 metrics csvs:

python3 compare.py

/data/Bitstreams/Scenario-5/265/S5-A01-265

/data/Bitstreams/Scenario-5/VTM/S5-A01-VTM

comparing 2 encoder configurations:

python3 compare.py

/data/Bitstreams/Scenario-5/265@S5-HM-01 (Filters to @S5-HM-01)

/data/Bitstreams/Scenario-5/VTM@S5-VTM-01 (Filters to @S5-HM-01)

Attached to this document, please find the initial script.

# Initial Results

Some initial results are produced and attached to this document.

| Anchor | Test | Result |
| --- | --- | --- |
| 265@S3-HM-01 | VTM@S3-VTM-01 | S3-HM-01.S3-VTM-01.csv |
| 265@S3-HM-02 | VTM@S3-VTM-02 | S3-HM-02.S3-VTM-02.csv |
| 265@S3-SCC-01 | VTM@S3-VTM-01 | S3-SCC-01.S3-VTM-01.csv |
| 265@S3-SCC-02 | VTM@S3-VTM-02 | S3-SCC-02.S3-VTM-02.csv |

| Anchor | Test | Result |
| --- | --- | --- |
| 265@S5-HM-01 | VTM@S5-VTM-01 | S5-HM-01.S5-VTM-01.csv |
| 265@S5-HM-02 | VTM@S5-VTM-02 | S5-HM-02.S5-VTM-02.csv |
| 265@S5-SCC-01 | VTM@S5-VTM-01 | S5-SCC-01.S5-VTM-01.csv |
| 265@S5-SCC-02 | VTM@S5-VTM-02 | S5-SCC-02.S5-VTM-02.csv |
| VTM@S5-VTM-01 | 265@S5-HM-01 | S5-VTM-01.S5-HM-01.csv |

The attached results have been verified with the excel files include the VBS script bdrate( ) to compute the BD-Rate performance between a test codec and a reference from four or five rate-distortion points for selected data points.

Errors are observed for two anchors, but also the VBS script does not solve the issue.

# Open Issues and Questions

The following issues are observed and the proposed solutions are addressed

- the bd-rate could not be computed with non concave or non strictly increasing distortion data points. For instance, when the same MS_SSIM or VMAF score is obtained for several variants of the same anchor. It was observed that for MS_SSIM, the metrics are still linear domain on the server.

  - Action: this needs to be changed to MS_SSIM in log domain

- However, it is expected that there are remaining issues that need to be addressed. For saturating values and non-concave characterization, an update needs to be provided.

  - Action: discuss on how to solve the issue of non-concave metrics

- The “inverse” execution does provide a positive bd-rate, but the absolute value is not identical. This is also the same for the excel sheet. While this is non-intuitive, it seems to be a feature, not a bug.

  - Action: please confirm that there is no issue on this

# Proposals

It is proposed to:

- Generally endorse the approach taken on the characterization

- Update the attachments and the online data with MS_SSIM in dB

- Resolve non-concave metrics functions

- Ask for verification of the script

- Confirm that there is no issue in BD-rate computation despite absolute values for inverse operation are not correct.