Agenda item: 	4.12

Source: 	Qualcomm Incorporated

Title: 	[FS_5GVideo] JM Anchor Updates

Document for	Discussion and Agreement

# Introduction

Based on the Permanent Document in S4-211554 as agreed during SA4#116-e, JM anchors are still missing and production of those is in progress.

| Scenario | Clause | JM anchors | HM anchors | Notes |
| --- | --- | --- | --- | --- |
| Full HD Streaming | 6.2.8 | Configurations frozen. Reference sequences frozen. | Results available, but VMAF and SSIM and DE100 and PSNRL100 missing. Anchor missing |  |
| 4K-TV | 6.3.8 | No Anchors | Results available, but VMAF and SSIM and DE100 and PSNRL100 missing. Anchor missing |  |
| Screen Content | 6.4.8 | Configurations frozen. Reference Sequences frozen | HEVC anchor streams are provided according to the key system here: https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/Anchors/Scenario-3 S3-A65-265. S3-A66-265. S3-A67-265. S3-A68-265. |  |
| Social Messaging | 6.5.8 | Configurations frozen. Reference Sequences frozen | Configurations frozen. Reference Sequences frozen |  |
| Gaming | 6.6.8 | Configurations frozen. Reference Sequences frozen | HEVC anchor streams are provided according to the key system here: https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/Anchors/Scenario-5 |  |

| S4-115-6-2 | Generate JM anchors for S3 | Qualcomm (Nils) |
| --- | --- | --- |
| S4-115-6-3 | Generate JM anchors for S5 | Qualcomm (Nils) |
| S4-115-6-4 | Generate JM anchors for S1 | Qualcomm (Nils) |
| S4-115-6-5 | Generate JM anchors for S4 | Qualcomm (Nils) |

This document provides an update on the action items.

# Scenario 1

The configurations had some issues as follows:

Issue 1:

> DPB size at specified level is smaller than the specified number of reference frames. This is not allowed.

LevelIDC = 42 is not valid, 50 and above has been working for me.

Proposal: Update the config to level 50 and also update the TR

The above updates provide allow to produce correct anchors and metrics.

# Scenario 3

No anchors have been produced yet.

# Scenario 4

Status

- Anchors are available and have been uploaded here: https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/Bitstreams/Scenario-4-Sharing/264/

- Metrics are available and have been uploaded here: https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/Bitstreams/Scenario-4-Sharing/264/Metrics

- Metrics are also attached.

Proposal: It is proposed to adopt the anchors and metrics

# Scenario 5

Anchors are currently produced. Attached are selected encoder logs.

Proposal: It is proposed to take this information into account.

# Proposal and Next Steps

The following is proposed:

- Change the config for Scenario 1

- Adopt the S4 anchors and metrics

- Add this information to an updated TR

- Progress the work

Concretely, we will for the next telco

- Complete anchors and metrics for S1, S3, and S5.

- Provide the characterization results

Anchor verification is welcome.