3GPP TSG SA WG4 Video SWG post 116-e	S4aV220863

E-meeting, 10th – February 1, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.998 | CR |  | rev |  | Current version: | 1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_5GSTAR] On Spatial Mapping |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, b<>com, Orange, Deutsche Telekom AG, PEREY Research & Consulting |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5GSTAR |  |  |  |  |  | Date: |  |  | 2022-01-11 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

=====  CHANGE  =====

#### 4.4.7.3	Visual features, Keyframes and Spatial Maps

Visual features, keyframes, and spatial maps are used for mapping the real world, typically as part of the SLAM process.

Visual features are characteristics of real-world elements that are searched, recognized and tracked in images captured by the AR device as the user moves in a real environment. These images provide a view of the same real-world elements, captured from different positions (as indicated by the camera parameters attached to them) from a single moving camera or multiple cameras. Visual features are generally extracted from points that are recognizable in multiple images.

From the captured images of the real world, keyframes that include one or multiple visual features can be stored for later use. Visual features from captured frames may be matched by comparing those frames with keyframes available to the AR Runtime in order to support the SLAM process. Keyframes have attached camera information defined in 4.4.3.2 to triangulate 3D points correctly from multiple cameras. These 3D points, triangulated from matching visual features, are called spatial features.

Finally, a spatial map may be generated from keyframes and their matched visual features. A spatial map is, thus, a digital representation of the real world surrounding users consisting of at least one spatial feature cloud, i.e., 3D points (vector of 3 floats) with their associated descriptors such as SIFT [59], SURF [60], or ORB [61]. The geometrical part of the spatial map can be represented as a sparse or dense point cloud or a mesh. The mapping process may be performed either at runtime or offline. The spatial map is then used at runtime to relocalize and thus register the AR device by matching the visual features extracted from the current captured frames with the spatial features stored in the spatial map. The spatial mapping approach described herein is one of well-known keyframe-based SLAM techniques [58].

The descriptors of features, whether visual or spatial, are generally vectors of numbers (e.g., vector of 128 floats for SIFT, vector of 64 floats for SURF, vector of 32 integers for ORB). Note that other features such as 3D segments (i.e., a 3D starting point and a 3D ending point) may also be used. During the localization process, the visual features extracted from the current frame, captured by the device, are matched with the spatial features of the map, resulting in 2D-3D correspondences used to estimate the pose of the cameras. However, since the 2D-3D matching process consists of comparing the descriptors of visual features extracted from the current image and those of the spatial features of the map, the complexity can quickly increase for maps containing several hundred thousand or even millions of spatial features. To accelerate the 2D-3D matching process, a spatial map typically also includes the following metadata:

-	Information required for keyframe retrieval. For example, a keyframe retrieval uses Bag-Of-visual-Words (BoW) model. In this case, the information consists of the vocabulary of the BoW model and corresponding descriptor for each keyframe (vector of occurrence counts of a vocabulary in the keyframe). Depending on the visual descriptor used, the vocabulary size is usually a 10-100 MByte, and this vocabulary may be reused.

-	The visual features for each keyframe (e.g. 2D points with their associated descriptors such as SURF, SIFT, ORB represented by a vector of numbers). The number of features extracted per keyframe varies between 200 and 1000.

-	A vector pair (UID of the visual features, UID of the spatial features) that matches the visual features of keyframes with the spatial features of the spatial feature cloud.

Using this metadata, instead of comparing all descriptors of visual features extracted from the current frame with all spatial feature descriptors (from the spatial feature cloud of the spatial map), reduces the otherwise high computational complexity.

The vision-based localization system can then speed-up the matching between visual and spatial features by:

-	Matching the closest keyframe to the current frame by retrieving it with the BoW model

-	Matching the visual features between the current frame and the retrieved keyframe

-	Matching the visual features between the current frame and spatial feature cloud (knowing matches between visual features of the keyframes and spatial features of the spatial feature cloud)

Figure 4.4.7.3-1 illustrates the localization process of a captured 2D frame using a spatial map.  The figure shows a current frame with visual features highlighted in green. The visual features from the current frame are matched with the spatial features and keyframe information stored in the spatial map to estimate the pose of the camera when it captured the frame.

Figure 4.4.7.3-1 Camera pose estimation by features matching between a 2D captured frame and a spatial map.