Source:	Xiaomi

Title:	Updates of EDGAR-1 device architecture

Document for:	Agreement

Agenda Item:	5.6

# 1	Introduction

At the 3GPP meeting #119-e, SA4 published the Permanent Document (PD) for MeCAR v2.0 [1]. In this PD, clause 4.2.1 depicts the current device architecture of the External Display Glasses for AR version 1 (EDGAR-1) type of device. As indicated by the corresponding editor’s note, further improvements of the architecture diagram are encouraged. As a result. this contribution proposes several updates of the architecture.

# 2	Motivation

## 2.1	General

While the 5G STAR Study item provided an exhaustive inventory of AR device designs, the MeCAR Work Item is building on this study to support the potential mass commercialisation of upcoming AR glasses. Therefore, MeCAR is initially focusing on the EDGAR-1 type described in clause 4.2.1.

In addition, it is crucial that the EDGAR-1 device architecture not necessarily fully aligns with but at least can be logically mapped onto existing AR frameworks and APIs, such as the Khronos OpenXR specification, since it is reasonable to anticipate the support of such APIs in the future AR devices. In the event that the device architectures defined in the MeCAR specification do not fit in those XR frameworks and APIs, there are serious doubts whether AR device manufacturers will be able to integrate the MeCAR specification regardless of the technical merits of the MeCAR specification itself.

## 2.2 Background on OpenXR high-level view and relation to the MeCAR device architecture

This clause summarizes information describing the different elements of the OpenXR architecture as available in [2].

OpenXR is a layered architecture, made up of the following elements:

- OpenXR Application

- OpenXR Loader

- OpenXR API Layers

- OpenXR Runtimes

Figure 1 illustrates the relationship with the different elements of the OpenXR architecture.

Figure 1 - High Level View of the OpenXR Loader

The OpenXR application is at the start of the execution chain, and interfaces directly with the OpenXR loader. The loader, in turn, detects, loads, and interacts with any number of OpenXR runtimes and API layers. Each OpenXR runtime controls a complete VR/XR/MR system that an application can choose to interact with. The loader may inject any number of optional API layers between the application and the runtime to augment behavior. As a result, any OpenXR command may involve executing code in a number of different modules, including the loader, API layers, and runtimes.

An OpenXR runtime is a software which implements the OpenXR API. There may be more than one OpenXR runtimes installed on a system, but only one runtime can be active at any given time.

As a result, it is not necessary to represent multiple runtimes in the MeCAR context since the specification would equally apply to any conformant runtime whichever is chosen by the application.

From the point of view of the application, the loader glues the chosen runtime and the related APIs together at the time the application runs. Consequently, the loader does not constitute a concrete entity in the context of MeCAR but merely a given combination of AR Runtime and AR Runtime API. Therefore, the loader does not materialise in any element in the MeCAR device architecture.

Regarding rendering, the application submits layers for presentation into the so-called swapchains. Based on these submitted layers, the compositor produces the final views, possibly adding for instance user interface graphics. Note that some types of layers in OpenXR may require the application to project textures according to the point of view of the eyes while with some OpenXR extensions, such as the XR_KHR_composition_layer_cube, “the user can look all around, and the compositor can display what they are looking at without intervention from the application”.

In the context of MeCAR, we could consider the compositor as part of the AR Runtime, and thus not represent it in the device architecture.

# 3	Proposal

## 3.1	General

Based on the motivations described in clause 4, we propose the following updates to the EDGAR-1 type:

- (technical) Rename AR Scene Manager to Graphics Device and associated API name since it is in fact independent from a scene concept but simply performing graphical operations, typically GPU-based.

- (technical) Set the arrow between the AR Runtime and the Graphics Device with dotted line to express that this communication is not in scope of MeCAR. It can for instance be data exchanged via memory buffers.

- (editorial) The arrows for downlink and uplink flows are changed to regular connectors for easier reference  in the future when it comes to interoperability points.

## 3.2	Proposed updates

### 4.2.1	Device architecture

[Editor’s note] At SA4#119, this section was added while further improvements were improved.

Figure 2  provides the technical architecture of EDGAR-1 UE.

Figure 2 - Device architecture of EDGAR-1 device

The EDGAR-1 is regular 5G UE with 5G connectivity provided through an embedded 5G modem and 5G system components. The EDGAR-1 UE also features several sensors and user controllers relevant for AR experiences that are cameras, microphones, speakers, display and generic user input. The AR/MR Application is responsible for orchestrating the various device resources to offer the AR experience to the user. In particular, the AR/MR Application can leverage three main internal components on the device which are:

- The Media Access Functions (MAF)

- The AR Runtime

- The Graphics Device

The AR/MR Application can communicate with those three components via dedicated APIs called the MAF-API, the Graphics Device API and the AR Runtime API. Among other functionalities, those APIs enables the AR/MR Application to discover and query the media capabilities in terms of support as well as available resources at runtime.

Once the AR/MR application is running, the downlink media flows from the 5G System to the MAF in compressed form and then from The MAF to the Graphics Device in a decoded form. In parallel, the EDGAR-1 UE is capable of establishing an uplink data flow from the AR Runtime to the MAF wherein the data may be in an uncompressed form and then from the MAF to the 5G System wherein the MAF may have compressed the data in order to facilitate the expected transmission over the network.

# 4	Conclusion

We recommend using the updated EDGAR-1 device architecture in clause 3.2 as basis for further work in the MeCAR WI.

# References

- S4-220760, MeCAR Permanent Document v2.0, 3GPP TSG SA WG4 119-e Meeting, 11th – 12th May 2022

- OpenXR® Loader - Design and Operation (with all published extensions), https://www.khronos.org/registry/OpenXR/specs/1.0/loader.html