3GPP TSG SA4-e (AH) Video SWG post 124                            S4aV230031

20 Jun – 1 Aug 2023

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.119 | CR | pseudo | rev | - | Current version: | 0.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [MeCAR] Available Visualization Space |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Tencent |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | MeCAR |  |  |  |  |  | Date: |  |  | 06/25/2023 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Define available visualization space format |  |  |  |  |  |  |  |  |
| Summary of change: |  | A clause defining the format for available visualization space |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clause 6.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change 1

### 6.2.4	Available Visualization Space format

The Application may optionally define a 3-D space that is suitable for rendering visual objects, for instance, to avoid the rendered AR objects occluding with the real objects in the scene. This Available Visualization Space (AVS) may be communicated to the network so that the remote application can take advantage of the information for defining the rendering location of the composed AR objects. The method of calculating the AVS space is out of the scope of this document.

The content of the availableVisualizationSpace type shall follow the following format:

Table 6.2.4-1 – Available Visualization Space

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| availableVisualizationSpace | Object | 0..1 | An object defining the coordinate of the visualization space |
| shape | string | 1 | A string defining the AVS shape, with the following values: - “cuboid”: define a cuboid space, with 6 parameters: location: x, y, z, and size: width, height, and depth, all measured in meters. |
| parameters | Array of float | 1..n | Parameters of the shape. The number of parameters is defined by the above shape’s value, for instance, 6 parameters for shape = “cuboid”. |

End of Change 1