3GPP TSG SA4-e (AH) Video SWG post 126	S4aV230126

Online, 16th Jan 2024

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.966 | CR | - | rev | - | Current version: | 1.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_HEVC_Profiles] On frame-packing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_HEVC_Profiles |  |  |  |  |  | Date: |  |  | 15/01/2024 |
| Category: | B |  |  |  |  |  | Release: |  |  | 18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There is an editor's note in clause 6.2.3.2 that analysis of frame packing is to be provided; this contribution provides this analysis. |  |  |  |  |  |  |  |  |
| Summary of change: |  | An analysis based on the solution of frame packing is provided. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Editors note on frame packing will remain |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.2, 6.x (New), 6.2.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 5.1.2	Review of previous work

Evaluation of AVC based stereoscopic 3D coding techniques has been done in TR 26.905 [3] and its normative support has been added for 3GPP DASH in TS 26.247 [3], the 3GPP file format in TS 26.244 [5], IMS in TS 26.114 [6], VR profiles in TS 26.118 [7], and MBMS in TS 26.347 [8]. The work done in TR 26.905 [3] for Rel-11 focused mostly on stereoscopic viewing on TVs, while today's applications have grown far beyond these, given especially advancements in AR devices. Also, today's requirements on quality are much higher owing to higher quality displays and the available channel capacities. Frame packing is not sufficient because of the detrimental impact on quality, as noted in TR 26.905 [3]: using frame sequential or side by side packed full resolution is not as efficient as multiview based coding.

With the established support for MV-AVC in 3GPP SA4 specifications, an assessment needs to be done to upgrade the support for multiview coding using MV-HEVC with its superior coding performance.

* * * Next Change * * * *

## 6.x	Solution #1.x: HEVC frame packing

### 6.x.1	Introduction

HEVC frame packing is considered a solution to addresses Scenario#1.

### 6.x.2	High-level Description

#### 6.x.2.1	Overview MV-HEVC

Frame packing can be used as one of the options to deliver multiview coded video content. This solution is focused on reusing the existing decoding and SEI signalling to deliver multiview content. Frame packing arrangement SEI message is for example specified by High Efficiency Video Coding (HEVC) [3] cl D.3.16. Various possibilities of frame packing include spatial arrangements i.e., side-by-side or top-bottom, or temporal interleaving.

#### 6.x.2.2	Transport of HEVC frame packing

Scheme for stereoscopic video arrangements ([23] cl 13.5.4) for restricted media tracks allows for signalling of stereo pair (frame packing).

Editor’s NOTE: Layered HEVC extensions of ISO BMFF only signal for SHVC, MV-HEVC, and 3D-HEVC, not frame packed content. Providing a complete picture for signalling support e.g., in ISO BMFF or CMAF is FFS.

### 6.x.3	Evaluation

#### 6.x.3.1 Assessment/discussion of hardware impact

Support for HEVC frame packing mostly reuses existing decoding mechanism with only additional SEI messages carrying signalling to pre/post-process the video data. Though the impact on the needed decoder profile/level is noted in the following section.

#### 6.x.3.2 Codec performance evaluation based on existing results

#### AVC/MVC-based solutions for spatial frame packing have been assessed in TR 26.905 [3]. These AVC/MVC based stereoscopic solutions include side by side frame packing, full resolutions spatial frame packing (where image resolutions are doubled to that of the original), temporal frame packing, and MVC in cl 5.3 (Video codecs for stereoscopic 3D). MVC is shown to have best performance for the simulation conditions. For further details, refer to TR 26.905 [3].

#### Except for full-resolution spatial packing and temporal interleaving, retaining same resolution for spatial frame packing with same profile/level for the decoder results in half the video resolution and hence reduced quality. Full resolution frame packing typically require decoders with higher profile/level.

#### Temporal interleaving would also require supporting double the frame rate and hence may impact on the needed decoder supported profile.

#### Compared to MV-HEVC:

- Frame packed encoding does not allow using non-referenced frame to be used for inter-view prediction.

- Frame packed content does not allow easy separation of views in multiple tracks to be offered for e.g., streaming.

- When MV-HEVC content is provided to a legacy player, it can playback the base view on a 2D display. This is not possible for frame-packed content.

* * * Next Change * * * *

#### 6.2.3.2 Codec performance evaluation based on existing results

The objective and subjective performance results comparing MVC, Simulcast HEVC (each view is coded independently) with MV-HEVC are documented in [12]. The test sequences used for this evaluation are 1080p 8-bit 4:2:0 either 25 or 30 Hz. IPP encoding is used to generate the results. The objective results demonstrate significant performance improvements achieved by MV-HEVC against both MVC and simulcast HEVC, demonstrated by the Bjøntegaard Delta (BD) bitrates table reproduced here:

| Test Sequence | BD-rate reduction of MV-HEVC [%] relative to |  |
| --- | --- | --- |
|  | MVC | Simulcast HEVC |
| S03: Undo_Dancer | -45.7 | -38.7 |
| S04: GT_Fly | -52.9 | -41.0 |
| S13: Band06 | -43.3 | -31.7 |
| S14: BMX | -60.6 | -25.6 |
| Average | -50.6 | -34.2 |

Hence at least 30% performance gains were observed against simulcast HEVC. The corresponding subjective tests using “Expert Viewing Protocol” (EVP) verified the objective gains via MOS for all the sequences above.

Although no formal evaluation exists for the Multiview Main 10 profile of MV-HEVC, considering the large gains achieved as noted above, it is expected that it’s performance should be similar to what is demonstrated for 8-bit content.

Editor's note: Potentially reproduce some EVP results, FFS.

* * * End of Changes * * * *