3GPP SA4-e (AH) Video SWG post 131-bis-e	S4aV250038

# Online, 7 May 2025

Source:	Orange

Title:	[FS_ARSpatial] pCR on Spatial Description Functions

Spec:	3GPP TR 26.819

Agenda item:	3.8

Document for:	Agreement

1. Introduction

The Study on Spatial Computing for AR Services (FS_ARSpatial) was approved during SA#104 meeting. The objectives of the study include identifying where spatial computing functions run and which media, metadata, and description formats are used for exchange between these elements based on the architecture defined in the TS 26.506, notably in split processing scenarios. And document relevant procedures, flows, configurations, and transport protocols.

This document adds details with examples about when it can be relevant to execute spatial computing functions on the cloud or on the edge on remote spatial computing servers.

2. Reason for Change

Spatial computing functions can be executed locally on the device or can run remotely on the cloud or on the edge in a spatial computing server as detailed in TR 26.298. Additional examples about remote spatial computing functions are provided to motivate the need for remote processing.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 26.819.

* * * First Change * * * *

### 4.2.3	Relocalization

Relocalization is a function that estimates the pose of the AR device according to a known real environment. It is used to estimate the pose of the AR device at initialization, when tracking is lost, or regularly to correct the drift of the tracking (TR 26.998 [3] clause 4.2.3). Cameras capture the real world, while sensors (accelerometers, gyroscopes, and depth sensors) contribute additional data for mapping and positioning. Computer vision algorithms process this data to determine the location and orientation of the device relative to its environment.

SLAM, Visual Localization, e.g., Visual SLAM (vSLAM), or Visual Positioning System (VPS) are all algorithms that can be used for mapping unknown environments while also maintaining the localization of the device/user within that environment, as explained in TR 26.928 [2] clause 4.1.4.

Combining a spatial description of the real world with the estimation of the pose of different trackables can also be used for localization as proposed with the World Storage and World Analysis components of the ETSI ARF architecture [21].

For relocalization, the following input data can be used:

-	Sensor data:

-	images (for SLAM)

-	depth maps

-	GNSS data

-	local pose of AR Device

The output of the relocalization function is a pose, which is defined by a position and an orientation.

Visual Positioning System (VPS) is an example of relocalization system that can require important computing and storage capabilities particularly when applied in large environment such as a building or a city. This can be an issue for some devices. That’s why it can be executed remotely on a remote server. In that case, the VPS relies on pre-existing mapping of the real environment. The previously mentioned sensor data can be sent to a server which then computes and provides the pose of the AR device in the reference coordinate system of a known real environment. In an alternative implementation, the AR device retrieves a local map from the server based on this GPS position and then performs the relocalization computation locally.

Examples of VPS include Google GeoSpatial [56] and Niantic Lightship [57] APIs. Standardized interfaces between AR application and remote relocalization services have been proposed by ETSI ARF as described in Section 5.2.2 and by the Open Spatial Computing Platform (OSCP) with the GeoPoseProtocol API.

* * * Next Change * * * *

### 4.2.5	3D model reconstruction

Spatial computing enables the creation of accurate 3D models of surrounding space. It accurately captures real-world scenes and objects using 3D scanning techniques or photogrammetry. These 3D models can be displayed in immersive 3D environments in real-time to provide users with a sense of interactivity and presence. When built in real time they can for example be used to manage occlusion and physics behaviors between real and virtual objects. Such 3D models can also be used offline for authoring AR experiences, and for completing or correcting a digital twin of a real environment.

The 3D model of a real-world environment may also be constructed collectively by aggregating meshes captured by an AR device.

HoloLens (Microsoft) and AR SDKs from Apple and Meta can all build a 3D model of the surrounding environment in real time.

To build the 3D model, the following input data can be used:

- Sensor data:

- Images captured by AR Device

- pose of AR Device

- Depth map (image or texture)

- Mesh captured by AR Device

The output of the 3D Model construction is a 3D Model. A 3D model is a broader term that encompasses the complete representation of a 3D object, including its geometry (which may be represented by a mesh), texture, and materials.

Achieving highly realistic and accurate 3D model reconstruction can require significant computing capabilities, especially when dealing with multiple users capturing the same space, and when using advanced structure-from-motion algorithms or radiance-field-based representations, such as Gaussian splatting. This can be an issue for some devices. These algorithms can then be executed offline on remote servers using sensor data provided by one or multiple AR devices, with the reconstructed models sent back upon completion.

* * * Next Change * * * *

# 2		References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

….

[56] 	Google Geospatial VPS https://developers.google.com/ar/develop/unity-arf/geospatial/check-vps-availability?hl=en

[57] 	Niantic’s Visual Positioning System (VPS) https://www.nianticspatial.com/products/visual-positioning-system

* * * End of Changes * * * *