3GPP TSG-SA SA4-(AH) Video SWG post 132		                                      S4aV250046

Online, 17th of June 2025

Source:	Huawei, HiSilicon

Title:	Considerations for market relevant video codec usage based on SA4 specifications for conversational services

Agenda item:	4.3

Document for:	Agreement

1. Introduction

In this paper we follow up on the LS S4-250763, for which a draft response was written in the Fukuoka meeting: S4-251045. The current draft response includes some discussion of the studies done in 3GPP SA4, mainly the 5G video codec study and the TR in 26.955. While we think this information will be useful and helpful for the community working on video codecs, a bit more is needed to cover the relevant SA4 implementations available in current 3GPP networks.

Some of the work mentioned are implemented over the top in practice and not using non SA4 specification for the implementation.  While this feedback is useful it does not relate to current specific SA4 based deployments. Some of the feedback from our networks departments on the usage of IP media subsystem and the wireless network should be taken into account more as it was lacking in the current draft LS response.

In this paper we highlight the importance of video codec usage in networks, mainly IP media subsystem and the 3GPP RAN/Core network. For usage in these cases SA4 has developed related, relevant and deployed stage 3 specifications.

2. IP Media Subsystem

While the base architecture of the IP media subsystem stage 2 is specified by SA WG 2, important stage 3 mainly on the codec and RTP usage is specified by SA4 such as in TS 26.114. Current video and voice call based on IMS currently make use of this specification that currently specifies the following codec usage:

MTSI clients in terminals offering video communication shall support:

-	H.264 (AVC) Constrained Baseline Profile (CBP) Level 1.2;

-	H.265 (HEVC) Main Profile, Main Tier, Level 3.1. The only exception to this requirement is for the MTSI client in constrained terminal offering video communication, in which case the MTSI client in constrained terminal should support H.265 (HEVC) Main Profile, Main Tier, Level 3.1.

In addition, they should support:

-	H.264 (AVC) Constrained High Profile (CHP) Level 4.0;

-	H.265 (HEVC) Main Profile, Main Tier, Level 4.0.

TS 26.114 has relied on RTP (real time transmission protocol) and covers some specific handling the sequence headers exchange.

Due to this dependency special emphasis on RTP usage makes sense as this is used in the IP media subsystem in the LS.

In addition, the specification should be mentioned as it represents a significant market that relies on 3GPP SA4 specifications and the way the codec is used matters as this is used in existing deployments. Therefore this should be included in the LS response.

3. IP Delivery in 3GPP RAN and low latency XR context

IP media subsystem deployed in the 5G network enables optimizing the (real-time) transmission using 5G core network and support for Quality of Service (QoS). The 5G core network enables direct connectivity to an IMS network.

In addition, 5G low latency XR, split rendering and conversational services have been considered as some of the key use cases in SA4.

While studies on XR this are mentioned, XR does have a major normative implication on the NG-RAN and 5G core for RTP transmission cases in 5G mobile network and for IMS and other 3GPP services.

As an example we give the downlink reception of RTP XR stream over the 5G radio access network.

The RTP stream is potentially marked with PDU Set information relative to the video syntax (see TS 26.522 for more details using HEVC or AVC). If this information is not available the 5G core can still detect PDU set information based on the AVC or HEVC syntax (see annexes of TS 26.522 for more details).

The 5G core transmits packets to NG RAN using GTP-U (see TS 29.281) including PDU set related metadata. The NG RAN can then use this information to apply congestion control and prioritization over highly fluctuating channel, prioritizing packets with lower delay requirements or higher PDU set importance (PSI).

It is important to note that low latency cases it is not always possible for NG-RAN to allocate or buffer resources and groups of packets (PDU Sets) may be discarded based on their relevance. In these cases, due to the low latency constraints buffering and other ways to avoid this cannot be pro-actively used.

Some details are described in TS 25.622, mainly on using the RTP binding, and we recommend taking this into account in the response LS, as NG-RAN and 5G core currently rely on identification of video bit-stream syntax elements.

SA4 developed stage 3 specification in TS 26.522 for supporting this use of the video codec in the RAN for IMS and other real time communication services using RTP.

AS

5G Core (UPF)

NG RAN

UE

RTP with 5G RTP header Extension TS 26.522

GTP-U (N3) carries the PDU set information

Transmission on Highly fluctuating Radio channel

NG Ran applies congestion control and prioritization based on PDU set information to achieve latency constraints

4. Proposal

It is proposed to mention the specifications related to these cases in the response LS to SC29 WG2 to have a balanced overview that also covers the market relevant 3GPP specifications. TS 26.114 covers the configuration of the IP media subsystem and TS 26.522 covers the use of RTP in 5G + IMS deployments including H.264 and H.265 deployment.

The LS should clearly mention the technical requirements in TS 26.114 (codec configuration and pointer to the specification) and the use case in TS 26.522 (where we rely on detecting the video bit-stream syntax elements in the 5G network) and the common deployment for IP telephony as used by operators today.