Agenda item: 	11.6

Source: 	Qualcomm Inc.

Title: 	Procedure Description for NBMP-enable FLUS

Document for	Discussion and Agreement

# Introduction

This contribution provides a description of the procedures for NBMP-enabled FLUS based on a simplified integration as discussed in S4aM200592.

# NBMP-enabled FLUS Session Establishment

The FLUS session establishment with NBMP processing is as follows:

The steps are as follows:

- The application provider establishes a Provisioning session with the AF, in which it configures QoS and Processing templates.

- The AF configures the FLUS Sink to enable discovery of the appropriate FLUS Sink

- The FLUS Source discovers a FLUS Sink that can perform NBMP workflows

- The FLUS Source establishes a connection with the selected FLUS Sink

- The FLUS Sink, which has been pre-provisioned to run NBMP workflows for the application, triggers the NBMP Workflow Manager, which establishes the workflow based on existing workflow description templates

- The uplink streaming starts

# Proposal

We propose to add the content of section 2 to the TR.