3GPP TSG-SA4 Meeting #112e	S4-210182

1-10 February 2021

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.114 | CR |  | Rev | - | Current version: | 16.8.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR Updates on Camera Calibration for Network-based Stitching. |
| --- | --- |
| Source to WG: | Intel, Nokia Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | ITT4RT |  |  |  |  |  | Date: |  |  | 2021-02-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First Change

- X.5.5.3	Viewport dependent delivery of fisheye video

By exposing the coverage information of each fisheye circular video using the parameters in section X.4.5.2, the collective multitude of which makes up the whole 360-degree video, a ITT4RT-Rx client can opt to select only the required fisheye circular videos needed to render the current viewport of the user.

Through the parameters defined in section X.4.5.2, a ITT4RT-Rx client can select the desired fisheye packing configuration of the video stream during SDP negotiation, as well as the initial desired fisheye videos using the id parameter.

]

## X.5.6	Camera Calibration for Network-based Stitching

Network-based stitching in the context of ITT4RT refers to generation of 360-degree videos in the ITT4RT MRF based on 2D video captures received from MTSI clients. This clause describes SDP-based signalling of camera calibration parameters for this purpose using the a=3gpp-camera-calibration attribute.

The SDP syntax for a=3gpp-camera-calibration is defined with the following semantics (detailed ABNF presented at the end of the clause), :

3gpp-camera-calibration = "a=3gpp-camera-calibration:" [SP “Request” SP "Param 1" SP "Param 2" SP ……. SP "Param K"]

where “Param 1”, …. , “Param K” express the set of intrinsic and extrinsic camera parameters as specified below.

If the ITT4RT-Tx client intends to perform network-based stitching to generate 360-degree video from a particular 2D video capture, it shall request camera calibration parameters from the corresponding MTSI sender by including the SDP attribute 3gpp-camera-calibration in the SDP offer under the relevant m= line for that particular video, with the Request flag set to 1 and remaining parameters omitted. The session-level SDP attribute a = group as defined in IETF RFC 5888 [X] shall be used in the SDP offer in order to group the media streams that contain the 2D videos to be used for stitching of a particular 360-degree video. If camera calibration signalling is supported by the corresponding MTSI client, the SDP attribute 3gpp-camera-calibration shall be included in the SDP answer under the relevant m= line to signal the camera calibration parameters associated with that particular video stream, with the Request flag set to 0.

More specifically, detailed camera calibration parameters based on ISO/IEC 23008-2 [3] are provided as follows, considering the multi-view acquisition information SEI message for HEVC. With these specifications, a 3-dimensional world point, wP = [ x y z ] is mapped to a 2-dimensional camera point, cP[ i ] = [ u v 1 ], for the i-th camera according to:

s * cP[ i ] = A[ i ] * R−1[ i ] * ( wP − T[ i ] )	(eqn. X.5.6.1)

where A[ i ] denotes the intrinsic camera parameter matrix, R−1[ i ] denotes the inverse of the rotation matrix R[ i ], T[ i ] denotes the translation vector, and s (a scalar value) is an arbitrary scale factor chosen to make the third coordinate of cP[ i ] equal to 1.

Equation X.5.6.1 can be extended to incorporate the entrance pupil variation to correct the incidence ray of  cP[ i ] = [ u v 1 ] such that it always passes through the camera optical center, thereby removing distortion. The resulting entrance pupil coefficients E[i] may be incorporated into Equation X.5.6.1 as

s * cP[ i ] = A[ i ] * R−1[ i ] * ( (wP + E) − T[ i ] )															(eqn. X.5.6.2)

where wP + E[i]) = [ x y z+E ], E = e1* 𝞡3 + e2* 𝞡5 + e3* 𝞡7 + e4* 𝞡9, 𝞡 is the incidence angle pertaining to each ray formed by the pixel cP[ i ] = [ u v 1 ], and [e1, e2, e3, e4] are entrance pupil coefficients. In addition, the accuracy of these entrance pupil parameters have an influence of the accuracy of estimated extrinsic parameters and thus improve the future imaging tasks. If not available, vector E is considered as 0 and a fallback to eqn. X.5.6.1 is expected.

Accordingly, the following intrinsic camera parameters can be signalled in the SDP for each 2D video capture using the a=3gpp-camera-calibration attribute:

focalLengthX[ i ] specifies the focal length of the i-th camera in the horizontal direction as a signed floating-point number.

focalLengthY[ i ] specifies the focal length of the i-th camera in the vertical direction as a signed floating-point number.

principalPointX[ i ] specifies the principal point of the i-th camera in the horizontal direction as a signed floating-point number.

principalPointY[ i ] specifies the principal point of the i-th camera in the vertical direction as a signed floating-point number.

skewFactor[ i ] specifies the skew factor of the i-th camera as a signed floating-point number.

The intrinsic matrix A[ i ] for i-th camera is represented by:

It is possible that the intrinsic camera parameters are equal for all of the cameras. In that case, only one set of values based on the above parameters would need to be signalled, e.g., via SDP signalling at the session level.

Furthermore, the following extrinsic camera parameters can be signalled in the SDP for each camera as per ISO/IEC 23008-2 [3]:

rE[ i ][ j ][ k ] specifies the ( j, k ) component of the rotation matrix for the i-th camera as a signed floating-point number.

The rotation matrix R[ i ] for i-th camera is represented as follows:

tE[ i ][ j ] specifies the j-th component of the translation vector for the i-th camera as a signed floating-point number.

The translation vector T[ i ] for the i-th camera is represented by:

For the i-th camera, E[ i ][ j ] specifies the j-th component of the entrance pupil coefficient [e1, e2, e3, e4] where j=1,…4. The parameters are represented as a signed floating-point number, as per eqn (2) above.

The syntax for the "a=3gpp-camera-calibration" attribute shall conform to the following ABNF:

3gpp-camera-calibration = "3gpp-camera-calibration:" PT 1*WSP attr-list

PT = 1*DIGIT / "*"

attr-list = ( set *(1*WSP set) ) / "*"

;  WSP and DIGIT defined in [RFC5234]

set= "[" "request=" requestvalue "focalLengthX=" sfloatvalue "," "focalLengthY=" sfloatvalue "," "skewFactor=" sfloatvalue "," "principalPointX=" sfloatvalue "," "principalPointY=" sfloatvalue "rotation00=" sfloatvalue "rotation01=" sfloatvalue "rotation02=" sfloatvalue "rotation10=" sfloatvalue "rotation11=" sfloatvalue "rotation12=" sfloatvalue "rotation20=" sfloatvalue "rotation21=" sfloatvalue "rotation22=" sfloatvalue "translation0=" sfloatvalue "translation1=" sfloatvalue "translation2=" sfloatvalue "epupil1=" sfloatvalue "epupil2=" sfloatvalue "epupil3=" sfloatvalue epupil4=" sfloatvalue "]"

sfloatvalue= [sign] sizevalue ["." 6*DIGIT]

sign = "-"

requestvalue = 0 / 1

sizevalue = onetonine *5DIGIT

; Digit between 1 and 9 that is

; followed by 0 to 5 other digits

onetonine = "1" / "2" / "3" / "4" / "5" / "6" / "7" / "8" / "9"

; Digit between 1 and 9

End of Changes