3GPP TSG SA WG4 #113e	S4-210463

# E-meeting, 6th – 14th April 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.955 | CR | <CR#> | rev | - | Current version: | 0.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | pCR26.955: Messaging and Social Sharing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5GVideo |  |  |  |  |  | Date: |  |  | 2021-03-31 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Provides a basic overview on Messaging and Social Sharing Additional input is expected from other contributions |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| 56 |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

=====  CHANGE  =====

### 6.5.3	Source Format Properties

Table 6.5.3-1 provides an overview of the different source signal properties for Social Sharing and Messaging. This information is used to select proper test sequences.

Table 6.5.3-1 Source Format Properties for Social sharing scenario

| Source format properties | Social Sharing |
| --- | --- |
| Spatial resolution | 1920 x 1080, 1080 x 1920 |
| Chroma format | Y’CbCr |
| Chroma subsampling | 4:2:0 |
| Picture aspect ratio | 16:9, 9:16 |
| Frame rates | 24, 25, 30, 50, 60 Hz |
| Bit depth | 8, 10 |
| Colour space formats | BT.709, BT.2020 |
| Transfer characteristics | BT.709, BT.2100 (HDR) |

=====  CHANGE  =====

### 6.5.6	Interoperability Considerations

Social sharing and messaging applications require that the content is included in a packaging and file format.

=====  CHANGE  =====

### 6.5.7	Reference Sequences

Table 6.5.7-1 provides the selected reference sequences for this scenario. Keys are identified to refer to the sequences in the context of the scenario. The sequences are named and a reference to the details of the sequence is provided. A justification is provided, why this sequence is selected.

Editor’s Note: This is not yet complete.

Table 6.5.7-1 Reference Sequences for Social sharing and messaging

| Key | Name | Reference | Justification/Comment |
| --- | --- | --- | --- |
| S4-R01 | SmoothSkater | Annex C.X | 4K Especially good on the face close-up. |
| S4-R02 | Neon | Annex C.X | 4K difficult sequence, dark and noisy |
| S4-R03 | Nightclub | Annex C.X | 4K |
| S4-R04 | Bees | Annex C.X | Full-HD, portrait Stationary sequence with graphic overlays. Easy. |
| S4-R05 | Frog | Annex C.X | Full-HD, portrait |
| S4-R06 | Walking-in-Street | Annex C.X | Difficult content, Full-HD Portrait |

=====  CHANGE  =====

### 6.5.8	Anchor Definition

#### 6.5.8.1	Overview

This clause provides details on how to generate the anchors for the Social sharing and messaging scenario.

#### 6.5.8.2	H.264/AVC Anchors

##### 6.5.8.2.1	Overview

Table 6.5.8.2.1-1 provides an overview of the H.264/AVC anchor tuples. Keys are identified to refer to the anchors in the context of the scenario.

Editor’s Note: This is not yet complete.

Table 6.5.8.2.1-1 Anchor Tuple generation with H.264/AVC for Social sharing and messaging

| Key | Clause | Reference Sequence | Reference Encoder | Config | Variations | Anchor Key |
| --- | --- | --- | --- | --- | --- | --- |
| S4-A01-264 | 6.5.8.2.2 | S4-R01 | JM19.0 | JM-0X | QP=[17,20,23,26,29,32] | S4-A01-264-<QP> |
| S4-A02-264 | 6.5.8.2.2 | S4-R02 | JM19.0 | JM-0X | QP=[17,20,23,26,29,32] | S4-A02-264-<QP> |
| S4-A03-264 | 6.5.8.2.2 | S4-R03 | JM19.0 | JM-0X | QP=[17,20,23,26,29,32] | S4-A03-264-<QP> |
| S4-A04-264 | 6.5.8.2.2 | S4-R04 | JM19.0 | JM-0X | QP=[17,20,23,26,29,32] | S4-A04-264-<QP> |
| S4-A05-264 | 6.5.8.2.2 | S4-R05 | JM19.0 | JM-0X | QP=[17,20,23,26,29,32] | S4-A05-264-<QP> |
| S4-A06-264 | 6.5.8.2.2 | S4-R06 | JM19.0 | JM-0X | QP=[17,20,23,26,29,32] | S4-A06-264-<QP> |

##### 6.5.8.2.2	JM-0X: Sharing, no Intra

To generate the anchor bitstreams, JM19.0 is used.

The settings are defined in the attached configuration file jm-0x.cfg.

In summary, each source sequence is encoded with the following important parameters:

-	QP: [17,20,23,26,29,32]

-	ProfileIDC = 100 (High Profile)

-	LevelIDC = 41, 42, 51

-	IntraPeriod = 0 (only first)

-	IDRPeriod = 0 (only first)

-	QPISlice = QPPSlice = QPBSlice = [17, 22, 27, 32, 37, 42]

-	SearchRange

= 64 for fullHD;

= 128 for 4K

-	NumberOfReferenceFrames = 5

-	NumberBFrames = 2

-	SearchMode = 0 (fast full search)

#### 6.5.8.3	H.265/HEVC Anchors

##### 6.5.8.3.1	Overview

Table 6.5.8.3.1-1 provides an overview of the H.265/HEVC anchor tuples. Keys are identified to refer to the anchors in the context of the scenario.

Editor’s Note: This is not yet complete.

Table 6.5.8.3.1-1 Anchor Tuple generation with H.265/HEVC for Social sharing and messaging

| Key | Clause | Reference Sequence | Reference Encoder | Configuration | Variations | Anchor Key |
| --- | --- | --- | --- | --- | --- | --- |
| S4-A1-265 | 6.5.8.3.2 | S4-R1 | HM16.22 | HM-0X | QP = [22, 25, 28, 31, 34, 37] | S4-A1-265-<QP> |

##### 6.5.8.3.2	HM-0X

tbd