Source:	Nokia Corporation

Title:	Considerations for defining performance requirements

Document for:	Discussion & Agreement

Agenda Item:	7.5 – IVAS_Codec

# 1. Introduction

Performance requirements for MASA format have been earlier discussed in [1]. Requirements were suggested to be formed based on mono EVS and dual-mono EVS with unquantized (UQ) MASA spatial metadata. Listening experiments based on this general approach have successfully been conducted and presented in [2] and [3].

This document discusses this idea further by identifying potential approaches for bit rate allocations of the EVS-based references.

# 2. Example formulation

In [1], the following example formulation was considered:

| Category | BW | Bitrate (kbit/s) | FER | DTX | Requirements |
| --- | --- | --- | --- | --- | --- |
| Stereo MASA: Clean speech | SWB | 48 | 0% | On/Off | NWT EVS @ 2x [X] kbps + UQ spatial metadata OR BT EVS @ 2x [Y] kbps + UQ spatial metadata |
| Mono MASA: Clean speech | SWB | 32 | 0% | On/Off | NWT EVS @ [Z] kbps + UQ spatial metadata OR BT EVS @ [W] kbps + UQ spatial metadata |

(NWT = not worse than, BT = better than)

# 3. Discussion

3.1 Bit rates

Regarding the IVAS bit rates, the draft IVAS design constraints [4] specify:

| Bit Rates | When input is a monaural signal without spatial metadata, the IVAS codec shall operate at bit rates of EVS (including all EVS Primary and AMR-WB IO modes). When input is a stereo signal and EVS interoperable operation is supported (see Backward interoperability), the bit-rates of the EVS bitstream representing a mono downmix shall be from 9.6 kbit/s to 24.4 kbit/s. In other cases: the IVAS codec shall operate at least at bit rates of 13.2, 16.4, 24.4, 32, 48, 64, 80, 96, 128, 160, 192, 256, 384, 512 kb/s. Editor’s note: The SID bit rate supported in the DTX/CNG/SID operation is [TBD]. Note: The bit rates specified above for IVAS operation are net bit rates meaning the payload bit rates excluding the rate for RTP payload header. |
| --- | --- |

For the MASA format, the bit rate range from 13.2 to 512 kb/s can be expected to cover roughly three smaller ranges with specific characteristics. While the exact values for these cannot currently be determined, it is at least understood that these ranges can be different for mono-based MASA (mono-MASA) and stereo-based MASA (stereo-MASA) due to differences in mono (EVS) and stereo coding (IVAS) quality.

Low bit rates:

- Transport channel coding is challenging

- Metadata coding is challenging, reference is transparent

Mid bit rates:

- Transport channel coding quality improves and begins to reach transparency

- Metadata coding quality improves and begins to reach transparency

High bit rates:

- Transport channel coding is fully transparent

- Metadata coding reaches transparency, and accuracy is improved further with increasing bit rate

3.2 EVS-based “quality steps”

For example, the values for “2x [X] kbps” and “[Z] kbps” indicating the stereo and mono transport signal coding quality, as shown in section 2, need to be defined. These “quality step” values for the EVS-based references have been discussed in the past, e.g., at the Wroclaw meeting. Some potential values that were identified are “one step” or “two steps” below the corresponding IVAS bit rate.

One additional approach to consider the spatial capability of the MASA format in this context is to think of the spatial metadata as, e.g., “one extra channel providing spatial capability”. Thus, mono-MASA is considered as mono + spatial channel enabling spatial audio. Similarly, stereo-MASA is considered as stereo + spatial channel enabling spatial audio. Thus, we may consider 2 x EVS and 3 x EVS at corresponding IVAS bit rate.

This gives three potential approaches for further consideration. For mono-MASA these are:

- EVS bit rate is 1 step below IVAS bit rate, e.g., 80 kb/s for IVAS at 96 kb/s

  - EVS at 64 kb/s + UQ metadata

- EVS bit rate is 2 steps below IVAS bit rate, e.g., 64 kb/s for IVAS at 96 kb/s

  - EVS at 64 kb/s + UQ metadata

- EVS bit rate is derived from 2 x EVS equal to IVAS bit rate, e.g.:

  - 2 x 48 kb/s for IVAS at 96 kb/s

Similarly, for stereo-MASA:

- 2 x EVS bit rate is 1 step below IVAS bit rate, e.g., 80 kb/s for IVAS at 96 kb/s

  - 2 x EVS at 32 kb/s + UQ metadata

- 2 x EVS bit rate is 2 steps below IVAS bit rate, e.g., 64 kb/s for IVAS at 96 kb/s

  - 2 x EVS at 24.4 kb/s + UQ metadata

- 2 x EVS bit rate is derived from 3 x EVS equal to IVAS bit rate, e.g.:

  - 3 x 32 kb/s for IVAS at 96 kb/s

As seen for the example IVAS rate of 96 kb/s, these three approaches can result even in the same EVS rates being used. However, the overall bit rate curves are different. Full tables are provided below for detailed consideration:

| Requirement based on 1 step below IVAS rate for mono |  |  |  |  |
| --- | --- | --- | --- | --- |
| IVAS bit rate | Target EVS bit rate | Target metadata | Closest EVS bit rate | Available for metadata |
| 13,2 | 9,6 | 3,6 | 9,6 | 3,6 |  |
| 16,4 | 13,2 | 3,2 | 13,2 | 3,2 |  |
| 24,4 | 16,4 | 8 | 16,4 | 8 |  |
| 32 | 24,4 | 7,6 | 24,4 | 7,6 |  |
| 48 | 32 | 16 | 32 | 16 |  |
| 64 | 48 | 16 | 48 | 16 |  |
| 80 | 64 | 16 | 48 | 32 |  |
| 96 | 80 | 16 | 64 | 32 |  |
|  |  |  | 96 | 0 |  |
| 128 | 96 | 32 | 96 | 32 |  |
| 160 | 128 | 32 | 128 | 32 |  |
| 192 | 160 | 32 | 128 | 64 |  |
| 256 | 192 | 64 | 128 | 128 |  |
| 384 | 256 | 128 | 128 | 256 |  |
| 512 | 384 | 128 | 128 | 384 |  |

According to above option:

- At IVAS rates of 16.4 kb/s and 32 kb/s, there is a drop in the available rate for UQ metadata compared to next lower IVAS rate. This is not ideal.

- Metadata bit rate is very low up to IVAS rate of 16.4 kb/s.

- At IVAS rate of 96 kb/s there are basically two possible EVS rates, but only one leaves room for UQ metadata.

- Starting from IVAS rate of 160 kb/s, EVS rate caps.

| Requirement based on 2 steps below IVAS rate for mono |  |  |  |  |
| --- | --- | --- | --- | --- |
| IVAS bit rate | Target EVS bit rate | Target metadata | Closest EVS bit rate | Available for metadata |
| 13,2 | 8 | 5,2 | 8 | 5,2 |  |
| 16,4 | 9,6 | 6,8 | 9,6 | 6,8 |  |
| 24,4 | 13,2 | 11,2 | 13,2 | 11,2 |  |
| 32 | 16,4 | 15,6 | 16,4 | 15,6 |  |
| 48 | 24,4 | 23,6 | 24,4 | 23,6 |  |
| 64 | 32 | 32 | 32 | 32 |  |
| 80 | 48 | 32 | 48 | 32 |  |
| 96 | 64 | 32 | 64 | 32 |  |
| 128 | 80 | 48 | 64 | 64 |  |
|  |  |  | 96 | 32 |  |
| 160 | 96 | 64 | 96 | 64 |  |
| 192 | 128 | 64 | 128 | 64 |  |
| 256 | 160 | 96 | 128 | 128 |  |
| 384 | 192 | 192 | 128 | 256 |  |
| 512 | 256 | 256 | 128 | 384 |  |

According to above option:

- At IVAS rate of 128 kb/s there are two possible EVS rates.

- Metadata bit rate is very low / low up to IVAS rate of 16.4 kb/s.

- Starting from IVAS rate of 192 kb/s, EVS rate caps.

| Requirement based on 2 x EVS rate for mono + UQ metadata |  |  |  |  |
| --- | --- | --- | --- | --- |
| IVAS bit rate | Target EVS bit rate | Target metadata | Closest EVS bit rate | Available for metadata |
| 13,2 | 6,6 | 6,6 | 7,2 | 6 |  |
| 16,4 | 8,2 | 8,2 | 8 | 8,4 |  |
| 24,4 | 12,2 | 12,2 | 13,2 | 11,2 |  |
| 32 | 16 | 16 | 16,4 | 15,6 |  |
| 48 | 24 | 24 | 24,4 | 23,6 |  |
| 64 | 32 | 32 | 32 | 32 |  |
| 80 | 40 | 40 | 32 | 48 |  |
|  |  |  | 48 | 32 |  |
| 96 | 48 | 48 | 48 | 48 |  |
| 128 | 64 | 64 | 64 | 64 |  |
| 160 | 80 | 80 | 64 | 96 |  |
|  |  |  | 96 | 64 |  |
| 192 | 96 | 96 | 96 | 96 |  |
| 256 | 128 | 128 | 128 | 128 |  |
| 384 | 192 | 192 | 128 | 256 |  |
| 512 | 256 | 256 | 128 | 384 |  |

According to above option:

- At IVAS rates of 80 and 160 kb/s there are two possible EVS rates.

- Metadata bit rate is low at IVAS rate of 13.2 kb/s.

- Starting from IVAS rate of 356 kb/s, EVS rate caps.

| Requirement based on 1 step below IVAS rate for stereo |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| IVAS bit rate | Target 2 x EVS | Target EVS bit rate | Target metadata | Closest EVS bit rate | Closest 2 x EVS | Available for metadata |
| 13,2 | 9,6 | 4,8 | 3,6 | 7,2 | 14,4 | -1,2 |  |
| 16,4 | 13,2 | 6,6 | 3,2 | 7,2 | 14,4 | 2 |  |
| 24,4 | 16,4 | 8,2 | 8 | 9,6 | 19,2 | 5,2 |  |
| 32 | 24,4 | 12,2 | 7,6 | 13,2 | 26,4 | 5,6 |  |
| 48 | 32 | 16 | 16 | 16,4 | 32,8 | 15,2 |  |
| 64 | 48 | 24 | 16 | 24,4 | 48,8 | 15,2 |  |
| 80 | 64 | 32 | 16 | 32 | 64 | 16 |  |
| 96 | 64 | 32 | 32 | 32 | 64 | 32 |  |
| 128 | 96 | 48 | 32 | 48 | 96 | 32 |  |
| 160 | 128 | 64 | 32 | 64 | 128 | 32 |  |
| 192 | 160 | 80 | 32 | 64 | 128 | 64 |  |
|  |  |  |  | 96 | 192 | 0 |  |
| 256 | 192 | 96 | 64 | 96 | 192 | 64 |  |
| 384 | 256 | 128 | 128 | 128 | 256 | 128 |  |
| 512 | 384 | 192 | 128 | 128 | 256 | 256 |  |

According to above option:

- At IVAS rate of 13.2 kb/s, there is no room available for UQ metadata.

- Metadata bit rate is very low up to IVAS rate of 32 kb/s.

- At IVAS rate of 256 kb/s there are two possible EVS rates.

- Starting from IVAS rate of 384 kb/s, EVS rate caps.

| Requirement based on 2 steps below IVAS rate for stereo |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| IVAS bit rate | Target 2 x EVS | Target EVS bit rate | Target metadata | Closest EVS bit rate | Closest 2 x EVS | Available for metadata |
| 13,2 | 8 | 4 | 5,2 | 7,2 | 14,4 | -1,2 |  |
| 16,4 | 9,6 | 4,8 | 6,8 | 7,2 | 14,4 | 2 |  |
| 24,4 | 13,2 | 6,6 | 11,2 | 7,2 | 14,4 | 10 |  |
| 32 | 16,4 | 8,2 | 15,6 | 8 | 16 | 16 |  |
| 48 | 24,4 | 12,2 | 23,6 | 13,2 | 26,4 | 21,6 |  |
| 64 | 32 | 16 | 32 | 16,4 | 32,8 | 31,2 |  |
| 80 | 48 | 24 | 32 | 24,4 | 48,8 | 31,2 |  |
| 96 | 48 | 24 | 48 | 24,4 | 48,8 | 47,2 |  |
| 128 | 64 | 32 | 64 | 32 | 64 | 64 |  |
| 160 | 96 | 48 | 64 | 48 | 96 | 64 |  |
| 192 | 128 | 64 | 64 | 64 | 128 | 64 |  |
| 256 | 160 | 80 | 96 | 64 | 128 | 128 |  |
|  |  |  |  | 96 | 192 | 64 |  |
| 384 | 192 | 96 | 192 | 96 | 192 | 192 |  |
| 512 | 256 | 128 | 256 | 96 | 192 | 320 |  |

According to above option:

- At IVAS rate of 13.2 kb/s, there is no room available for UQ metadata.

- Metadata bit rate is very low up to IVAS rate of 16.4 kb/s.

- At IVAS rate of 256 kb/s there are two possible EVS rates.

| Requirement based on 3 x EVS rate for stereo + UQ metadata |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| IVAS bit rate | Target 2 x EVS | Target EVS bit rate | Target metadata | Closest EVS bit rate | Closest 2 x EVS | Available for metadata |
| 13,2 | 8,8 | 4,4 | 4,4 | 7,2 | 14,4 | -1,2 |  |
| 16,4 | 10,9 | 5,5 | 5,5 | 7,2 | 14,4 | 2 |  |
| 24,4 | 16,3 | 8,1 | 8,1 | 8 | 16 | 8,4 |  |
| 32 | 21,3 | 10,7 | 10,7 | 9,6 | 19,2 | 12,8 |  |
| 48 | 32,0 | 16,0 | 16,0 | 16,4 | 32,8 | 15,2 |  |
| 64 | 42,7 | 21,3 | 21,3 | 24,4 | 48,8 | 15,2 |  |
| 80 | 53,3 | 26,7 | 26,7 | 24,4 | 48,8 | 31,2 |  |
| 96 | 64,0 | 32,0 | 32,0 | 32 | 64 | 32 |  |
| 128 | 85,3 | 42,7 | 42,7 | 48 | 96 | 32 |  |
| 160 | 106,7 | 53,3 | 53,3 | 48 | 96 | 64 |  |
| 192 | 128,0 | 64,0 | 64,0 | 64 | 128 | 64 |  |
| 256 | 170,7 | 85,3 | 85,3 | 96 | 192 | 64 |  |
| 384 | 256,0 | 128,0 | 128,0 | 128 | 256 | 128 |  |
| 512 | 341,3 | 170,7 | 170,7 | 128 | 256 | 256 |  |

According to above option:

- At IVAS rate of 13.2 kb/s, there is no room available for UQ metadata.

- Metadata bit rate is very low up to IVAS rate of 16.4 kb/s.

- Starting from IVAS rate of 384 kb/s, EVS rate caps according to this approach.

One important observation is that at the very lowest rates it can be difficult to find a suitable dual-mono EVS reference for stereo-MASA. Therefore, it seems practical to consider mono-based spatial reference for both mono-MASA and stereo-MASA at the very lowest rates. Another general observation is that references (and similarly expected IVAS quality) for mono-MASA are likely to saturate significantly sooner than for stereo-MASA.

The size of spatial metadata in MASA format [4] can be 268.8 kb/s or 422.4 kb/s in each frame. In addition, there is a minor amount (0.8 kb/s) of descriptive metadata in each frame. It is important to have some reasonable minimum metadata bit rate for considering the UQ spatial metadata of the EVS-based references, especially for the lowest IVAS bit rates. One such number can be derived from the thinking of spatial metadata as “one extra channel”. This corresponds to minimum EVS rates of 7.2, 8 (WB), 9.6, 13.2 (SWB), and 16.4 kb/s (FB). On the other hand, the example tables suggest that at 13.2 kb/s, this value cannot be higher than 6 kb/s. In addition, with increasing codec bit rate, the additional bit rate for metadata should be proportionally increased.

# 4. Summary

This document provides potential approaches for defining EVS-based references for mono-MASA and stereo-MASA requirements.

We propose to adopt at least the approaches discussed here as basis for future work to find and agree suitable MASA performance requirements.

# References

[1]	S4-200137: “On spatial audio Performance Requirements”, Nokia Corporation.

[2]	S4-210848: “IVAS MASA spatial speech quality evaluation”, Nokia Corporation.

[3]	S4-220138: “IVAS MASA spatial speech quality evaluation using EVS”, Nokia Corporation.

[4]	S4-220551: “IVAS Design Constraints (IVAS-4)”