Source:	Ericsson LM

Title:	IVAS-7a updates

Document for:	Agreement

Agenda Item:	7.5

# Introduction

At SA4#118-e, IVAS-7a [1] was updated by agreed audio track designators and configurations.

This contribution suggests a couple of additional updates, on the generic file naming convention and the example given for BS.1770 level normalization, being adapted to the agreed 7.1+4 loudspeaker setup.

# References

- Pdoc IVAS-7a: Processing plan for selection phase, v.0.5.0

## File naming

### Generic definitions

The following generic filename convention is used for input/output audio files:

input<tr><fs>		Input to a processing module (or sequence of modules)

output<tr><fs>		Output from a processing module (or sequence of modules)

Table 6 specifies suffixes used whenever needed for differentiation.

Table 6: File name suffixes

| Suffix | Specification |
| --- | --- |
| <tr> | Track designator according to Table 3, delimited by initial underscore |
| <fs> | Sampling frequency in kHz being .8k, .16k, .32k or .48k |

Editor’s Note: Not fully consistent with usage in section 5.

#### 5.1.3.4 	BS.1770 Level adjustment

To normalize the level of a 48 kHz sampled file, use:

bs1770demo.exe –nchan N –lev L -conf xxxx input.48k output.48k

where N is the number of channels, L the target level in LKFS (default: -26), and xxxx a configuration string with one value per channel specifying channel weighting according to:

x = ’1’ 	loudspeaker position within |elevation| < 30 deg, 60 deg <= |azimuth| <= 120 deg, 
(weighted by 1.41)

x = ’L’ 	LFE channel (weighted by 0)

x = ’0’ 	otherwise (weighted by 1)

E.g. normalizing stereo to -26 LKFS:

bs1770demo.exe –nchan 2 –lev -26 -conf 00 input.48k output.48k

E.g. normalizing 7.1+4 multi-channel configuration, as specified in Table 5, to -26 LKFS:

bs1770demo.exe –nchan 12 –lev -26 -conf 000L00110000 input.48k output.48k

#### BS.1770 Level measuring

To measure the level of a 48 kHz sampled file, use:

bs1770demo.exe –nchan N -conf xxxx input.48k

where N is the number of channels, and xxxx a configuration string with one value per channel specifying channel weighting according to:

x = ’1’ 	loudspeaker position within |elevation| < 30 deg, 60 deg <= |azimuth| <= 120 deg, 
(weighted by 1.41)

x = ’L’ 	LFE channel (weighted by 0)

x = ’0’ 	otherwise (weighted by 1)