Agenda item: 	10.5

Source: 	InterDigital Communications.

Title: 	iRTCW client functional components and architecture

Document for	Discussion and Agreement

# Introduction

In this contribution, we discuss the functional components of an iRTCW client. More specifically we discuss the additional functional components that extend the architecture defined in Figure 4.2 of TS 26.114 [1] to enable WebRTC based real-time communication.

# iRTCW Client functional components

The proposed functional components of an iRTCW client using 5G System are shown in Figure 1. An iRTCW client with these functional components is capable of supporting the four collaboration scenarios discussed in the permanent document [2].

Figure 1 Functional components of an iRTCW client in UE using 5G System

WebRTC session setup and management:

This component in the iRTCW client exchanges the SDP information along with ICE STUN and TURN server information between iRTCW peers and setup the RTC media channels and Data channels. This module also manages the WebRTC session and QoS handling.

RTC media channel:

This component represents a network channel for an established media delivery session and is used for unidirectional or bidirectional media transfer between iRTCW peers using RTP/SRTP protocols.

WebRTC Data channel:

This component represents a network channel which can be used for unidirectional or bidirectional peer-to-peer transfers of arbitrary data. Each Data channel is associated with a WebRTC session. When the SDP message contains a scene description URL, a WebRTC data channel is established to retrieve scene descriptions, including scene updates, over the network. In some use cases, the data channel may also be used for downloading some of the AR media in the scene over the network.

Scene Manager:

The Scene Manager is a software component that can create an immersive scene graph representation after parsing the scene description information (e.g., as described in clause 4 of TR 26.998 [2]). When an SDP message contains a URL for a scene description, the WebRTC session manager instantiates and initializes a Scene Manager.

AR media access:

A generic interface module on the client device that enables access to AR media and other related data that is needed by the scene manager or AR Runtime to provide an AR experience. The AR Media access module typically uses 5G System functionalities to access AR media and other related data.

AR Runtime:

The AR Runtime creates a local AR/MR session and performs registration with the local environment (e.g., using the processes described in clause 4 of TR 26.998 [3]). The Scene Manager requests the creation of a local AR/MR session from the AR Runtime interface module present in an iRTCW client.

3D media encoder:

A generic encoder module capable of encoding the 3D media representation content in real-time. Appropriate codecs, media encoding formats and profiles will be defined by the MeCAR work item. For example, a 3D media representation may be a point cloud or a 3D mesh.

3D media decoder:

A 3D media decoder is a decoding module capable of decoding the 3D media representation in real-time.

# Proposal

We propose to agree the baseline functional components of an iRTCW client in clause 2 and document them in the iRTCW PD.

# References

- 3GPP TS 26.114: “IP Multimedia Subsystem (IMS) Multimedia telephony, media handling and interaction”.

- S4aR220010, “iRTCW Permanent Document”

- 3GPP TR 26.998: “Support of 5G glass-type Augmented Reality / Mixed Reality (AR/MR) devices”.