3GPP TSG SA WG4#121	S4-221327

Toulouse, 14th – 18th November 2022

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.119-PD | CR | pseudo | rev | - | Current version: | 3.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [MeCAR] Updated AR Framework |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | MeCAR |  |  |  |  |  | Date: |  |  | 08/11/2022 |
| Category: | C |  |  |  |  |  | Release: |  |  | 18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The AR framework as presented in the initial version missed details on audio. In addition, the uplink management was unclear. Finally call flows have been refined |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

=====  CHANGE  =====

### 4.2.1	Device architecture Framework

[Editor’s note] At SA4#119, this section was added while further improvements were improved. In particular, the audio rendering may need further clarification in terms of interfaces and roles between the AR/MR Application, the AR Scene Manager and the AR Runtime.

Figure 11  provides the technical architectural framework of the AR UE.

Figure 11 – 5G AR Framework

The AR UE is a regular 5G UE with 5G connectivity provided through an embedded 5G modem and 5G system components. The AR UE also features several sensors and user controllers relevant for AR experiences that are cameras, microphones, speakers, display and generic user input.

The AR/MR Application is responsible for orchestrating the various device resources to offer the AR experience to the user. In particular, the AR/MR Application can leverage three main internal components on the device which are:

- The Media Access Functions (MAF)

- The XR Runtime

- The Scene Manager together with the presentation engine that includes functions such as scene composition and possible complex audio or visual rendering.

- The XR Source Management address the management or sources provided through the XR runtime such as microphones, cameras, trackers, etc. The source management may expose information to the application or may provide a subset to the media access function to be sent remote.

The AR/MR Application can communicate with those components via dedicated APIs. Exposed in the diagram are

- the XR Runtime API which provides access to the core functionalities of an XR device.

- the MAF-API, which is essential to allow the application to make use of dedicated media functions available on the XR device

In addition, it is expected that an application can communicate with the scene manager and the presentation engine to establish the presented scene. It can also communicate with the XR source management in order to coordinate which data is made available for uplink streaming.

Among other functionalities, those APIs also enables the AR/MR Application to discover and query the media capabilities in terms of support as well as available resources at runtime.

Regarding rendering, it is expected that the XR Scene manager has access to the latest the pose and tracking information from the XR Runtime which is then provided. Based on this information, the XR Scene Manager may for example determine the objects visible to the user at a given point in time or more generally the objects that may be needed to be rendered in the next rendering cycles. The XR scene manager is responsible together with the presentation engine to submit the rendered views to the XR Runtime as frames written to the images of the Swapchains. Swapchains formats are established during the configuration phase of the using the information provided by the XR Runtime API. From those images in the Swapchains, the XR Runtime then generates the left and right eye buffers possibly based on late adjustment techniques using updated pose information, if available, commonly known as late stage reprojection (LSR).

Once the XR application is running, the downlink media flows from the 5G System to the MAF in compressed form and then from the MAF to the XR Scene Manger in form of primitive buffers. Note that for real-time media, the primitive buffers are provided with a rendering time and an associated pose. The XR scene management may be guided by a static or dynamically updated scene description.

In parallel, the 5G AR UE is capable of establishing an uplink data flow from the XR Runtime to the MAF wherein the data may be in an uncompressed form and then from the MAF to the 5G System wherein the MAF may have compressed the data in order to facilitate the expected transmission over the network.

A typical call flow for establishing an XR Presentation is as follows:

- The AR/MR Application is launched

- The AR/MR application launches and XR Session in the runtime

- The AR/MR application communicates with the scene manager to provide a scene description either through an application interface or through a well-defined scene description document, possibly retrieved over the network

- Based on the scene requirements, the capabilities of the XR Runtime, as well as the capabilities of the media access function, a set of media pipelines in the uplink and downlink are established.

- The media access function accesses the network resources or sends data to the network using the established media pipelines.

- The XR Runtime captures and renders the relevant data to match the device capabilities and the user interaction.