Source:	Dolby Laboratories Inc.

Title:	On send side audio performance assessment for Immersive Audio Systems –additional metrics

Document for:	Discussion and agreement

Agenda Item:	7.6

# Introduction

In tdoc S4-221447 Dolby suggests clarifying some points in TS 26.260 and updating it accordingly. To further progress regarding the sending direction testing within ATIAS, the present document reviews various ways to characterize the ambisonics capture of IVAS-capable UE:s.

The industry would gain from having documented methods for UE performance characterization. A subset of such test methods may later be selected as a basis for minimum performance requirements.

# Discussion: How to characterize Immersive Audio Systems in the Sending Direction

The following discusses conceivable characteristics of immersive audio systems in the sending direction. We present a definition of such characteristics, the corresponding characteristics of an ideal capture system and a sketch of how the characteristics could be measured.

## Send frequency response of captured Ambisonics components

Definition:

Ratio of the sound pressure magnitude spectrum of the DUT for Ambisonics component (m, l): Plmf and a reference diffuse field sound pressure spectrum Pref(f) ( =P00f ). Letters l and m respectively denote Ambisonics degree and index. This means, for each Ambisonics component a send frequency response is measured:

Glmf=PlmfPref(f) .

Ideal characteristic:

The ideal send frequency response of the captured Ambisonics components would be flat, i.e. be frequency independent (for plane wave input).

How to formulate requirements:

One conceivable way to measure against requirements can be taken from TS 26.260. A decorrelated pink noise test stimulus is simultaneously played over all speakers of a periphonic array. The reference diffuse field sound pressure spectrum is obtained through recordings with a diffuse-field microphone positioned at the geometric centre of the periphonic array. The sound pressure magnitude spectrum of the DUT for Ambisonics component (m, l) is obtained through measuring with the DUT at the geometric centre and extracting and analysing that (B-format) component after coding, transmission and decoding as in 26.260 Figure 1.

The sensitivity/frequency response shall meet the ideal characteristics, within some tolerance.

## Send directional response of captured Ambisonics components

### Angular-dependent sensitivity

Definition:

Microphone angular-dependent sensitivity can be described by Sensaθ,φ (in Volt/Pascal or digital amplitude per Pascal), e.g. referring to the output voltage or digital level generated by a microphone for a given sound pressure at the microphone location for an incident plane wave from a certain direction θ,φ. Likewise, the angular-dependent sensitivity of the capture of each Ambisonics component (m, l) can be described as Senslmθ,φ. As this sensitivity may also be frequency dependent, frequency is a further parameter:

Senslmθ,φ,f .

Angular-dependent sensitivity of the capture of an Ambisonics component (m, l) can also be defined for multiple (K) incident plane waves from respective directions θk,φk, k=1…K. This yields:

SenslmΘ,Φ,f with Θ=(θ1,…,θK) and Φ=(φ1,…,φK).

Of interest may be the cases K=1, K=2 and K=N. For K=2, the two directions should be 90 degrees apart (see [1]). For K=N, a reasonable setup could be where the directions would correspond to the Fliege positions of the considered Ambisonics order N.

Ideal characteristic:

The ideal angular-dependent sensitivity for the captured Ambisonics component (m, l) is obtained from the spherical harmonics equations. For simplicity, it is assumed that the spherical coordinate systems of the measurement room and the captured Ambisonics signals are aligned.

For the case K=1, we get

Senslmθ,φ,f=constant1∙Ylmθ,φ

with Ylmθ,φ being the real valued spherical harmonics of the degree m and index l with -m≤l≤m.

Thus, if the reference client offers output of this Ambisonics component it will in the ideal case and valid single plane-wave assumption (e.g. a distant-enough point source to approximate plane waves at the UE) follow the above relation. The ideal frequency characteristic is flat. Care needs to be taken to apply the proper Ambisonics component ordering and normalization.

For any K, we get

SenslmΘ,Φ,f=constant1∙k=1KYlmθk,φk .

These cases are not discussed further in this contribution.

How to formulate requirements:

The measurement for the simplest case of K=1 can be done according to the principles outlined above (under 2.1) using pink noise test stimuli. Unlike 2.1, only a single speaker at a time is used.

The angular-dependent sensitivity shall meet the ideal characteristics, within some tolerance.

### Direction of arrival

Definition:

Taken from [5]:

The FOA signals are formed with a set of pre-calculated filters that convert the signal from Eigenmike to FOA. Creation of such filters has been described, e.g., in [5]. The result is an FOA signal that contains four signal channels. This signal is formulated into FuMa-ordering (i.e., the traditional B-format ordering) as the public DirAC equations used in the reference have been defined for it. Thus, the signal is:

b=bwbxbybz.

The next step is to calculate intensity and energy related parameter values according to equations:

e=b22

i=Rbwbxbybz*.

The energy and the intensity are calculated per subframe, i.e., the values of the four different time slots are summed together to produce the total subframe intensity and energy. Further steps are thus performed with subframe accuracy.

It should be noted that these are not the actual physical properties but parameters that are closely related to the physical properties. Actual direction of intensity is opposite of above and scaling factors have been omitted for efficiency and SN3D scaling is assumed for the FOA signal to simplify calculations. This analysis methodology is an optimized version of the typical DirAC analysis (see [6, pages 90-91]).

After formation of these parameters, they are combined in frequency to form frequency-band based values to reduce required amount of further calculations and data. This combination is simply done by summation of all values within a frequency band as further steps do not require that absolute scale is maintained. In further processing, it is assumed that these parameters work on the desired frequency resolution.

The next step is to estimate the direction of arrival (DOA), i.e., the spatial direction parameter, and the direct-to-total energy ratio. For spatial direction, this is done with equation:

θ=arctaniyixπ180°

for azimuth and equation:

φ=arctanizix2+iy2π180°

for elevation. Here, arctan function is assumed to be the computational variant “atan2” that solves the correct quadrant automatically.

Ideal characteristic:

The DOA angle calculated from the Ambisonics components from the UE capture system, is identical to the ground truth angle to the incident sound

How to measure:

The UE is connected via a wireless link to a reference client

Loudspeakers are placed on a suitable grid (reusing what is already specified, e.g. according to Annex A or B).

The loudspeakers emit sound one at a time. Their incidence angle is known. It remains to be evaluated whether an exponential sweep, pink noise or speech is the most suitable.

The Ambisonics components from the reference client are used to calculate the DOA (according to above).

The DOA is compared to the ground truth angle, potentially in several frequency bands and potentially time-averaged.

If the sending UE is properly implemented in terms of directionality, phase and scaling of Ambisonics components, the DOA metric is expected to correspond to the ground truth angle.

How to formulate requirements:

The DOA angle calculated from the Ambisonics components from the UE capture system shall be within some tolerances w.r.t. the ground truth angle to the incident sound.

### Virtual microphones

Definition:

A virtual microphone is created by a linear combination of the ambisonics components. As an example, for FOA, a cardioid, super-cardioid, figure-of-eight etc microphone can be constructed and pointed in an arbitrary direction, by adding or subtracting portions of the four FOA ambisonics components.

It is called a virtual microphone because its characteristics are manipulated without affecting the microphone itself. Compare with music production; the mix engineer can in post-production select where to point a microphone, and even construct an infinite number of virtual microphones pointing in various directions with various beam widths. A simplified variant of FOA capture is MS stereo recording using one omni and one figure-of-eight capsule, where the stereo width is decided in post-production by turning (usually two) virtual microphones’ mutual angle and setting their pickup pattern to taste. For ATIAS, this means the test system can steer an infinite number of virtual microphones anywhere and controlling the width of the pickup beam, without interacting with the UE that is “just” supplying ambisonics components to the test system.

Ideal characteristics

An ideal first-order directional microphone’s directional pickup in a plane can be described as:

f=p+1-pcosθ

, where p =1 provides omni-directional, p=0.5 cardioid, p =0.37 super-cardioid, p =0.25 hyper-cardioid, p =0 bi-directional, etc.

This polar pattern can further be rotated, as illustrated below. The GIF illustrates adding portions of FOA components W, X and Y (“Pattern” corresponds to p above):

Figure 1 Illustration of beam and pattern steering (copied from [4])

With these simple means we are able to e.g. construct two simultaneous virtual microphones, one pointing at a sound source, the other pointing away from the sound source. This can be as simple as summing/subtracting the outputs from the reference decoder:

- Virtual mic A = W+Y

- Virtual mic B = W-Y

In the ideal case, if using two virtual back-to-back cardioids, one pointing towards a single sound source and one pointing away from it, only the first microphone shall produce an output signal.

How to formulate requirements:

Example: When a single sound source is placed at the positive Y direction in relation to the UE, the level of signal (W+Y) shall exceed the level of signal (W-Y) by at least X dB.

This test can be expanded to cover a variety of incidence angles. Or it can be expanded to rotate the virtual microphones while keep a single source constant.

The merit of using two virtual microphones (which are preferably implemented simultaneously), instead of only one, is that the testing is robust to dynamic range processing in the UE, thus spatial aspects can be robustly assessed with this method. This is achieved by the virtual microphones “pointing” at two or several incidence angles while the UE is subjected to only one source from one angle. A merit compared [1], is that we avoid two simultaneous sound sources and two separable signals.

The method is equally usable for HOA, by defining appropriate linear combinations for the ambisonics components from the decoder.

# Conclusion and next steps

It is suggested to create a permanent document to collect candidate methods for UE ambisonics capture performance assessment. It is proposed to include the methods provided in this document. It is envisioned to select suitable methods among the candidate methods based on practicality aspects in a further stage of the ATIAS work.

Appendix A

Figure 2 Example of two simultaneous virtual cardioid microphones, pointing towards or away from a sound source

Appendix B

To compare with an historic example, the Calrec Soundfield microphone was connected to a control unit that could combine the FOA components to form two virtual microphones, forming a virtual X/Y stereo pair.

Figure 3 Analogue Calrec controller unit for the Soundfield FOA microphone [2]. A virtual stereo pair of microphones is configured in terms of their directional pattern and angle between them.

A more recent example is a plugin from Voyage:

Figure 4 Ambisonics plugin example [3]

# References

[1]	S4-220482: On ATIAS acoustic performance testing for FOA audio, Dolby Laboratories Inc.

[2]	Image of Calrec equipment: https://jp.mercari.com/item/m60935101404

[3]	Image of Voyage plugin: https://voyage.audio/spatial-mic-converter-plugin-virtual-mic-output-overview/

[4]	GIF image: https://commons.wikimedia.org/wiki/Category:Microphone_polar_patterns

[5]	S4-191167: Description of the IVAS MASA C Reference Software, Nokia Corporation

Depending on the defined format for the ambisonics, scaling factors may be applied to the X and Y components in this example