Agenda item: 	10.7

Source: 	InterDigital Communications

Title: 	[GA4RTAR] EAS discovery in edge-enabled 5G RTC Architecture

Document for:	Discussion and Agreement

# Introduction

This contribution addresses one of the objectives of the GA4RTAR work item, namely:

“Specify support for AR relevant functionalities such as split-rendering or spatial computing on top of a 5G System based on this architecture”.

In this contribution, we discuss the EAS discovery issue in the 5G-RTC extended architecture for edge computing when the WebRTC application is a web application or in the Application Provider-driven approach.

# 5G-RTC Architecture extension

Begin Change

4.4.2 Extended 5G-RTC architecture for Edge Computing

The 5G-RTC architecture can be extended to add support for media processing in the edge. The extended architecture is an integration of the 5G-RTC architecture defined in TS 26.506 with the architecture for enabling Edge Applications defined in TS 23.558.

The extended 5G-RTC architecture supports both client-driven as well as Application Provider-driven management of the edge processing session.

The 5G-RTC Application Provider may request the deployment of edge resources as part of the Provisioning Session.

- In the client-driven approach, the WebRTC Application is aware of the support of edge processing in the network and takes steps, such as using the EDGE-5 APIs, to discover and locate a suitable 5G-RTC AS instance in the Edge DN.

- In the Application Provider-driven approach, the 5G-RTC Application Provider configures the 5G-RTC AF to automatically deploy edge processing for the media sessions of the corresponding Provisioning Session. The WebRTC Application may not be aware of the edge deployment and the EAS is discovered through other means, such as DNS resolution with support from the EASDF as specified in 3GPP TS 23.548 [1].

When the WebRTC application is a web application, the implementation of the EDGE-5 interface to discover the 5G-RTC AS/EAS location by accessing the EEC is difficult as the Web browser providers may not implement interfaces necessary for supporting edge enabled 5G-RTC applications/services. Also, in the Application Provider-driven approach the Application Client (AC) and EEC are not used to discover the 5G-RTC AS/EAS location.

To resolve the above EAS discovery issue in the Application Provider-driven approach and when the WebRTC application is a web application, the EAS information can be shared with the Media Session Handler by the 5G-RTC AF using RTC-5 interface. In another method, the location of the 5G-RTC AS/EAS can be provided directly to the Web application using RTC-8 interface.

The extended architecture is as shown in Figure 1.

Figure 1. Edge-enabled 5G-RTC architecture

End Change

# Proposal

We propose to agree the updated text in 5G-RTC architecture extension and document it in the GA4RTAR TS.

# References

- 3GPP TS 23.548: "5G System Enhancements for Edge Computing; Stage 2".