Source:	Fraunhofer IIS

Title:	Proposed test plans for IVAS Codec selection testing with Stereo content and P.SUPPL800 tests

Document for:	Discussion & Agreement

Agenda Item:	7.5

Introduction

In Table 4 of Section 5 in the IVAS-8a Test Plan [1] there is a high-level overview of the experiments to be held using P.SUPPL800 tests. Experiments P800-1, P800-2 and P800-3 give an overview of the experiments for Stereo input as can be seen in the respective rows from Table 4 below.

| Exp | Input format | Source material | Listening environment | Bitrates kbps | FER/jitter | DTX | Headtracking | Nb of test conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| P800-1 | Stereo | Clean speech | Headphones | ≤ 48 | ≤ 3% | Y | No | 10 |
| P800-2 | Stereo | Speech+Background | Headphones | ≤ 48 | ≤ 3% | Y | No | 10 |
| P800-3 | Stereo | Mixed & Music | Headphones | ≤ 48 | ≤ 3% | N | No | 10 |

This contribution aims to give a detailed description of the P.SUPPL800 experiments to be held for the selection phase for this input format.

As stated in Section 4.5.1 of the IVAS-8a Test Plan [1], P.SUPPL800 test experiments will use artificially created immersive audio. For the stereo case, there exists the ITU-T Reverbation Tool according to the STL G.191 [2] that could be used for this purpose. The proposed experiments below are based on the capabilities provided from this software tool, however, as is analyzed in the following, this comes with certain disadvantages (e.g. bandwidth limitations, test scenarios). The source is open to other proposed methods (HRTFs, tools), if available, to address these issues.

Proposed Test Plans

Generic Considerations for all Stereo experiments P800-1,2,3

Some generic considerations that are common for all tests are the following:

Room reverberance

Environments without and with reverberance should be covered in the tests.

Microphone setup

A/B, M/S and binaural microphone setups are realistic for conferencing scenarios in both using desktop microphones in conferencing rooms or using mobile handheld devices or earbuds for conferencing.

For A/B microphone setups, different distances between microphones should also be tested. For example, a 30 cm microphone distance is a realistic setup for handheld devices. However, it should be noted that the ITU-T audio software tool  [2], only provides HRTFs for two A/B setups with distance 150 and 100 cm (conferencing rooms).

Talker positions

Varying talker positions with respect to the capture point both in terms of direction and distance should be covered.

Overtalk

Overtalk (double talk) is frequently happening especially in conferencing scenarios. It should be tested in all three Stereo experiments 1,2 and 3 covering 1/3 of the categories.

Audio bandwidth

IVAS operation is required for WB, SWB and FB audio input-output. Since not all audio bandwidths can be covered in the selection tests, it is suggested for the Stereo clean and noisy speech experiments at SWB for the following reasons:

    - For bitrates up to 24.4kbps there is only SWB coding is available

    - Available ITU-T scripts [2] for creating the test material utilize HRTFs with 32kHz sampling rate

For mixed/music experiments audio bandwidth should be at maximum, i.e. FB

FER/Jitter

Good frame loss behavior is critical for the IVAS codec. It is proposed to cover frame loss in the clean speech and mixed/music experiment. It is suggested to use 5% error rate.

Experimental outline

Six categories of content types.

30 subjects, six listening panels (five subjects per panel), each panel with an independent randomization.

Six samples per category (one for each listening panel).

Randomizations constructed under “partially-balanced/randomized blocks” experimental design described in “Practical procedures for subjective testing”, [ITU-T Handbook, 2011].

Every condition has 36 different samples passed through it (6 categories x 6 panels). Each of these are voted on by the 5 subjects in the panel, giving: (36 samples x 5 subjects/panel) = 180 votes per condition.

36 test conditions x 6 blocks/categories = 216 DCR trials.

Expected average trial duration: 17 s (6.5 s reference sample +0.5 s silence + 6.5 s test sample + 3.5 s voting period).

Expected test duration: ~2 h per listening panel. Test duration comprises 50% of actual listening/voting time (60 min) and 50% test overhead including orientation, instructions, preliminaries, and rest breaks

The listening sessions should be split into a number of sub-sessions with breaks in between to allow for the subject to relax. This is to avoid listener fatigue.

Test Plan for Experiment P800-1: clean speech

Factors and conditions

| Main Codec Conditions |  |  |
| --- | --- | --- |
| Codec under Test (CuT) | 6 | IVAS candidate operated at 13.2, 16.4, 24.4, 32, 48 kbps with DTX off at 0% FER [IVAS candidate operated at 24.4 kbps with DTX on at 0% FER] |
|  | 6 | IVAS candidate operated at 13.2, 16.4, 24.4, 32, 48 kbps with DTX off at 5% FER [IVAS candidate operated at 13.2 kbps with DTX on at 5% FER] |
| Codec references |  |  |
| Codec references | 7 | Dual-mono EVS operated at 2*7.2, 2*8.0, 2*9.6, 2*13.2, 2*16.4, 2*24.4, 2*32 kbps, with DTX off at 0% FER |
|  | 7 | Dual-mono EVS operated at 2*7.2, 2*8.0, 2*9.6, 2*13.2, 2*16.4, 2*24.4, 2*32 kbps, with DTX off at 5% FER |
| Other references |  |  |
| Direct | 1 | Nominal input level |
| P.50 MNRU | 4 | Q=16, 20, 24, 28 dB (all: nominal level) |
| ESDRU [P.811] | 3 | α = 0.1, 0.4, 0.7 (output loudness forced to nominal level after application of ESDRU) |
| Common Conditions |  |  |
| Test item generation: pre-processing incl. spatialization | 1 | Model-based relying on convolution of raw mono clean speech sentences with Room Impulse Responses respective to various talker positions relative to a capture point as described in the ITU-T Reverberation Tool [2] |
| Audio sampling frequency/bandwidth | 2 | 32 kHz/maximum available audio bandwidth up to SWB |
| Content types (categories) | 6 | 6 Different environments and talker interactions |
| Kind of samples | 1 | Sentence pair uttered by different talkers and genders (3 male and 3 female) |
| Number of samples | 7 | per content type |
| Input frequency mask | 1 | Flat |
| Nominal output loudness | 1 | -26 LKFS ([ITU-R BS.1770-4]) |
| Listening Level | 1 | 73 dB SPL |
| Listeners | 30 | Naïve Listeners |
| Randomizations | 6 | 6 panels of 5 listeners |
| Rating Scale | 1 | DCR with modified instructions according to [P.SUPPL800] |
| Replications | 1 |  |
| Languages | 1 | [tbd] |
| Listening System | 1 | High-quality headphone for diotic presentation |
| Listening Environment | 1 | No room noise |

Content type categories and scene definitions (Exp P800-1: clean speech)

| Category | Room(1 | Reverb | Microphone Setup | Level [dB] | Overtalk [s](2 | Bandwidth( | Talker positions(3 | Talker selection by panel(4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cat 1 | small | anechoic | A-B (100cm) | -26 | 1 | Max [available up to SWB] | Random | P1: f1m1 P2: m2f2 P3: f3m3 P4: m1f1 P5: f2m2 P6: m3f3 |
| cat 2 | large | anechoic | A-B (150cm) | -26 | -1 | max [available up to SWB] | Random | P1: m3f3 P2: f1m1 P3: m2f2 P4: f3m3 P5: m1f1 P6: f2m2 |
| cat 3 | small | anechoic | M-S | -26 | -1 | max [available up to SWB] | Random | P1: f2m2 P2: m3f3 P3: f1m1 P4: m2f2 P5: f3m3 P6: m1f1 |
| cat 4 | small | echoic | A-B (100cm) | -26 | -1 | max [available up to SWB] | Random | P1: m1f1 P2: f2m2 P3: m3f3 P4: f1m1 P5: m2f2 P6: f3m3 |
| cat 5 | large | echoic | A-B (150cm) | -26 | 1 | max [available up to SWB] | Random | P1: f3m3 P2: m1f1 P3: f2m2 P4: m3f3 P5: f1m1 P6: m2f2 |
| cat 6 | large | echoic | Binaural | -26 | -1 | max [available up to SWB] | Random | P1: m2f2 P2: f3m3 P3: m1f1 P4: f2m2 P5: m3f3 P6: f1m1 |

Notes:

(1 The specific room characteristic and resulting reverb characteristic will be defined by the choice of the specific Spatial Room Impulse Responses used in the convolution process with the raw mono sentences.

(2 Overtalk [s] means the duration in seconds by which the two sentences in the sound item uttered by different talkers are overlapping. A negative number means that there is a corresponding pause between the two sentences.

(3 The talker positions are part of the scene definition of the different categories. They should be chosen in a random way from the available set of SRIRs for the used room making sure that there is a good coverage of different possible positions. Different random selections should be made for the different listener panels. The details will be specified in the IVAS processing plan IVAS-7a.

(4 All sentences by the 6 talkers shall be unique.

Test Plan for Experiment P800-2: noisy speech

Factors and conditions

| Main Codec Conditions |  |  |
| --- | --- | --- |
| Codec under Test (CuT) | 5 | IVAS candidate operated at 13.2, 16.4, 24.4, 32, 48 kbps with DTX off at 0% FER |
|  | 5 | IVAS candidate operated at 13.2, 16.4, 24.4, 32, 48 kbps with DTX on at 0% FER |
| Codec references |  |  |
| Codec references | 7 | Dual-mono EVS operated at 2*7.2, 2*8, 2*9.6, 2*13.2, 2*16.4, 2*24.4,2*32 kbps, with DTX off at 0% FER |
|  | 7 | Dual-mono EVS operated at 2*7.2, 2*8, 2*9.6, 2*13.2, 2*16.4, 2*24.4,2*32 kbps, with DTX on at 0% FER |
| Other references |  |  |
| Direct | 1 | Nominal input level |
| P.50 MNRU | 4 | Q=12, 17, 23, 28 dB (all: nominal level) |
| ESDRU [P.811] | 3 | α = 0.1, 0.4, 0.7 (output loudness forced to nominal level after application of ESDRU) |
| Common Conditions |  |  |
| Test item generation: pre-processing incl. spatialization | 1 | Model-based relying on convolution of raw mono clean speech sentences with Room Impulse Responses respective to various talker positions relative to a capture point as described in the ITU-T Reverberation Tool [2] |
| Audio sampling frequency/bandwidth | 2 | 32 kHz/maximum available audio bandwidth up to SWB |
| Content types (categories) | 6 | 6 Different environments with different background types (office, street, car) and talker interactions |
| Kind of samples | 1 | Sentence pair uttered by different talkers and genders (3 male and 3 female) |
| Number of samples | 7 | per content type |
| Input frequency mask | 1 | Flat |
| Nominal output loudness | 1 | -26 LKFS ([ITU-R BS.1770-4]) |
| Listening Level | 1 | 73 dB SPL |
| Listeners | 30 | Naïve Listeners |
| Randomizations | 6 | 6 panels of 5 listeners |
| Rating Scale | 1 | DCR with modified instructions according to [P.SUPPL800] |
| Replications | 1 |  |
| Languages | 1 | [tbd] |
| Listening System | 1 | High-quality headphone for diotic presentation |
| Listening Environment | 1 | No room noise |

Content type categories and scene definitions (Exp P800-2: noisy speech)

| Category | Room(1 | Reverb | Microphone Setup | Level [dB] | Background | SNR In dB | Overtalk [s](2 | Bandwidth( | Talker positions(3 | Talker selection by panel(4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cat 1 | small | anechoic | A-B (100cm) | -26 | car | 15 | 1 | Max [available up to SWB] | Random | P1: f1m1 P2: m2f2 P3: f3m3 P4: m1f1 P5: f2m2 P6: m3f3 |
| cat 2 | large | anechoic | A-B (150cm) | -26 | office | 15 | -1 | max [available up to SWB] | Random | P1: m3f3 P2: f1m1 P3: m2f2 P4: f3m3 P5: m1f1 P6: f2m2 |
| cat 3 | small | anechoic | M-S | -26 | street | 15 | -1 | max [available up to SWB] | Random | P1: f2m2 P2: m3f3 P3: f1m1 P4: m2f2 P5: f3m3 P6: m1f1 |
| cat 4 | small | echoic | A-B (100cm) | -26 | street | 15 | -1 | max [available up to SWB] | Random | P1: m1f1 P2: f2m2 P3: m3f3 P4: f1m1 P5: m2f2 P6: f3m3 |
| cat 5 | large | echoic | A-B (150cm) | -26 | office | 15 | 1 | max [available up to SWB] | Random | P1: f3m3 P2: m1f1 P3: f2m2 P4: m3f3 P5: f1m1 P6: m2f2 |
| cat 6 | large | echoic | Binaural | -26 | car | 15 | -1 | max [available up to SWB] | Random | P1: m2f2 P2: f3m3 P3: m1f1 P4: f2m2 P5: m3f3 P6: f1m1 |

Notes:

(1 The specific room characteristic and resulting reverb characteristic will be defined by the choice of the specific Spatial Room Impulse Responses used in the convolution process with the raw mono sentences.

(2 Overtalk [s] means the duration in seconds by which the two sentences in the sound item uttered by different talkers are overlapping. A negative number means that there is a corresponding pause between the two sentences.

(3 The talker positions are part of the scene definition of the different categories. They should be chosen in a random way from the available set of SRIRs for the used room making sure that there is a good coverage of different possible positions. Different random selections should be made for the different listener panels. The details will be specified in the IVAS processing plan IVAS-7a.

(5 All sentences by the 6 talkers shall be unique.

Test Plan for Experiment P800-3: mixed music

Factors and conditions

| Main Codec Conditions |  |  |
| --- | --- | --- |
| Codec under Test (CuT) | 6 | IVAS candidate operated at 13.2, 16.4, 24.4, 32, 48 kbps with DTX off at 0% FER [IVAS candidate operated at 24.4 kbps with DTX on at 0% FER] |
|  | 6 | IVAS candidate operated at 13.2, 16.4, 24.4, 32, 48 kbps with DTX off at 5% FER [IVAS candidate operated at 13.2 kbps with DTX on at 5% FER] |
| Codec references |  |  |
| Codec references | 7 | Dual-mono EVS operated at 2*7.2, 2*8, 2*9.6, 2*13.2, 2*16.4, 2*24.4,2*32 kbps, with DTX off at 0% FER |
|  | 7 | Dual-mono EVS operated at 2*7.2, 2*8, 2*9.6, 2*13.2, 2*16.4, 2*24.4,2*32 kbps, with DTX off at 5% FER |
| Other references |  |  |
| Direct | 1 | Nominal input level |
| P.50 MNRU | 4 | Q=12, 17, 23, 28 dB (all: nominal level) |
| ESDRU [P.811] | 3 | α = 0.1, 0.4, 0.7 (output loudness forced to nominal level after application of ESDRU) |
| Common Conditions |  |  |
| Audio sampling frequency/bandwidth | 2 | 48 kHz/maximum available audio bandwidth up to FB |
| Content types (categories) | 6 | Different music and mixed categories: classical music, modern vocal music, modern instrumental music, jingle, advertisement, trailer |
| Number of samples | 7 | per content type |
| Input frequency mask | 1 | Flat |
| Nominal output loudness | 1 | -26 LKFS ([ITU-R BS.1770-4]) |
| Listening Level | 1 | 73 dB SPL |
| Listeners | 30 | Naïve Listeners |
| Randomizations | 6 | 6 panels of 5 listeners |
| Rating Scale | 1 | DCR with modified instructions according to [P.SUPPL800] |
| Replications | 1 |  |
| Languages | 1 | [tbd] |
| Listening System | 1 | High-quality headphone for diotic presentation |
| Listening Environment | 1 | No room noise |

Content type categories and scene definitions (Exp P800-3: mixed music)

| Category | Content Type | Level [dB] | Bandwidth( |
| --- | --- | --- | --- |
| cat 1 | Classical music | -26 | Max FB |
| cat 2 | Modern vocal music | -26 | Max FB |
| cat 3 | Modern instrumental music | -26 | Max FB |
| cat 4 | Jingle | -26 | Max FB |
| cat 5 | Advertisement | -26 | Max FB |
| cat 6 | Trailer | -26 | Max FB |

# References

S4-230025: IVAS Permanent Document IVAS-8a: Test Plan for Selection Phase, v.0.7.4.

ITU-T Recommendation G.191: Software tools for speech and audio coding standardization