Source:	Ericsson LM

Title:	Proposed test plan for IVAS selection experiment P800-3

## Document for:	Discussion and Agreement

## Agenda Item:	7.5

Introduction

The current draft IVAS test plan in IVAS-8a [1] lists tentative subjective experiments in Table 4 and Table 5. More detailed outlines of the experiments are however yet missing. This contribution suggests an initial test plan for the mixed and music stereo P.800 experiment, P800-3. This experiment assumes pre-mixed content in contrast to the other P.800 experiments where test material is assumed to be generated from mono samples and stereo/spatial backgrounds. The proposal aims to test the stereo requirements of IVAS-3 [2] for bitrates up to 48 kbps, in accordance with [1]. The mono compatible requirements are not covered in the proposed experiment.

Proposal

It is proposed to integrate the following draft test plan for the P800-3 test (clause X.3) into IVAS-8a. The proposal is structured in a similar way as experiments were outlined for the EVS codec in EVS-8b but should be aligned with other IVAS test plans when available.

References

Tdoc S4-230453, IVAS Permanent Document IVAS-8a: Test Plan for Selection Phase, v.0.7.4

Tdoc S4-230368, IVAS Performance Requirements (IVAS-3), v.1.0.0

X.3	Experiment P800-3: Mixed & Music

The purpose of this experiment is to evaluate the performances of the IVAS candidate algorithm with respect to EVS reference conditions in FB mixed content and music for clean and impaired channel conditions [including delay/jitter].

X.3.1	Experiment setup

Tables X.3.1 to X.3.3 show conditions to be used for this experiment, list of preliminaries and full list of conditions, respectively.

Table X.3.1: Factors and conditions for Experiment P800-3

| Main Codec Conditions |  |  |
| --- | --- | --- |
| Candidate | 1 | CuT |
| Operating modes DTX | 5 0 | 13.2, 16.4, 24.4, 32 and 48 DTX off |
| Input level | 1 | -26 LKFS |
| Input frequency mask | 0 | Flat |
| Noise | 0 | No noise |
| Error Conditions | 2 | 0%, [3% random frame erasures / 1 delay/jitter profile] |
| Codec references |  |  |
| Codec references | 7 | EVS dual mono |
| Input level DTX | 1 0 | -26 LKFS DTX off |
| Input frequency mask | 0 | Flat |
| Noise | 0 | No noise |
| Error Conditions | 2 | 0%, [3% random frame erasures / 1 delay/jitter profile] |
| Other references |  |  |
| Direct | 1 | -26 LKFS |
| P.50 MNRU ESDRU | 5 5 | Q = 10, 16, 22, 28, 34 dB α = 0, 0.18, 0.35, 0.53, 0.7 |
| Input frequency mask | 0 | Flat |
| Common Conditions |  |  |
| Number of categories | 6 | 3 mixed content and 3 music (See clause 4.5.3) |
| Number of samples | 7 | 6 + 1 (preliminaries) samples per category |
| Listening Level | 1 | 73 dB SPL |
| Listeners | 30 | Naïve listeners |
| Randomizations | 6 | 6 panels of 5 listeners |
| Rating Scale | 1 | DCR |
| Listening System | 1 | Headphones, in accordance with clause 4.6 |
| Listening Environment | 1 | No room noise, in accordance with clause 4.6 |

Table X.3.2: Preliminaries for Experiment P800-3

| Trial # | Label | Sample | Condition | Bitrate | FER/Profile |
| --- | --- | --- | --- | --- | --- |
| 1 | c22 |  | EVS | 2x13.2 | [3%, profile] |
| 2 | c02 |  | MNRU Q=34 dB | - | - |
| 3 | c07 |  | ESDRU α = 0.7 | - | - |
| 4 | c16 |  | EVS | 2x16.4 | No errors |
| 5 | c11 |  | ESDRU α = 0 | - | - |
| 6 | c19 |  | EVS | 2x7.2 | [3%, profile] |
| 7 | c04 |  | MNRU Q=22 dB | - | - |
| 8 | c01 |  | Reference | - | - |
| 9 | c18 |  | EVS | 2x32 | No errors |
| 10 | c09 |  | ESDRU α = 0.35 | - | - |
| 11 | c06 |  | MNRU Q=10 dB | - | - |
| 12 | c24 |  | EVS | 2x24.4 | [3%, profile] |

Table X.3.3: Test conditions for Experiment P800-3,
mixed contents and music under clean and impaired channel conditions [including delay/jitter profiles]

| Label | Condition | Bitrate [kbps] | FER/Profile | ToR |
| --- | --- | --- | --- | --- |
| c01 | Reference | - | - |  |
| c02 | MNRU Q=34 dB | - | - |  |
| c03 | MNRU Q=28 dB | - | - |  |
| c04 | MNRU Q=22 dB | - | - |  |
| c05 | MNRU Q=16 dB | - | - |  |
| c06 | MNRU Q=10 dB | - | - |  |
| c07 | ESDRU α=0.7 | - | - |  |
| c08 | ESDRU α=0.53 | - | - |  |
| c09 | ESDRU α=0.35 | - | - |  |
| c10 | ESDRU α=0.18 | - | - |  |
| c11 | ESDRU α=0.0 | - | - |  |
| c12 | EVS | 2x7.2 | No errors |  |
| c13 | EVS | 2x8 | No errors |  |
| c14 | EVS | 2x9.6 | No errors |  |
| c15 | EVS | 2x13.2 | No errors |  |
| c16 | EVS | 2x16.4 | No errors |  |
| c17 | EVS | 2x24.4 | No errors |  |
| c18 | EVS | 2x32 | No errors |  |
| c19 | EVS | 2x7.2 | [3%, profile] |  |
| c20 | EVS | 2x8 | [3%, profile] |  |
| c21 | EVS | 2x9.6 | [3%, profile] |  |
| c22 | EVS | 2x13.2 | [3%, profile] |  |
| c23 | EVS | 2x16.4 | [3%, profile] |  |
| c24 | EVS | 2x24.4 | [3%, profile] |  |
| c25 | EVS | 2x32 | [3%, profile] |  |
| c26 | CuT | 13.2 | No errors | NWT c13 OR BT c12 |
| c27 | CuT | 16.4 | No errors | NWT c14 OR BT c13 |
| c28 | CuT | 24.4 | No errors | NWT c15 OR BT c14 |
| c29 | CuT | 32 | No errors | NWT c16 OR BT c15 |
| c30 | CuT | 48 | No errors | NWT c18 OR BT c17 |
| c31 | CuT | 13.2 | [3%, profile] | NWT c20 OR BT c19 |
| c32 | CuT | 16.4 | [3%, profile] | NWT c21 OR BT c20 |
| c33 | CuT | 24.4 | [3%, profile] | NWT c22 OR BT c21 |
| c34 | CuT | 32 | [3%, profile] | NWT c23 OR BT c22 |
| c35 | CuT | 48 | [3%, profile] | NWT c25 OR BT c24 |