Agenda item: 	10.8

Source: 	InterDigital Communications

Title: 	[5G_RTP] UPF configuration for PDU set information identification

Document for:	Discussion and Agreement

# Introduction

SA4 has agreed to work on the RTP HE for PDU set marking providing the following PDU set information as part of the RTP HE fields as:

- PDU set sequence number (PSSN)

- PDU sequence number (PSN) within a PDU set

- End PDU of a PDU set indication (E)

- PDU set importance (PSI)

- PDU end-of-data burst (EoB) indication

- Optionally, PDU set size (PSS) in bytes

In S4aR230057, the RTC working group has discussed that the RTP HE local identifier is not a fixed value and is negotiated during the SDP. It is not clear how this information can be communicated to the UPF.

In this contribution, we discuss how a UPF identifies the presence of a PDU set information RTP header extension in the RTP packets associated with the PDUs/ PDU sets.

# UPF configuration for PDU set information identification

For a given QoS flow, the AF may configure PDU set information RTP HE usage in an RTP session in PCF, e.g., through the NEF. The PDU set information RTP HE usage is negotiated between AS and AF through M3 or RTC-3 interface, configured in the PCF (e.g., in the PCC rule), transferred by PCF to SMF (e.g., in the PCC rule), and transferred by SMF to UPF (e.g., in an N4 message).

Figure 1 illustrates an example procedure for configuring the UPF.

Figure 1. Example Procedure for Configuring the UPF with PDU set information RTP HE usage

- In step 1a, the AS and AF negotiates the usage of RTP HE for transmitting the PDU set information between AS and UPF during a media session and informs the network about the support for PDU set information. In an example, the information of the RTP header extension ID generated in the RTP Session creation time is signaled to the AF so that this information is carried to the UPF through PCF and SMF. In another example, the RTP header extension ID is generated and signalled by AF to AS and this ID is used by AS during the RTP Session creation time.

- In step 2a, the AF invokes an API (e.g., an Nnef_AFsessionWithQoS) to provide the network with the usage of PDU set information RTP HE. In step 2b, the NEF may respond to the API call.

- In steps 3, 4 and 5, the NEF invokes an API to provide the PDU set information RTP HE to the PCF, PCF may send a PCC rule or a notification to the SMF and finally SMF sends the PDU set information RTP HE usage information to the UPF.

# Proposal

We propose to agree the proposed signaling method between AS and UPF about the usage of PDU set information RTP header extension in 5G_RTP permanent document.