3GPP TSG SA WG4#125 meeting                                                                      S4-231181

Goteborg, Sweden, 21-25 August 2023

Source:	Qualcomm

Title:	[FS_eiRTCW] Key Issue#5 Tethered Cases

Document for:	Agreement

Agenda item	10.9

# Background

The 3GPP SA4 SmarTAR study item investigated the session setup, media handling and QoS for the tethering case where, for example, a pair of AR glasses is tethered to a phone which is connected to a 5G network, as documented in TR 26.806 [1]. The tethering link may be a 3GPP sidelink or a non-3GPP link such as Wi-Fi. The work there was in generic terms, without targeting a concrete implementation. With eiRTCW, which focuses on WebRTC, the work done in SmarTAR needs to be particularized and/or further developed for the WebRTC framework currently under development in eiRTCW. We propose updates to the eiRTCW PD.

# Proposed changes

# * * * * 1st change * * * *

## 5.6	Key Issue #5: Tethered Cases

Editor’s note: Key issue need to be described. The title of this key issue is re-considered corresponding to concluded issues.

Editor’s Note: SmarTAR-related clause;
Identify enhancements for E2E QoS realizations over 5G systems for communications between MNOs and WebRTC clients operating over non-5G links (e.g., Wi-Fi) using WebRTC-based transport.  This also includes communication between WebRTC clients operating on tethering/tethered devices.

AR glasses may have limited computing and communication capabilities due to the size and weight constraints. AR glasses can be tethered to a smart phone to tap the latter’s better capabilities. The tethering case was studied in the SA4 SmarTAR study item and the conclusions are documented in TR 26.806 [1]. There are two network architectures relevant to eiRTCW. Fig. 5.6-1 shows an example of a network architecture where only one MNO is involved.

Figure 5.6-1: A tethering case with only one MNO involved.

Fig. 5.6-2 shows an example of another network architecture with the involvement of two MNO’s (MNO A and MNO B) in the E2E path.

Figure 5.6-2: A tethering case with only one MNO involved.

The key issue may be decomposed into several sub-issues:

- Key issue #5.1: Should the WebRTC Endpoint be on the tethered device or the tethering device for the scenarios considered in eiRTCW?

To elaborate, in Figure 5.6-1, one of the WebRTC Endpoint is on the Application Server, but it is not clear where the other WebRTC Endpoint should be.

- Key issue #5.2: Is the signaling for the cases without tethering sufficient for the tethered cases?

- Key issue #5.3: How to provide E2E QoS when there are non-3GPP networks also involved?

# * * * * End of 1st change * * * *

# * * * * 2nd change * * * *

## 6.6	Solution #5: Tethered Cases

### 6.6.1	Solution description

This solution addresses key issue #5.

Editor’s Note: SmarTAR-related clause;
Identify enhancements for E2E QoS realizations over 5G systems for communications between MNOs and WebRTC clients operating over non-5G links (e.g., Wi-Fi) using WebRTC-based transport.  This also includes communication between WebRTC clients operating on tethering/tethered devices.

For key issue #5.1, there are two design options:

- Solution #5.1.1: the WebRTC Endpoint resides on the tethering device (e.g., on the phone).

- Solution #5.1.2: the WebRTC Endpoint resides on the tethered device (e.g., on the AR glasses).

For key issue #5.2, if solution #5.1.1 is chosen to address key issue #5.1, then we have the following solution:

- Solution #5.2.1: the signaling for the case without tethering can be reused for the tethered case given Solution # 5.1.

### 6.6.x	Solution evaluation

Editor's note:	This clause provides an evaluation of the solution.

Solution #5.1.1 is justified for the following reason:

- For collaboration scenarios 3 and 4 considered in eiRTCW and TS 26.506, the tethered device is not a 5G device, but the WebRTC Endpoint needs to communicate over the RTC-5 interface and the RTC-4 interface with the RTC AF and the RTC AS, respectively. Therefore, the tethered device cannot host the WebRTC Endpoint.

Solution #5.1.2 is justified for the following reason:

- For collaboration scenario 1, i.e., the OTT scenario, the WebRTC Endpoint can be on the tethered device.

Note: For the 5GS to provide service beyond the best effort service to the WebRTC session, information about the WebRTC traffic flows need to be conveyed to the 5GS.

FFS: Feasibility for the techered device to convey traffic flow information to the 5G system if the WebRTC Endpoint is on the techered device.

Solution #5.2.1 is justified for the following reason:

- Because of Solution # 5.1.1, from the perspective of the WebRTC Endpoint, the interfaces toward the 5GS stay the same, and hence the call flows and message formats can be reused.

# * * * * End of 2nd change * * * *

# References

[1] 3GPP TR 26.806, "Study on Tethering AR Glasses – Architectures, QoS and Media Aspects", V18.0.0 (2023-06).