3GPP TSG-SA4 Meeting #127-bis-e	S4-240672

# April 8 – 12 April 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.8xx | pCR | xxxx | rev | - | Current version: | 0.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of application-layer FEC schemes for Key issue #3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm, Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5G_RTP |  |  |  |  |  | Date: |  |  | 2024-04-02 |
| 3 |  |  |  |  |  |  |  |  |  |  |
| Category: | B |  |  |  |  |  | Release: |  |  | 19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The information about Application-layer (AL) FEC in the SID lacks details. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Populate empty clause 5.3 to describe Key Issue #3 Provide more information about influential AL FEC schemes |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TR is not complete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.2.0, 10.2.1.12 (new), 10.2.2, 10.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** First change ***

## 5.3	Key Issue #3: Enhancements for application-layer FEC support

### 5.3.1	Description

According to clause 5.7.4 of TR 26.926 [6], commercial XR split rendering and cloud gaming services use Application Layer Forward Error Correction (FEC). This clause also introduces several RTP based FEC schemes defined by IETF primarily to be used in WebRTC. It is worthwhile to study if any of these FEC schemes can be added to 3GPP specifications, for example to support split rendering.

*** Next change ***

## 6.3	Solution #1: introduction of AL-FEC schemes defined in IETF

### 6.3.1	Key Issue mapping

This maps to Key Issue #3

### 6.3.2	Description

### IETF defined a few AL-FEC schemes including the codes, packet formatting and transmission methods, as detailed below. Some of them are Maximum Distance Seperable (MDS) codes, meaning that they enable a receiver to recover the k source symbols from any set of k received symbols.

- Non-MDS FEC schemes:

  - FlexFEC: or Flexible Forward Error Correction, as defined in RFC 8627

    - FlexFEC relies on XOR operation in generagig repair packets from source packets.

    - FlexFEC currently is supported in the WebRTC implementation.

    - The encoding may be done in 1-dimenstional or 2-dimensional fashion.

    - A repair packet may prototect a limited number of source packets.

    - In the WebRTC implementation, the amount of redundancy depends on the packet loss rate, bitrate and RTT.

    - The source packets have the same RTP packet format as regular packets without FEC, and the repair packets carry encoding information in the FEC Header (shown below) indicating which of the source packets are protected by this repair packet.

      - Note that the FEC Header is part of the RTP payload and becomes invisible in the case of SRTP.

Figure 6.1: RTP packet format for the repair packet for FlexFEC.

  - ULPFEC: or Uneven Level Protection Forward Error Correction, as defined in RFC 5109

    - ULPFEC is similar to FlexFEC in the encoding operation but has the additional feature of providing multiple FEC levels for different parts of an application data unit.

    - ULPFEC currently is supported in the WebRTC implementation.

    - The source packet (called media packet in RFC 5109) follows the same RTP packet format without FEC, and the repair packet (called FEc packet in RFC 5109) follow the format shown below. Note that multiple FEC levels (protection levels) are supported.

      - Again, the FEC Headers will be invisible in the case of SRTP.

Figure 6.2 RTP packet format for ULPFEC

- MDS or near-MDS schemes:

  - Reed-Solomon (RS) FEC: defined in RFC 5510 and RFC 6865.

    - RS FEC codes are MDS.

    - They are commercially deployed in for example Meta Messenger.

    - The srouce packet format and the repair packet format are shown in Figure 6.3.

Figure 6.3 Format of the source packet and repair packet for RS FEC

  - Raptor: defined in RFC 5053.

    - Raptor is a fountain code, i.e., as many encoding symbols as needed can be generated by the encoder on-the-fly from the source symbols of a source block of data. The decoder can recover the source block from any set of encoding symbols only slightly more in number than the number of source symbols.

  - RaptorQ: defined in RFC 6330.

    - RaptorQ is a fountain code.

    - RaptorQ codes provide superior flexibility, support for larger source block sizes, and better coding efficiency than Raptor codes.

*** End of changes ***