Source:		Xiaomi (PD editor)

Title:			VOPS Permanent Document v0.2.0

Version:		0.2.0

Agenda Item:		9.5

Document for:		Information

Work Item information:

| Work Item Name | Video Operating Points - Harmonization and Stereo MV-HEVC |
| --- | --- |
| Acronym | VOPS |
| Unique ID | 1030002 |
| Target release | 19 |

Source details:

| Rapporteur | Name | Waqar Zia |
| --- | --- | --- |
|  | Company | Apple Inc. |
|  | Email | waqar_zia@apple.com |
| Editor | Name | Emmanuel Thomas |
|  | Company | Xiaomi |
|  | Email | thomase@XIAOMI.COM |

Revision history:

| Version | Date | Meeting | Subject/Comment |
| --- | --- | --- | --- |
| 0.1.0 | 2025-05-20 | SA4#132 | - Added - Conformance framework in clause 3.1. - Conformance content in clause 4.1.1, 4.1.2, 4.1.3. |
| 0.2.0 | 2025-07-09 | SA4-(AH) Video SWG post 132 | - Added: - Bitstream validator description and repository - Modified: - Background on conformance from externa organisation moved to Annex |

# Contents

Contents	2

1	Introduction	3

2	Definitions, symbols and abbreviations	3

2.1	Definitions	3

2.2	Abbreviations	3

3	Conformance Requirements	3

4	Conformance Framework	3

4.1 Sample bitstream platform overview	3

4.2 The bitstream validator	4

4.2.1 General	4

4.2.2 Code repository	4

4.2.3 Usage	5

4.2.4 VOPS operation points as XML schema	5

4.3 The bitstream database	6

4.4 The bitstream hosting server	6

4.5 The public portal	6

5	Conformance Material	6

5.1 HEVC Conformance Material	6

5.1.1 Source content	6

5.1.2 Compressed Bitstreams	6

5.1.3 Reference Software	8

6	References	8

Annex A – Background on other validations by	9

A.1 Background on the DASH-IF conformance suite	9

# Introduction

During SA4#132 it was considered that it is necessary to gather all material related to conformance provided to SA4 meetings. This permanent document is created to serve this purpose.

# 2	Definitions, symbols and abbreviations

## 2.1	Definitions

## 2.2	Abbreviations

# 3	Conformance Requirements

This subclause gathers requirements for conformance testing. This includes features that need to be covered by the conformance.

# 4	Conformance Framework

This subclause gathers documentation on a potential framework for conformance, e.g. tools to tabulate, gather, and present conformance content.

## 4.1 Sample bitstream platform overview

At high level, the sample bitstream platform is composed of three main components that are:

- A database which contains the description of the available sample bitstreams

- One or more servers to host the submitted bitstreams

- A portal for external users to search through and download the sample bitstreams.

In addition, a bitstream validator validates prior to upload that the submitted bitstream does comply with the constraints defined in TS 26.265. To harmonize the bitstream description in the database, the bitstream validator would also generate a bitstream description file for the bitstream database.

The bitstream database should contain the description of each available bitstream. In addition, each bitstream would be linked to a TS number and possibly a profile defined within this TS (using URNs defined to identify each profile).

Since using JSON (or other markup text file) for this purpose seems advantageous, the bitstream database could simply be a git repository on any web-base git service platform (e.g. GitHub, GitLab, etc…)

For storing the bitstreams, any http server could be used. Using git LFS would add a finer control with versioning functions but also comes with added constraints for space and effort to set-up. This does not seem necessary to use git LFS at this point.

Ideally, the external users would be able to search for TS numbers, profiles and retrieve the corresponding bitstreams. Additionally, it should also be possible to browse the database by TS number, codec, etc…

In a first version, the home page of the git repository (e.g. GitHub repository’s home page) could be sufficient. It times allows a simple static web frontend could also be developed. The web frontend would retrieve the information about the available bitstream from the database and present them to the user. Using static website would also have the advantage to maintain them on the same web-base git service provider, i.e. GitHub. GitLab, etc…

Figure 1 - Overall view of the sample bitstream platorm

## 4.2 The bitstream validator

### 4.2.1 General

The bitstream validator shall:

- validate that the bitstream is a compliant bitstream according to the corresponding video coding specification and profile.

- validate that the bitstream is compliant with the bitstream constraints defined in TS 26.265.

For the point 1), the reference video decoder developed by JVET has conformance checks embedded. As a result, one way to achieve this validation is to execute the reference decoder with no picture output and verify the absence of error.

For the point 2), there is a need to programmatically express the constraints defined in TS 26.265 to validate the input bitstream. To this end, the input bitstream is first parsed and a XML dump of the bitstream is generated which contains the relevant syntax element. Then, the set of VOPS constraints expressed as XML constraints can be validated against this XML representation of the bitstream.

This approach has the advantage of i) being codec agnostic, i.e. the constraint document could be used for any codec, ii) writing constraints does not require to be familiar with language programming, and iii) the bitstream description generated for the validation can be reused for the database in order to search through the different bitstreams.

### 4.2.2 Code repository

The bitstream validator is hosted on the 3GPP forge at this address: https://forge.3gpp.org/rep/thomasem/sa4-bitstream-validator

NOTE	The repository will be moved to the SA4 namespace by the maintainer of the SA4 group on the 3GPP forge.

### 4.2.3 Usage

The project folder contains two main assets which are:

- The Python module to execute the validation (/sa4_bitstream_validator)

- The folder containing XML schema defining VOPS operation points constraints (/bitstream_rules)

The firs step to validate a bitstream is to generate its XML description via the command ‘dump’.

$ python -m sa4_bitstream_validator dump bitstream_path description.xml

Then followed by the validation step of the XML description generated by the previous step:

$ python -m sa4_bitstream_validator validate description.xml bitstream_rules/a_vops_operation_point.xsd

### 4.2.4 VOPS operation points as XML schema

As explained above, the validation of VOPS bitstream constraints are implemented as XML schema applied to XML description of the tested bitstreams.

The folder /bitstream_rules collects those XML schemas. Those XML schemas use the specification XSD 1.1 which provides the ability to express assertions with the element xs:assert. In addition, the XML schema which described the HEVC XML structure is defined in the hevc.xsd which can be included in each VOPS operation point schemas for efficient reuse.

Here is an example of such XML schema:

<?xml version="1.0" encoding="UTF-8"?>

<xs:schema

xmlns:xs="http://www.w3.org/2001/XMLSchema"

xmlns:vc="http://www.w3.org/2007/XMLSchema-versioning"

elementFormDefault="qualified"

attributeFormDefault="unqualified"

vc:minVersion="1.1">

<xs:include schemaLocation="hevc.xsd"/>

<xs:element name="HEVCBitstream">

<xs:complexType>

<xs:sequence>

<xs:element ref="NALUnit" maxOccurs="unbounded"/>

</xs:sequence>

<xs:attribute name="uri" type="xs:anyURI"/>
	<!—The assertion requires the first NAL unit to be a VPS-->

<xs:assert test="exists(NALUnit[1]/VideoParameterSet)"/>
	<!—The assertion requires the vps_max_layers_minus1 in the VPS to be equal to 35 -->

<xs:assert test="NALUnit[1]/VideoParameterSet/vps_max_layers_minus1 eq 35"/>

</xs:complexType>

</xs:element>

</xs:schema>

NOTE	This schema is purely illustrative and does not represent an actual VOPS operation point.

## 4.3 The bitstream database

TBD

## 4.4 The bitstream hosting server

TBD

## 4.5 The public portal

TBD

# 5	Conformance Material

This subclause gathers all materials e.g. source content, encoding, decoding tools and conformance software.

## 5.1 HEVC Conformance Material

### 5.1.1 Source content

#### 5.1.1.1. Polytech Nantes database

Uncompressed stereoscopic 3D video content dataset which can be used to, for example, generate MV-HEVC conformance streams by Polytech Nantes was pointed to in SA4 contribution [1]. At the time of writing this paper, all the links provided were down. This has already been observed in the past. The content itself, composed of 31 sequences, 1920 x 1080, 10-bit 4:2:2 YUV at 25 fps, was made freely available via the Creative Commons Attribution-ShareAlike License, and hence suitable for 3GPP SA4 purposes.

At the time being the proposal is:

- Rechecking if this resource gets back online

- Checking with the source

- Check if this can be moved to a more stable home.

### 5.1.2 Compressed Bitstreams

#### 5.1.2.1 JVET1

A set of conformance bitstreams (elementary streams) for testing Multiview Extended 10 and Multiview Main 10 profile are provided by JVET [2]. The streams themselves are available for download at [3]. The various parameters of the bitstream including the profile to tested, resolution of content, frame rate and number of frames are tabulated below.

|  |  |  |  |  |  | Base layer profile |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Categories | Subcategory | Conformance Profile | Bitstream | File name | Resolution | Main | Main 10 | Level | Frame rate (Frames/sec) |  | # of frames |
| Prediction Structure | Inter-view prediction | Multiview Extended 10 profile | MVHEVCS-J | MVHEVCS_J_APPLE_2 | 1024x768 |  | X | 4 | 30 |  | 300 |
| (2-view) | Inter-view prediction | Multiview Extended 10 profile | MVHEVCS-K | MVHEVCS_K_APPLE_2 | 1024x768 |  | X | 4 | 30 |  | 300 |
|  | Inter-view prediction | Multiview Extended profile | MVHEVCS-L | MVHEVCS_L_Bytedance_2 | 1024x768 | X |  | 4 | 30 |  | 300 |
|  | Inter-view prediction | Multiview Extended profile | MVHEVCS-M | MVHEVCS_M_Bytedance_2 | 1024x768 | X |  | 4 | 30 |  | 300 |
|  | Inter-view prediction | Multiview Main 10 profile | MVHEVCS-N | MVHEVCS_N_APPLE_1 | 1024x768 |  | X | 4 | 30 |  | 300 |
|  | Inter-view prediction | Multiview Main 10 profile | MVHEVCS-O | MVHEVCS_O_APPLE_1 | 1920x1088 |  | X | 4 | 30 |  | 300 |

#### 5.1.2.2 MV-HEVC stream

An MV-HEVC UHD streaming example is available at [4]. This stream playable only on specific devices listed on the page.

### 5.1.3 Reference Software

- Reference software for non-multiview content is HM reference software for HEVC [5]

- For multi-view content the reference software is: HEVC multi-view (MV-HEVC) and 3D video coding (3D-HEVC) extensions HTM reference software [6]

# 6	References

[1] S4-231297 [FS_HEVC_Profiles] Discussion source MV-HEVC content, Goteburg, August 2023

[2] Conformance testing for HEVC multiview extended and monochrome profiles, JVET-AM1008

[3] [Online 5.2025]: https://www.itu.int/wftp3/av-arch/jvet-site/bitstream_exchange/HEVCMultiview/under_test/

[4] Streaming Examples [Online 5.2025]: https://developer.apple.com/streaming/examples/

[5] HEVC HM reference software [Online 5.2025]: https://vcgit.hhi.fraunhofer.de/jvet/HM

[6] HEVC multi-view (MV-HEVC) and 3D video coding (3D-HEVC) extensions HTM reference software [Online 5.2025]: https://vcgit.hhi.fraunhofer.de/jvet/HTM

[7] Dash-Industry-Forum/DASH-IF-Conformance., https://github.com/Dash-Industry-Forum/DASH-IF-Conformance/

[8] gpac/gpac: GPAC Ultramedia OSS for Video Streaming & Next-Gen Multimedia Transcoding, Packaging & Delivery, https://github.com/gpac/gpac

# Annex A – Background on other validations by

## A.1 Background on the DASH-IF conformance suite

The DASH-IF Conformance test suite [7] is a collection of tools to validate DASH MPD and segments. For segments, the ISOSegmentValidator parses ISOBMFF segments and validates their structures. Since NAL units can be present in decoder configuration box, this tool has some parsing functionality of NAL units. However, it does not implement video bitstream validation.

Other open-source tools such as GPAC and its MP4Box tool have the capability to parse NAL units and optionally to dump to an XML format the content of a video bitstream.