3GPP TSG-SA4 Meeting #133-e	S4-251247

# Online, , 18th Jul 2025 - 25th Jul 2025										revision of S4aI250104

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.143 | CR | 0005 | rev | 1 | Current version: | 18.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [MeME-MED] Media Messaging Enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | MeME-MED |  |  |  |  |  | Date: |  |  | 2025-07-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Additional stage-3 work has been recommended in clause 6.3.2 of TR 26.841: 4. For Key Topic #6: Media Service Enabler as introduced in clause 5.6 and based on the conclusions in clause 5.6.4: |  |  |  |  |  |  |  |  |
| Summary of change: |  | For Key Topic #6: Media Service Enabler as introduced in clause 5.6 and based on the conclusions in clause 5.6.4 of TR 26.841: a. Connect TS 26.143 to 3GPP TR 26.857 that it fulfils some MSE concepts b. Add a call flow to TS 26.143 aligned with what is in clause 4.1 of TR 26.841 c. Add a set of stage-2 APIs and parameter that can be assigned to player and generator d. Add more examples for valid content |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work item objectives not fulfilled. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.2, 4.6 (new), Annex B, Annex D (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## =====  CHANGE =====

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	Khronos glTF 2.0, glTF™ 2.0 Specification (khronos.org)

[3] 	ISO/IEC 23090-14 AMD 2, Information technology — Coded representation of immersive media — Part 14: Scene description — Amendment 2: Support for haptics, augmented reality, avatars, Interactivity, MPEG-I audio, and lighting

[4]	3GPP TS 26.511: "5G Media Streaming (5GMS); Profiles, Codecs and Formats".

[5]	3GPP TS 26.117: "5G Media Streaming (5GMS); Speech and audio profiles".

[6]	IETF Draft draft-ietf-mimi-content-01: "More Instant Messaging Interoperability (MIMI) message content", 	Rohan Mahy

[7]	3GPP TS 22.140: "Multimedia Messaging Service (MMS); Stage 1".

[8]	Open Mobile alliance, "MMS Architecture" OMA-AD-MMS-V1_3-20110913-A.

[9]	Open Mobile alliance, "Multimedia Messaging Service Encapsulation Protocol" OMA-TS-MMS_ENC-V1_3-20110913-A.

[10]	GSMA "RCS Universal Profile Service Definition Document", Version 2.6, 19 December 2022.

[11]	GSMA PRD RCC.07 version 13.0 - "Rich Communication Suite - Advanced Communications Services and Client Specification" 19 December 2022.

[12]	IETF RFC 2046, "Multipurpose Internet Mail Extensions (MIME) Part Two: Media Types".

[13]	ISO/IEC 14496-12: "Information technology - Coding of audio-visual objects -Part 12: ISO base media file format".

[14]	ISO/IEC 23000-24:2023 Preliminary Draft of: Information technology — Multimedia application format (MPEG-A) — Part 24: Messaging Media Application Format (MeMAF) ".

NOTE: 	A preliminary draft of this standard is available as MDS23345_W03_N1082 here: https://www.mpeg.org/wp-content/uploads/mpeg_meetings/144_Hannover/w23345.zip

[15]	3GPP 23.140: "Multimedia Messaging Service (MMS); Functional Description; Stage 2".

[16]	ITU-T Recommendation T.81: "Information technology; Digital compression and coding of continuous-tone still images: Requirements and guidelines".

[17]	"JPEG File Interchange Format", Version 1.02, September 1, 1992.

[18]	"Exchangeable image file format for digital still cameras: EXIF 2.2", Specification by the Japan Electronics and Information Technology Industries Association (JEITA), April 2002, URL: http://www.exif.org/

[19]	CompuServe Incorporated: "GIF Graphics Interchange Format: A Standard defining a mechanism for the storage and transmission of raster-based graphics information", Columbus, OH, USA, 1987.

[20]	Compuserve Incorporated, Columbus, Ohio (1990): "Graphics Interchange Format (Version 89a)".

[21]	IETF RFC 2083: "PNG (Portable Networks Graphics) Specification version 1.0 ", T. Boutell, et. al., March 1997.

[22]	ISO/IEC 23000-22:2019 Information technology — Multimedia application format (MPEG-A) — Part 22: Multi-image application format (MIAF)

[23]	IETF RFC 2045, "Multipurpose Internet Mail Extensions (MIME) Part One: Format of Internet Message Bodies", November 1996

[24]	ISO/IEC 23008-12:2019 Information technology — High efficiency coding and media delivery in heterogeneous environments — Part 12: Image File Format

[25]	ITU-T Recommendation H.265 (02/2018): "High efficiency video coding".

[26]	3GPP TS 26.244: "Transparent end-to-end packet switched streaming service (PSS); 3GPP file format (3GP)"

[27]	3GPP TS 26.245: "Transparent end-to-end packet switched streaming service (PSS); Timed text format"

[28]	ISO/IEC 14496-30: "Information technology - Coding of audio-visual objects - Part 30: Timed text and other visual overlays in ISO base media file format".

[29]	IETF RFC 2387, "The MIME Multipart/Related Content-type"

[30]	IETF RFC 6381, "The 'Codecs' and 'Profiles' Parameters for "Bucket" Media Types"

[31]	3GPP TS 26.307, "Presentation Layer for 3GPP Services"

[32]	3GPP TS 26.140, "Multimedia Messaging Service (MMS); Media formats and codecs"

[33]	IETF RFC 2077, "The Model Primary Content Type for Multipurpose Internet Mail Extensions"

[34]	3GPP TS 26.119, "Media Capabilities for Augmented Reality"

[35]	3GPP TR 26.857, "5G Media Service Enablers"

## =====  CHANGE =====

## 4.2	System Description

Based on the background and assumptions in clause 4.1, Figure 4.2-1 provides an example system for a messaging services and highlights scope of this specification, namely the definition of a multimedia messaging body part (MMBP) and the associated metadata.

Figure 4.2-1 Example system for Messaging multimedia message exchange

A Messaging Service Sender instructs a MMBP generator to generate an MMBP, for example using an API. This for example allows to define configurations on codecs, size, experiences or other attributes of the MMBP. The details of such an API are outside the scope of this specification. The sender adds the MMBP to a Container Message (either included as a body part or by reference), together with MMBP metadata parameters that provide information about the MMBP. Metadata includes, but is not limited to:

-	The media type of the MMBP, including subtypes and parameters for codecs, etc.

-	The size of the MMBP

-	Accessibility or language information about the MMBP

-	processing requirements of recommendations of the MMBP

The client of the messaging service receives the container message that includes the above information. The client communicates with a MMBP player its capabilities whether the MMBP can be played back, and if multiple alternatives are present, which of those are to be selected. Then the messaging service client instructs the MMBP player to playback the MMBP as part of the messaging service, based on the processing requirements and instructions. Playback may be combined with additional instructions for a player, including play, pause, seek, etc.

In TR 26.857 [35], Media Service Enablers are introduced. The basic concept of the Media Service Enabler is to support third-party delivery of media over the 5G System. The Application Provider is equipped with a set of 3GPP-specified functions, possibly both on UE and network side, in order to simplify operations. These functions are bundled as a Media Service Enabler (MSE) and offered to the Application Provider.

The MMBP Generator and MMBP Player are aligned with the Media Service Enabler concept. In order to support MSE concepts, the specification includes:

-	A call flow and high-level procedures in clause 4.6

-	Stage-2 APIs and parameter that can be assigned to player and generator in Annex D.

-	Several examples for content offerings in Annex B.

## =====  CHANGE =====

## 4.6	Typical Procedures for Messaging Services

Figure 4.6 provides a generic call flow based on the architecture shown in Figure 4.2-1 and the player model shown in Figure 4.3-1.

Figure 4.6-1 High-Level Call flow for messaging

The following high-level call flow is executed to address messaging:

1) 	A Message Service Sender instructs and MMBP generator to generate a message with the requested formats where the format is defined through a set of parameters handed over on an MMBP-GEN-API.

2) 	The MMBP generator creates an MMBP with relevant media data and adds relevant static and dynamic metadata.

3)	The generated message is provided to the Message Service Sender.

4) 	The generated file with associated relevant static and dynamic metadata is provided in a container message that is uploaded to a messaging server, together with a signalling in the media type and associated parameters. The media signalling may include that the file is suitable for different clients.

5)  A messaging service client is informed about the availability of a message in this format using the media type signalling together with appropriate sub-parameters.

6)	A messaging service client uses the MMBP-PLAY-API to query the MMBP player with it can process, decode and render the message.

7-10)	If the MMBP player indicates that it supports the capability, the messaging service client selects and downloads the message and hands it to the MMBP player for playback.

11)	The MMBP player processes, decodes and renders the message based on its decoding and rendering capabilities.

## =====  CHANGE =====

Annex B (informative):
Examples

# B.1	MMBP message with a 3D asset

In this example we show an excerpt of a message that contains a 3D asset. The root entry is a glTF 2.0 file. The message has two additional parts: a binary file that contains the geometry of the 3D asset and an image that provides the texture of the 3D asset.

MIME-Version: 1.0 Content-Type: multipart/related; boundary="===============" =============== Content-Type: model/gltf+json Content-Disposition: attachment; filename="sofa.gltf" { "asset": { "version": "2.0" }, "buffers": [ { "uri": "buffer.bin", "byteLength": 1024 } ], "images": [ { "uri": "texture.jpg" } ], "scenes": [ { "nodes": [0] } ], "nodes": [ { "mesh": 0 } ], "meshes": [ { "primitives": [ { "attributes": { "POSITION": 0 }, "indices": 1 } ] } ], "accessors": [ { "bufferView": 0, "componentType": 5126, "count": 24, "type": "VEC3" } ], "bufferViews": [ { "buffer": 0, "byteOffset": 0, "byteLength": 288 } ] } =============== Content-Type: application/octet-stream Content-Disposition: attachment; filename="buffer.bin" … binary data … =============== Content-Type: image/jpeg Content-Disposition: attachment; filename="texture.jpg" … binary data … ===============

# B.2	Single Media Type

## B.2.1 	Introduction

This clause provides examples for MMBP message with a single media type according to 26143_CONTAINER_RFC2046_SINGLE.

## B.2.2	Audio

The following example provides an MMBP that includes a single media in an 3gp file encoded with EVS.

MIME-Version: 1.0 Content-Type: audio/mp4, profiles="3gp9" codecs="sevs"; name="evs.3gp" Content-Transfer-Encoding: base64 Content-Disposition: attachment; filename="evs.3gp" AAAAHGZ0eXBNNA... (base64-encoded 3GP EVS audio-only data truncated)

## B.2.3	Images

The following example provides an MMBP that includes a single png image containing the 3GPP logo.

MIME-Version: 1.0 Content-Type: image/png; name="3gpp.png" Content-Transfer-Encoding: base64 Content-Disposition: attachment; filename="3gpp.png" url(data:image/png;base64,iVBORw0KGgoAAAANSUhEUgAAAQ8AAAC6CAMAAACHgTh+AAAAz1BMVEX///8AAADi4uL39/fw8PD6+vrsAACIiIi2trbq6upFRUX09PTMzMy7u7toaGiZmZknJyfY2NiqqqpLS0uRkZHS0tLs7OxQUFBZWVltbW3k5ORycnJ8fHwhISGsrKyjo6PFxcUZGRk3NzctLS07Ozt6enoRERFgYGAzMzNXV1ednZ0cHBxGRkaEhIT71NULCwvuLDP4sbT1mp34p6n1io3zdXnsABD6ysv5wcL2kpb83+HxY2fuIiv+7e70honxVlrvOD74u7ztER3wREnwWFylhzClAAAM9ElEQVR4nO2deX+bOBOALTAxNjY2tvF94PtIHKdNum2z7Xbb7ff/TK81EphDAuEYnPdXPf/EgEBoJEajmYEUChKJRCKRSCQSiUQikUgkEolEIpFIJBKJRCKRSCQSiYRJ0aq1h5vNbmOupnb51ndzU+zpcLe8Rz6OD/tB7rfRUSxLKeZebQijjdi0nNzuwZ4cKke33sVIV3OrOcKUIw3M3Mm+fqPabkVrNu3sa2ahbWLEcWKTrSIp6+aRU7PZybRmDoyuCaFnVrda68VVfMyuZi6rRHEg1M6maiu57lE2NfNxBMSRiUA0fRGpZrZYdnu91rxx3vWgXb/qOHZC8kDOlavttNfBCubmtKp4c0rZrnfpgaVx5apjKYfavTbNCkse91etdXAIXt1sKtFhMKgf8xdILXhjddxDqtOPCiSk2ZTafrirVCpds12zUtbZCaqNns5rsFqCAq0L2nUpdXajyxG1Xzmfo9XC8tpMxPuwM/KfOavHGqMWaBLzgoZdyMR/c35lHhTUCdcKsYfRwYN7uSZmUCr+k7q1pOIqWAPTC1p2GUX/7QWe4XCzq7DX6SIe47aQ4bY8jyohAxTKK6kbdim+AVINHCiGmou7svrAlQawElgAuhVuBNVOEWvVZfqGXYq3eKmGDkTkoTLUbJhh4hjR2LXxAQspvyemoOxgsEcmvFBLbTHLDY2SVutkrk1hieNumOU56ZZtKzr968FmHhNWfT7qUc2qFsuDgXJiMOhUoVBX/PYsXH7/lgZeAXUm3P4IDa/zDas6KfV7D9ux7yj5I/7AFPDkv83Zbg+TvOqNo4KV5Wi3XceUWQwFp2gyVNNafVfFWMY0RIRNIaKAWCQbIBiwWeoZNzmO6uyN4kBYqY6Ti5300lRgkOBh1s+81TzosuEtgPYTHGON5DGCJZujCRKk1khsQhILuFBgDbTomqvSiZXZioybxK7HZ1x3hS2MxVzup4TMHWQZuys1nbDZGjZlkpb061vJwxZ0D1HGy15/tOr3WsvFzLd7SC4GQYwZsx5wja3Pa6FtrEBAn1biSmSCWkszyY77uuVrRVGxbL0+MrtbhKhbHFYCR6a5ShYJquX5hTZxNwbz7ep6DRXCPqTRG6bDdeJ03ChBE0oyYwYGHDotSoqu7o6zP3u0cE4YilVbsSfH5py5O3nVhiEWP3vJC0vDLf6lUIXFX9IrMde5PtUtbwzM9oU9a39PMEDlxLQT1iSIOEBIsHTLvQ6snPJyGvIit6iia4UOY/9ayKTE2FCeY2fDqDiQ32QgTTiXIS7enCJTPHGsoOsYDo+uePB9ECcPkEFD9W3cs1dsJAKQkzVW5YwNoiyt6JFSiourcAbHKWjAUs+hW+CtZWpMjWj5FKvht8CZUqhzwowcSKfk4+RB3EKe5xzbPawlvTFP3Q1vgDEAKAcl5AoHeI84h9ieJVcv+recSKEB6bBdunovZsCVx6nrlMiyLq0JMIuTB4mUNt2tEWLEWWi0rJubL4g712LCzpzUg3YeKw9orGeFw1AN2ngGDXls8ksV4j8wUUIeTyUO0jBYnzjcykHennmChReYypvHC7vhLdhCThsYLYGJVovNY6HWE+hjvrlSD7QWb/kChPqMXOk+p5nFRd0Lum2CBlG8OKgD5BA9zw+JeLlbjncaDu3TTMdjNAySPQNLb/eTFnPBp4Vr1VLmUAoUcpNVJQEMcYdulKlwVGvv+gHW8cHuTNHKzdg+D5qZCeJAPSgFUouZlMAa9CYVvNFeVbwcWLN24xBDwZhyR0kvULDGK+ZCtAJ4OeL84qC8GutoVGLedrJrZhpqHG0StDJH7EJniOEGyxJu6pmm30dPXM+7I92+YS5uhCarfaHFONsr4oMmR+CfB049ltk4y2O9aIHSmN4k4zSeAcNICw36JHHQTBKwbvgZk5pRLFLpVzUymHKeXMVQo90fcupYCRGaBikGU8YwvjJi/5wKQWpJrrmEwhjh9i2iD7Q9irHk6JgAd1JCcIWq5g6MpRvFWBIJ5TqwE9o0Z8hQiABdBxe55/ogT2cdXE88XXNzQg3lLSFUjslCfb/g84mNJBSCsambxvDjCE2oXBuTnYC+oIaUivu+xzvX5ZyTliJHJmdC9rjDK8eWhxc6wvGtRF/O2Wf5bodHITR/cBMj2Rl2sIRTavtSQ0QeBTcTK9eFfTpCrx44vHLspDKIz9hUVrNSQre7udB5OQUvYBJqIHcNwsxHdUNHxT2dkhcHvp3V8YZifjm3aYk4krlzRPQVFhRYsZz1UG/CCjg6vgH2bh8XhhOREzxlJyH6no9gfKe/rxZVMvdoRseph5IrbmOcFhU7/g1X1mKeYygx3a8NXwHiwF9MTG9NtF72TNPcBZcERIPkmIPsUetBl963Spw5Q2Ov5dkPNzNlOSA7cG2czHC1rB942SWmTeap/DPELL93Y8vybE45GaNz5vWYsgvoTpLMQDc0ZRJ50Za8OkM0lnPNtgoQfhKWIbVghV7j8sNapmreU7A0z7lPgSKH6K6C4uiTeqnUnuqOlzkBkk00Za9LNP6IHO+gYZdig1MsP5enLfF87P4OThNk5k7WlLG5ABnBMp5gJTqw65tZrDCiDfVfEDSGa6oG20REJpDbAwMp3xwxZiu3rQrTiMA0lMDTbob83Z42hS3q6QopGlW030nqTa5TbuIICDEKe4aOjv9y3quqTf9meNKEnSJuQLBTc01ST5ejv4QJOWRh9M6ztKc8qK9ZI8MsbNkkxHDPkJF0lYYKwvScc2i4qR5hE6NStwadjqKfPUGuiMA2jygZ2CuUcgYl8/yAgTYTlcba99IBw1yd+Y2UswT0ORpGQmowwQjl0cCzmWsWsuAbcONJirMCq3RGgBEeODG1AMmduX4EJe5DMC6biPKzY0on9ieoBbGvVqjYPZDju9iFZIE8TFm5tgPuOw8VGBHPn/mRZ1Czgo2Eu8v3G04O79s0uHF1nktG4xjyfSqHr0+fPjy+vj4+Pn55/voaPBcnxQi6viC3MqNvr/Aw2K/fo00zNgubmUd0dru//n1H+Pbxe+hMbFeIakk8Ax7zznJQwk7g424qoMWas9Bpw4Cp8f3n138+/gwLo0BiW8Ir+XsUm0CTEUV92KrMt9t5pTucOsJ+y5rvWwaLkuhXp/AqcpFcjICFNxYtfGW09AOz7JT6u26v37a9c7XnL/Hn4IXJWHhdgrXvuwzyi/Py9NGnQl8/fwgGv7XT5LQW/oQZtv7eb5hOjM9/3X37/fL88eXTj6e7u6/hYYcnGPFYAp7a32+cTpDnf8ns8uvHh+hD2E7VwpM1PCOLGPU9ZU2l5fHDy8vzZ8b0Qh6BFFZ4m6oPta6bhXebAMFGSCUbVcu6xM9jG6X9/5k8Cs8/s7u2VazVnewunw1fn6IS+f54gxt5L3y5+/Xy6OoN7fvrh5f/PjHVyJ/C99PS5dvvT/+8/PPp7/9+3d39+JNHB/D5x93dX5jTjPvpj5cG5vXj76dvTz9evtw6A18ikUgkEolEIpFIJBKJRNw/wSl5XQfHzd0lzWh2Rfkwp4H6pRcoLA5nxzlNbbPnlc05y63rRdpWi2DQrTimwQH4fJ2zdmBjOobQy77S3Wy6S8ir08deep37L6Im5DBUo5Pvw6olnJg2qEA0ptjYrrMIcxtrFI6k1hFa0JQc702NIkLt5pqmktqoVPeS5mrnVI5eKOhWdFPc1zj6VKNR6Tr52zRHaDYyHfjtpRtW3avp5hCh0Qb2T0mOYQdSN0fkO49lVNGzyDwsRVJ12qjlNctLSzDxLQ3oW9k2qiluQpzWGHsZkZtQUNaTB8L5lTV6dO9F69ED/XGWR6Phpe9qjTX9NaE17FH7JAeSOWJk8/VkBU2bwVxQhWaAkTt25bFGWBI0lctGS/d7dad7HDy4OdUmVx7mHDUdmgc2PcvDHVneLUyQtXHzd43GseDupeHbMSoc6NkGavQyeFvo1O8175uB9HZPPb/akSa68lCPRB5r+PiEjeoT+oqggSrVg5ssaIYy387yaBtL9JAsD/W4qLbdVLuoPE4/Su6wMNCyLfwZSWEgb3IcSCF2cEt3tJO8r3xUcAJtk96Mfbq/Bsmjxm8f37vB6U1IFfnkUdC67v+LiZEHjvvPju4IiMoDv9lPq8jmeWnA5e8D47yPUO/oyuMkLFAmyknNtNCcjJoq2oyRQ3Z3YZs80zs0C+TZFuk9dyBDcEW/i1snpxZ80qYKcwDq26LpUSdN7x0+p4q71++gY8v/Wt51cGBk6NPArO+semaJPEKl0dAko7zc7pkTWkwZlmpEMioJLx/I+JiOVn2/zjdGJBGuOITubxIZOyNX+iO3cdURXEDbQ7kRGR9quxQ8jPHSVo3VaJj0XQSJRCKRSCQSiUQikUgkEolEIpFIJBKJRCKRSCQSiUQikUgkEolEIrky/wPjxbQKm0Pj6gAAAABJRU5ErkJggg==)

# B.3	3GP File with multiple media types

## B.3.1 	Introduction

This clause provides examples for MMBP message with conforming to 26143_CONTAINER_MP4_3GP9.

## B.3.2	Audio and Video

The following example provides an MMBP that includes an EVS audio track and an HEVC video track in a single media in an 3gp file.

MIME-Version: 1.0 Content-Type: video/mp4, profiles="3gp9" codecs=" hvc1.1.2.L93.B0,sevs"; name="av.3gp" Content-Transfer-Encoding: base64 Content-Disposition: attachment; filename="av.3gp" AAAAHGZ0eXBNNA... (base64-encoded 3GP AV file data truncated)

## =====  CHANGE =====

Annex D (normative):
API Definitions

# D.1	Introduction

This Annex collects the stage-2 API definition supported with Interface Definition Language (IDL) description MMBP-GEN-API and MMBP-PLAY-API. Note that this does not form a full implementation but can be used as a reference for implementations in different environments.

# D.2	MMBP-GEN-API

### D.2.1	Introduction

### D.2.2	Methods

#### D.2.2.1	Overview

Editor’s Note: For MMBP generator, we need

- The source assets (audio, video, image, etc.)

- The generation functionality on the highest level

  - SINGLE

  - MIXED

  - ALTERNATIVE

  - PARALLEL

  - 3GP9

- For each track, how the generation is done:

  - Picking the encoding options available for generator

  - Possibly for each codec, add some configuration parameters

### D.2.3	IDL Definition