Agenda item: 	10.7

Source: 	Qualcomm Inc.

Title: 	[AvCall-MED] 2D Avatar Profile

Document for	Discussion and Agreement

# Introduction

In this contribution, we propose a definition for the 2D avatar profile of MPEG ARF, which is defined for usage with 2D avatars.

# Proposed changes

### 5.6.3	2D Avatar Format

An AR-MTSI client that supports 2D avatars shall support the following data formats and requirements for the avatar assets:

- 2D mesh representations that consist of a single planar mesh or quad suitable for texture mapping, conforming to the binary glTF (GLB) format,

- Static image assets for the base avatar representation that conform to still image formats as defined in section 5.5, with support for at least PNG and JPEG formats,

- Landmark sets for facial animation, where landmarks shall be defined as 2D coordinates (x, y) on the avatar image plane,

- Texture data components that contain the avatar appearance, which shall conform to still image formats as defined in section 5.5,

- Animation data based on landmark positions, where the landmark animation samples shall use the 2D coordinate format (ala_is_3d_flag set to false) as specified in clause 8.4 of ISO/IEC 23090-39 [11], and

- Sparse tensor data formats for landmark vertex indices as described in the ARF specification [11].

For 2D avatars, the ARF document shall include:

- At least one landmark set that defines facial feature points for animation,

- Support for the landmark animation type in the supportedAnimations list, identified by an appropriate URN,

- A simplified asset structure with a single level of detail containing the 2D mesh and associated texture, and

- LandmarkSet components that reference vertices on the 2D mesh corresponding to key facial features (e.g., eyes, mouth, eyebrows).

The 2D avatar animation shall be achieved through landmark-based deformation, where:

- Landmark positions from the animation stream drive the deformation of the 2D mesh,

- The mesh deformation should preserve the natural appearance of the facial features, and

- The number of landmarks should be sufficient to capture essential facial expressions while maintaining computational efficiency.

- Data items of the 2D avatar should signal no compression or protection schemes to ensure broad compatibility.

NOTE: The specific landmark configuration and animation URNs for 2D avatars may be defined in operator-specific profiles or through industry fora.

# Proposal

We propose to agree the content of section 2 to the base CR.