3GPP TSG-S4 Meeting#133-e	S4-251337

# Online, 18th–25th July 2025

Source:	Apple Inc.

Title:	[VOPS] On Conformance bitstreams

Agenda item:	9.5

Document for:	Agreement

# Introduction

This contribution provides a compressed multiview bitstream that conforms to the 3GPP MV-HEVC Stereo Operation Point. The sequence was generated by rendering a 3D scene from two virtual cameras in a 3D CAD software environment. The rendered views were encoded using the Main (Layer 0) and Multiview Main (Layer 1) HEVC profiles, which are subsets of the Main 10 and Multiview Main 10 HEVC profiles. The resulting bitstream is thus compliant with the 3GPP MV-HEVC Stereo Operation Point. The bitstream is intended for use in conformance testing and evaluation of stereo coding tools in 3GPP.

All animations and 3D assets were created in-house and are available under a CC0 license.

# File Description

The provided bitstream file, hummingbird_spatial.265, is a stereo MV-HEVC encoded bitstream structured with two layers:

- Layer 0: encoded using the HEVC Main profile, carries the left-eye view.

- Layer 1: encoded using the HEVC Multiview Main profile, carries the right-eye view.

The encoding includes the presence of the three_dimensional_reference_displays_info SEI message.

Figure 1 shows the first frame of the captured sequence hummingbird_spatial.265. The left view (coded in layer 0) is shown alongside the right view (coded in layer 1). No frame packing is used; the two views are displayed separately for illustrative purposes.

Figure 1: First frame (left and right) of the sequence hummingbird_spatial.265

Figure 2 provides a view into the bitstream structure as displayed in VQ Analyzer. The screenshot highlights the NAL unit breakdown and coding structure across the two layers.

Figure 2: NAL unit and coding structure of hummingbird_spatial.265 visualized in VQ Analyzer

Two auxiliary files are provided alongside the bitstream:

- hummingbird_spatial_syntax_stats.csv: Contains parsed syntax elements per picture, including slice headers and other relevant bitstream statistics.

- hummingbird_spatial_dump_headers.txt: Lists high-level syntax elements per NAL unit for inspection and analysis.

These files are intended to assist in the detailed examination of the bitstream structure and to support integration into test workflows. It should be highlighted that the encoded files contain multiple CRA NAL units about every coded 28 frames. A single IDR NAL unit is included at the start of the coded sequence.

# Decoding using the MV-HEVC reference software

The bitstream can be decoded, after certain updates are performed as discussed in the subsequent section, using the MV-HEVC reference software (HTM) available at:

https://vcgit.hhi.fraunhofer.de/jvet/HTM

## Build Instructions

macOS (Apple Silicon) Build Fix

We encountered build issues on macOS with Apple Silicon. A patch file (macos_build_fixes.patch) is included with this contribution to resolve these issues. The patch also contains some bug fixes based on issues we discovered during testing these files.

The patch can be applied by running:

git apply macos_build_fixes.patch

Compilation

After applying the patch, one can compile the project as follows:

mkdir mybuild && cd mybuild

cmake .. -DCMAKE_BUILD_TYPE=Release -DHEVC_EXT=1

make -j

## Decoding Process

The decoder can be run with the following command:

TAppDecoder -b hummingbird_spatial.265 -o bird_1920x1080_420p_8bit.yuv

This will generate the following decoded output files:

bird_1920x1080_420p_8bit_0.yuv

bird_1920x1080_420p_8bit_1.yuv

# Proposal

It is proposed to add the provided conformance material for VOPS conformance work.