Source:	Nokia

Title:	Towards finalizing changes on ambience transmission assessment

Agenda item:	7.7 – ATIAS_ph2

Document for:	DISCUSSION

# 1. Introduction

In the [1], thorough review on the status of the ATIAS_ph2 was presented. In this document the source presents further clarifications and proposals on how to progress the work, once the dependencies on other standardization activities are solved.

# 2. Discussion

At least following aspects towards the finalization of the ambience transmission testing specification for IVAS-based UEs are identified and further discussed.

# 2.1 Dependency on the IVAS_ph2

Editorial harmonization

When considering envisioned performance of IVAS-based UEs in ambient sound field, the performance is currently divided into three main ambience transmission types for the discussion, namely “Transparent”, “Nominal”, and “Suppressed”. Relating ns-mode SDP parameter to negotiate the applied transmission modes within an IVAS session has been discussed in IVAS_ph2 work item, although with a slightly different terminology. In the latest proposal [2], the potential names for the parameters are “min”, “def”, and “max”. Currently envisioned mapping of the parameters and names is as follows:

- “min” = “Transparent”

- “def” = “Nominal”

- “max” = “Suppressed”

Harmonization of these terms is expected, once the concept/implementation is found agreeable within the IVAS_ph2. To avoid parallel work, the harmonization in ATIAS_ph2 is proposed to be done based on the outcome of the IVAS_ph2. In addition, further definition of the terms is expected, to verify that the applied terms are representative

Definition of the different noise suppression modes

Initial considerations on the capture type definitions for the different ambience transmission modes were presented in the [3]. For clarity, corresponding SDP parameters are displayed in the parenthesis:

- Transparent capture type (ns-mode: ‘min’): Sound field is intended to be captured and transmitted “as is”. The receiver expects accurate representation of the captured sound field in terms of loudness, frequency response, temporal characteristics, and spatial characteristics. Minor attenuation and/or modification of the captured ambience is allowed, to facilitate robust speech transmission. If speech is present, target speech-to-ambience ratio is typically improved [0-6] dB. - Nominal capture type (ns-mode: ‘def’): All speech is intended to be captured and transmitted “as is”. Moderate attenuation and/or modifications of the captured ambience is expected in order to enhance the speech transmission. Improvement to speech-to-ambience ratio is typically at least [6-12] dB. - Suppressed capture type (ns-mode: ‘max’): All background is expected to be attenuated, while foreground speech (the main talker) is transmitted. Spatial characteristics of the transmitted speech are expected to be maintained. Target improvement on speech-to-ambience ratio is over [12] dB.

While the definitions above are not formally agreed, the current proposals in both ATIAS_ph2 and IVAS_ph2 do not contradict with the previously presented definitions above. It is proposed that the definitions above are used as the starting point for finalizing the definitions of corresponding noise suppression modes.

Configuration of the assessed noise suppression mode

The latest proposal on configuring noise suppression mode for the IVAS session comprises both ns-mode SDP parameter and Dynamic Audio Suppression (DAS) PI data requests. The ns-mode SDP parameter can be used to negotiate applied noise suppression mode for the session, as well as negotiate the supported noise suppression range which can be further changed via DAS requests.

It is recognized that the proposed ns-mode SDP parameter for configuring the applied ambience transmission mode between UE and reference client has evolved from the initial considerations presented in ATIAS_ph2. In addition, potential applicability of DAS PI data in both forward and backward directions is not currently recognized in the ATIAS-2 permanent document.

Furthermore, at least following potential configuration scenarios may be obtained for the UE assessment:

- ns-mode SDP parameter is supported by the UE and reference client. Assessed ambience transmission mode can be configured and determined via the ns-mode parameter.

- ns-mode SDP parameter is not supported, but DAS requests towards UE direction and forward indications towards reference client direction are supported. Assessed ambience transmission mode can be requested via DAS request and determined with DAS forward indication.

NOTE: while the UE may support higher granularity in terms of applied noise suppressions via DAS requests, it is considered that all noise suppression levels should not be tested, i.e., assessing only “min”, “def” and “max” noise suppression levels is appropriate

- ns-mode SDP parameter is not supported, but DAS requests towards UE direction are supported. DAS forward indication towards reference client is not supported. Assessed ambience transmission mode can be requested via DAS request and determined with test method defined in Annex TBD or loudness test (potential fallback solutions)

- ns-mode SDP parameter or DAS requests/indications are not supported by the UE.

  - Applied ambience transmission mode by UE can be defined and determined by manufacturer.

  - If no specification is provided by the manufacturer, the applied ambience transmission mode by UE can be determined with test method defined in Annex TBD or loudness test.

It should be noted that the above list may not be exhaustive, as the final implementation of the ns-mode parameter may still change. However, source would like to highlight, that the current text in ATIAS-2 permanent document need to be modified based on the IVAS_ph2 outcome in terms of applied configuration between UE and reference client. In addition, UE configurations for the tests in receiving and in both send and receiving direction (e.g., AEC tests) has been currently out of the scope. These aspects should be considered in the potential follow-up work.

# 2.2 Clarification on scope and dependency on the ETSI TS 103 224

# Scope of the ambience transmission testing

# While assessing spatial characteristics of transmitted ambient sound field is considered important aspect in order to provide sufficient quality for immersive communication, source’s current view is that efforts should be targeted towards finalizing aspects on assessing the noise suppression performance, i.e., noise reduction vs transparent transmission. Thus, it is proposed that remaining time is used to finalize already proposed, existing test methods in ATIAS-2, which are targeted on assessing the level of applied noise/ambience suppression by the UE. Additional test methods for assessing the spatial characteristics of transmitted ambient sound field could be considered in the potential follow-up work.

Dependency on the ETSI TS 103 224

ETSI STQ indicated that they have started work towards the extension of ETSI TS 103 224 for reproduction of complex sound fields. While the extended sound field simulation system may be desired for assessing IVAS-based UEs, the expected timeline regarding the extension is unclear.

As the extension may not be available within Release-19 timeline, it is proposed that the current background sound field simulation system defined in ETSI TS 103 224 (v1.6.1) [4] will be used as the fallback solution, in case the extension is not available before the finalization of ATIAS_ph2. Furthermore, as the scope of the ambience transmission testing is considered to cover performance assessment of the noise suppression, the existing sound field simulation system may be sufficient in any case.

# 3. Conclusion

Several aspects on dependencies with other standardization activities are highlighted. Potential approaches to progress the work are discussed and proposed.

# 4. References

[1]	S4aA250071: “Status and possible ways for completion of ATIAS_Ph2”, HEAD acoustics GmbH

[2]	S4-251372: “Noise suppression mode SDP parameter for IVAS”, Nokia

[3]	S4-250205: “On testing UEs in ambience”, Nokia

[4]	ETSI TS 103 224, “Speech and multimedia Transmission Quality (STQ); A sound field reproduction method for terminal testing including a background noise database”, v1.6.1, March 2022.