3GPP TSG-S4 Meeting  #134	S4-251922

# Dallas, USA, 17th November – 21st November 2025

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.941 | CR | 0003 | rev |  | Current version: | 19.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_AMD_Ph2] AF requested modification of set of Network Slice(s) for a UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung Electronics Co. Ltd., |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_AMD_Ph2 |  |  |  |  |  | Date: |  |  | 2025-11-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | TS 23501 describes a new procedure on AF influenced slice change for a UE. A document S4aI250082 was looked at on this topic in Rel-19. SA4 MBS group decided to pursue this study topic in Rel-20. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Proposes a brief description of the new procedure on AF request modification of set of Network Slice(s) for a UE specified in TS 23.501. Proposes a key issue description related to media delivery on this topic for study in Rel-20. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Network slicing features for 5G Media Streaming will be incomplete |  |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2, 6.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | .. |  |  |  |  |  |  |  |  |

CHANGE

### 4.2.2	Network slicing for specific applications

Before application services are allowed to access specific network slices, a third-party Application Service Provider can negotiate with the MNO and the MNO may create or allocate the network slices based on the service requirements. For example, a cloud gaming service provider may interact with the MNO to reserve specific network slices supporting low latency, and high computing resources.

Afterwards, the Application Function, on behalf of the Application Service Provider, informs the 5GC that the target application service can use the specific network slices, i.e., by providing application guidance for UE Route Selection Policy (URSP) determination as defined in clause 4.15.6.10 of TS 23.502 [15]. Depending on the nature of the application guidance, the operator may update the Network Slice Selection policies in the URSP accordingly. As a consequence, the application service may be migrated to the new network slice/DNN duple based on the updated URSP rule.

The URSP rules in the UE, which are used to associate applications with usage of particular network slices, may be pre-configured or provided by the PCF as defined in TS 23.503 [16]. Each URSP rule is expressed as a traffic descriptor for application detection, e.g. IP descriptors, application descriptors, domain descriptors.

NOTE:	There is no restriction on which part of UE should (re-)evaluate the URSP rules. This may be done by either the Operating System or the modem layer.

Once an application is started or detected on the UE, the following procedure is followed:

1.	The UE evaluates its URSP rules in the order of Rule Precedence and determines whether the application matches the Traffic descriptor of any URSP rule.

a.	When a URSP rule is determined to be applicable for a given application, the UE derives the suitable network slices based on the applicable URSP rule.

b.	If the UE determines that there is more than one existing PDU Session which matches a given URSP rule, it is up to UE implementation (Operating System or modem layer) to select one of them to use. Otherwise, the UE tries to establish a new PDU Session using the derived network slices.

2.	If there is no matching URSP rule (except the “match all” rule), the UE uses its own local configuration (if any) to determine which PDU Session to use.

NOTE:	The UE local configuration in this context is information about the associated application, such as application-specific parameters to set up a PDU Session or end user configuration for specific applications. This can be provisioned in the UE via the application layer, e.g. following interaction between the Edge Enabler Client (EEC) and the Edge Configuration Server (ECS), as defined in TS 23.558 [24].

3.	When URSP rules are updated, or when a particular URSP rule’s validity changes, the association of existing applications to PDU Sessions may need to be re-evaluated.

4.	Depending on UE implementation, the associations between applications and PDU Sessions may also be re-evaluated periodically, independent of any changes to URSP rules.

#### 4.2.2.1		Network slice replacement without Application influence

When a network slice becomes unavailable (e.g. due to overload), the AMF is triggered, either by local configuration (e.g. trigger from OAM) or by a notification from the Access and Mobility Management PCF (AM PCF) or by the NSSF [26], to replace the current S-NSSAI with a previously chosen Alternative S-NSSAI. Using a suitable NAS procedure (e.g. UE Configuration Update) the AMF informs the UE about the Alternative S-NSSAI as well as providing the mapping between S-NSSAI(s) and Alternative S-NSSAI(s) in the Allowed NSSAI and/or in the Configured NSSAI.

1.	In the case where there is no existing PDU Session in the unavailable slice and the UE is trying to establish a new one to support a 5G Media Streaming session, the UE may provide both the Alternative S-NSSAI and the current S-NSSAI in the PDU Session Establishment message, in which case the AMF provides both S-NSSAI values to the SMF for the PDU Session establishment. The SMF proceeds with the PDU Session Establishment using the Alternative S-NSSAI. As a result, the new PDU Session is established over the Alternative S-NSSAI with a new IP address.

2.	In the case where an ongoing 5G Media Streaming session is already being carried over the PDU Session associated with the unavailable slice, the AMF informs the SMF responsible for the PDU Session that it is to be transferred to the Alternative S-NSSAI. Then, depending on the Session and Service Continuity (SSC) mode of the existing PDU Session, either:

-	SSC mode 1: The SMF further updates the network slices in the UE/RAN/UPF via the PDU Session Modification procedure. In this case, the IP address of the PDU Session remains the same.

-	SSC mode 2 or 3: The SMF triggers the modification/release of the PDU Session and re-establishment of the PDU Session in the Alternative S-NSSAI. In this case, a new IP address is allocated during the PDU Session re-establishment procedure and the ongoing 5G Media Streaming session at reference point M4 and M5 needs to be migrated to the new PDU Session.

When the AMF is notified that the replaced network slice has become available again (e.g., congestion has been mitigated), the AMF reconfigures the UE (e.g., by using the UE Configuration Update procedure or else as a matter of course when the UE next registers with the network) to use the replaced S-NSSAI if it has already configured the UE to use the Alternative S-NSSAI when the S-NSSAI became unavailable. Furthermore, if a PDU Session was established in the Alternative S-NSSAI when the replaced S-NSSAI became unavailable, the AMF triggers transfer of that PDU Session to the replaced S-NSSAI when the S-NSSAI becomes available again by updating the SMF(s) of the PDU Session using the Nsmf_PDUSession_UpdateSMContext service operation described in clause 5.2.8.2.6 of TS 26.502 [15].

#### 4.2.2.2	AF requested modification of set of Network Slice(s) for a UE

Clause 5.15.5.2.2a of TS 23.501[7] specifies procedure for AF requested modification of the Set of Network Slice(s) for a UE. As specified in this procedure

-	An authorized AF, or an AF subject to authorization via the NEF, may request the PCF for the UE to replace a certain S-NSSAI (Replaced S-NSSAI) with another S-NSSAI (Alternative S-NSSAI) which is part of the UE subscription

-	The PCF notifies the AF may subscribe to notification on the outcome of the AF requested network slice replacement

-	The PCF sends slice replacement management in the access and mobility management policies to the AMF per implicitly subscription, as described in clauses 6.1.2.1 and 6.1.2.6 of TS 23.503[16]. The slice replacement management policy includes the Replaced S-NSSAI and the corresponding Alternative S-NSSAI as provided in the AF request, and a Network Slice Replacement Type. The Network Slice Replacement Type indicates that the network slice replacement is requested by an AF.

-	The AMF, upon receiving the above slice replacement management policy, performs necessary slice management operations (e.g., remove Replaced S-NSSAI from Allowed S-NSSAI, and add the Replaced S-NSSAI into the Rejected S-NSSAI by UE Configuration Update procedure). The UE and AMF release PDU sessions that were using the Replaced S-NSSAI. After updating necessary UE context information locally, the AMF reports to the PCF the outcome of network slice replacement as specified in clause 6.1.2.5 of TS 23.503[16]. The PCF further notifies the AF of this outcome

-	Operator policies in the PCF ensure that UE is configured with a URSP rule that contains the Alternative S-NSSAI to establish a new PDU Session.

The AF, or a NEF, may then later on, request the PCF for the UE to terminate the slice replacement, and switch back to the original network slice. The PCF for the UE sends updated access and mobility management policies to the AMF as described in TS 23.503[16] to indicate that the original network slice is available again. The AMF updates the Allowed NSSAI accordingly, and removes Replaced S-NSSAI from Rejected S-NSSAI by a UE configuration update procedure.

In both the cases, when switching from original network slice to Alternate network slice, or vice versa, any PDU Sessions carrying 5G Media Streaming session traffic are transferred as described in clause 4.2.2.1 of the present document.

change-2

## 6.X	Key Issue #X: AF requested slice change for a UE

### 6.X.1	Description

#### 6.X.1.1	Identification and usage of Alternate S-NSSAI during AF requested modification of Network Slice(s) for a UE procedure

Clause 4.2.2.2 of the present document describes the procedure for AF requested modification of set of Network Slice(s) for a UE. As part of this procedure, AF interacts with PCF to modify a current network slice (Replaced S-NSSAI) of a UE with a network slice associated with Alternate S-NSSAI. When the PCF for the UE receives a request for modification of network slice for a UE, it interacts with the AMF to facilitate such modification of the network slice, and migration of any PDU sessions carrying M4 application flows to the Alternate S-NSSAI.

In current 5G Media Streaming specification in TS 26.501[26501] and TS 26.510[26510], the 5GMS Application Provider uses Provisioning API to configure the network slice information at the 5GMS AF at reference point M1. TS 23.501 specifies that how the AF obtains information about Alternate S-NSSAI is out of its scope.

Open issues:

-	How the AF obtains the information about S-NSSAI to be replaced, and the Alternate S-NSSAI.

-	When and how a decision is made by the AF to trigger the procedure for AF requested modification of set of Network Slice(s) for a UE

### 6.X.2	Candidate solutions

### 6.X.3	Conclusions

end of CHANGEs