TSG SA4#94 meeting	Tdoc S4-170534

26-30 June 2017, Sophia-Antipolis, France	Agenda item: 5.3

3GPP TSG-SA WG3 Meeting #87			Tdoc S3-171450

Ljubljana, Slovenia, 15-19 May 2017.

Title:	Reply LS on external interface for TV services

Response to:	LS (S4h160679 and C3A170068) on external interface for TV services from SA4 and CT3

Release:	Release 14

Work Item:	AE_enTV-MI_MTV

Source:	SA3

To:	SA4, CT3

Cc:	SA2, SA, CT

Contact Person:

#### Name:	Noamen Ben Henda

E-mail Address:	noamen.ben.henda at ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	S3-171134

1. Overall Description:

SA3 thanks SA4 and CT3 on their LS on security aspects of external interface (xMB) for TV services. In reply LS to the CT3 and SA4 LS (S3-170356) SA3 promised to come back with further analysis of the security aspects of xMB.

SA3 discussed the security for the xMB interface and agreed on a CR to TS 33.246 (S3-171134) to specify the use of TLS and DTLS over xMB interface, which refers to the 3GPP TLS profiles in TS 33.310. SA3 kindly asks CT3 and SA4 to refer to TS 33.246 accordingly in their specifications on xMB.

Use of TLS or DTLS ((D)TLS in short) according to CR S3-171134 provides a mutually authenticated, confidentiality and integrity protected xMB interface between the BM-SC and a content provider. Separate (D)TLS connection is required between the BM-SC and each content provider. Authorization of the content provider towards the BM-SC could be based on the (D)TLS connection, i.e. the BM-SC checks if the content provider who sent a request over an authenticated (D)TLS connection is authorized to send that specific request. This would allow the authorization to happen on (D)TLS connection level, i.e. between the BM-SC and the (D)TLS peer, like an application server in the content provider domain.

SA4 and CT3 suggest in their LSs and xMB related TSs 33.246 and 29.611 the use of OAuth to authorize the content provider to the BM-SC. However, it is not clear if the use of OAuth is needed in case there is already a mutually authenticated (D)TLS connection. A more granular authorization might be needed for example for authorizing separate users within a content provider domain to perform different actions over xMB. SA3 assumes that authorization on (D)TLS connection level would be sufficient for SA4 and CT3 purposes.

SA3 would also like to note that the use of the above authorization mechanism cannot be used for identifying and removing any individual content streams that are submitted by the content provider that may be reported as being in violation of the rights of the copyright owners. Any such violations should be dealt with separately (e.g.,  by removing the offending content from the BM-SC).

2. Actions:

To SA4 and CT3 groups.

ACTION: 	SA4 and CT3 are kindly requested to take the above into account.

3. Date of Next TSG-SA WG3 Meetings:

SA3#88	7-11 August 2017	Dali City, China

SA3#88Bis (Adhoc on 5G)	9-13 October 2017	Singapore