Source:	HEAD acoustics GmbH

Title:	Proposal on new work item regarding evaluation of SWB/FB speech transmission in noisy scenarios

## Document for:	Discussion

## Agenda Item:	9.7

# Introduction

The DESUDAPS work item was successfully finalized at 3GPP SA4 #93, a summary is provided in [1]. A framework for collecting listening test data as well as test methods and performance objectives for devices regarding super-wideband and fullband speech quality in background noise were defined here. In conjunction with the development process in ETSI STQ [2], new prediction models for SWB/FB were released as ETSI TS 103 281 [3], utilizing the databases collected in 3GPP in the DESUDAPS work item. Finally, the test specification was updated in Release 14 for the usage in SWB and FB scenarios [4].

# Discussion

The DESUDAPS work item itself is complete. However, several topics regarding the usage of the new prediction models may require additional work in order to clarify possible ambiguities and remove/finalize provisional values. In addition, 3GPP should consider the dynamic behavior of devices including time-variant setups reflecting the typical use conditions of mobile devices. Thus, a subsequent work item seems eligible to address further work on this topic.

The main goal would be to define performance requirements for handset/hands-free/headset mode for SWB and – if applicable - FB terminals. Clauses 7.10.2, 7.10.3, 8.10.2, 8.10.3 of TS 26.131 [5] should be updated in order to remove all values in brackets and/or are marked as TBD.

Some items for further consideration:

- Collect measurement results and experiences with new prediction models and the combination of both in order to define adequate requirements

- Improve the test procedure described in TS 26.132 [6] in general, e.g., with regard to different handset positions to stress noise reduction capabilities.

- Analyze possible usage of TS 103 224 8-speaker setup [7] for handset/hands-free/headset, since the new predictors were mainly trained with this playback system.

- Invite members to present new/additional listening test results for further validation/evaluation (e.g. additional languages, use cases, background noises, etc.).

- Evaluate the usage of the new model(s) for NB/WB case; the focus of the new work would be mainly on new SWB/FB terminals, but is not restricted. NB and WB may be included as well.

- Investigate time-variant/dynamic scenarios reflecting typical use cases

# Conclusion

A new WID should be consented at next meeting 3GPP SA4 #95.

References

| [1] | 3GPP SP-170346, „DESUDAPS: Summary Description of the Work Item "Development of Super-wideband and fullband P.835 test framework, Databases, and Performance Specifications",“ Knowles Inc., Qualcomm Inc., West Palm Beach, Florida, USA, 5-9 June 2017. |
| --- | --- |
| [2] | ETSI Work Item DTS/STQ-232, „Speech quality performance in the presence of background noise: Background noise transmission for mobile terminals-objective test methods for superwideband and fullband“. |
| [3] | ETSI TS 103 281 v1.1.1, „Speech quality in the presence of background noise: Objective test methods for super-wideband and fullband terminals,“ 04/2017. |
| [4] | 3GPP SP-170321, „ CRs to TS 26.131 and TR 26.132 on 'Development of super-wideband and fullband P.835' (Release 14) (DESUDAPS),“ Knowles Inc., Qualcomm Inc., West Palm Beach, Florida, USA, 5-9 June 2017. |
| [5] | 3GPP TS 26.131, „Terminal acoustic characteristics for telephony; Requirements,“ Release 14.1.0. |
| [6] | 3GPP TS 26.132, „Speech and video telephony terminal acoustic test specification,“ Release 14.1.0. |
| [7] | ETSI TS 103 224, „Speech and multimedia Transmission Quality (STQ); A sound field reproduction method for terminal testing including a background noise database,“ V1.1.1. ETSI, 2014. |