Source:	Xperi

Title:	VRStream Audio Quality Characterization Test Results

## Document for:	Approval

## Agenda Item:	10.5.2

- Summary

This contribution presents the Characterization Test Results of Xperi VRStream Audio Profile, with the tests conducted according to the 3GPP TS 26.259 specification [1]. The Test 1 results show the Audio Profile at 256, 384 and 512 kbps performing in the range from 85 to 95 MUSHRA points (i.e. all within the “Excellent” range), and the FOA Audio Profile at 128 kbps performing within 5 MUSHRA points of the uncompressed FAO reference (i.e. also “Excellent” with respect to its equitable reference). The optional Test 2 was not fully executed before the specified deadlines due to time and resource constraints. For Test 3, the Xperi VRStream Audio Profile renderer is equivalent to the CIBR reference, the two using the same renderer configuration.

- Test 1: Codec Quality Characterization Over Loudspeakers

2.1 Test Design

Test 1 has been conducted according to the 3GPP TS 26.259 Section 5 using the MUSHRA test methodology ITU-R BS.1534-3 [2].

Two tests were performed: Test 1a and Test 1b. Both tests consisted of ten tracks and five systems under test (plus hidden reference and two low-pass anchors). The same ten post-screened listeners were used, so that the results from Tests 1a and 1b may be combined for statistical analysis.

The following eight test signals were included in each of the MUSHRA trials:

- Reference (uncompressed)

- Content coded at 512 kbps

- Content coded at 384 kbps

- Content coded at 256 kbps

- FOA reference/anchor

- FOA reference coded at 128 kbps

- 7 kHz low-pass anchor (low-passed reference)

- 3.5 kHz low-pass anchor (low-passed reference)

The FOA conditions have been included in all of the MUSHRA trials since, as it was pointed out during the telco discussions, including the FOA signals in some of the trials and not others would make the statistical analysis of the results challenging. It has been suggested to make the FOA conditions optional for Test 1, but that would amount to not characterizing the FOA profile performance. There have also been suggestions to test the FOA conditions separately (some companies may have chosen this route), but that requires adding one more separate test. Given the above, we decided to include the FOA conditions for all the test materials. This has the added value of assessing how an FOA profile may perform, should one decide to convert various content types to an FOA representation.

Since testing 128 kbps FOA without the corresponding equitable uncompressed FOA signal would not allow assessing the impact of the compression itself, the uncompressed FOA reference/anchor is included throughout.

For the HOA-only content, the FOA reference/anchor was created by truncating HOA to FOA. For all the other content, the FOA reference was created by rendering the source material to FOA with the Audio Profile renderer. The compressed FOA bit-streams have been generated by encoding the FOA reference with the same Audio Profile encoder configuration as all the other content.

2.2 Test Processing

The source material and the corresponding metadata have been encoded with the Xperi DTS-UHD VRStream Audio Profile encoded/decoded at the target bit-rates of 256, 384, and 512 kbps, with the FOA content encoded at 128 kbps. The DTS-UHD codec [3] uses a Variable Bit-Rate (VBR) model that guarantees the target bit-rate when measured over a Group of Frames (GoF, an interval between two consecutive sync frames). The GoF duration has been set in these tests to 2 sec; i.e., the codec achieves Constant Bit-rate for each consecutive 2 sec audio segments. All the results correspond to the instantaneous VBR peak rate not exceeding 2.5 times the target bit-rate.

The Xperi VRStream rendering process is based on the CIBR configuration (see 3GPP TS 26.259 Section 6):

with the following:

- “Documented Loudspeaker Renderer” as described in the DTS VBAP renderer specification ETSI 103 584 [3]

- The same ETSI 103 584 [3] VBAP renderer used for the ESD domain signal conversion to the 7.4.1 loudspeaker configuration per Test 1 specification

The following acronyms are defined to describe the processing of the test material:

- SM: source material

  - SM-FOA: source material rendered to the FOA domain

- BS: compressed bit-stream

- HOA-ESD: conversions between ambisonics and ESD

  - ESD16-HOA: conversion from ESD-16 to HOA as specified in AHVIC-157 [5]

    - HOA-ESD16: inverse of ESD16-HOA

  - ESD4-FOA: conversion from ESD-4 to FOA as specified in AHVIC-157 [5]

    - FOA-ESD4: inverse of ESD4-FOA

- R-ESD: rendering to the ESD domain

  - R-ESD16: rendering to ESD-16

    - For Channels/Objects: ETSI 103 584 specification

    - For HOA: the HOA-ESD16 conversion

  - R-ESD4: rendering to ESD-4

    - For Channals/Objetcs: ETSI 103 584 specification

    - For FOA: the FOA-ESD4 conversion

- RX-7.1.4: rendering from ESD to the 7.1.4 loudspeaker output

  - Per ETSI 103 584 specification

With the above, the test material has been generated as follows:

- Reference (uncompressed): SM → R-ESD16 → RX-7.1.4

- FOA reference/anchor (SM-FOA, uncompressed): SM → R-ESD4 → ESD4-FOA

- Non-FOA Test Signals (compressed): BS → Decoder → R-ESD16 → RX-7.1.4

- FOA Test Signals (compressed): BS → Decoder → R-ESD4 → RX-7.1.4

2.2 Test Material

Test materials were provided by 3GPP and included tracks from the four audio profile proponents.

| Test | Track | Duration | Content Type |
| --- | --- | --- | --- |
| Test 1a | 8Obj_Music+Bird | 10.560 s | Object-based (8 Objects) |
|  | Capoeira | 11.000 s | Scene-based (4th Order HOA) |
|  | CICP_1A | 9.066 s | Channel-based (7.1.4) |
|  | CosmosTwister | 10.000 s | Channel-based (7.1.4) |
|  | Fork | 12.021 s | Object-based (10 objects) |
|  | HOA6_Musicopter | 12.000 s | Scene-based (6th Order HOA) |
|  | Indiana | 10.072 s | Channel-based (7.1.4) |
|  | leaf_A | 12.000 s | Mixed (3rd Order HOA + 4 Objects + LFE) |
|  | silent_A | 11.997 s | Mixed (3rd Order HOA + 4 Objects + LFE) |
|  | silent_B | 12.000 s | Channel-based (7.1.4) |
| Test 1b | 8Obj_reservoir | 10.560 s | Object-based (8 Objects) |
|  | audiosphere_A | 11.998 s | Mixed (3rd Order HOA + 4 Objects) |
|  | audiosphere_B | 11.997 s | Mixed (3rd Order HOA + 4 Objects) |
|  | CICP19_Festival | 11.456 s | Mixed (Channels-7.1.4 + Objects) |
|  | CosmosJungle | 11.000 s | Scene-based (6th Order HOA) |
|  | DronesAndAnimals | 12.000 s | Scene-based (6th Order HOA) |
|  | JammJam | 10.000 s | Object-based (12 Objects) |
|  | LaLechera | 11.700 s | Channel-based (7.1.4) |
|  | PitStop | 10.000 s | Channel-based (7.1.4) |
|  | Spoon | 12.021 s | Object-based (12 Objects) |

Table 1: Test Items Presented in Tests 1a and 1b

2.3 Listening Environment

The tests were performed in a BS.1116-3 compliant listening lab in XPERI’s Calabasas office over a 7.1.4 speaker layout.

2.4 Grading Interface

ARL’s STEP software was used to conduct the test and to gather listeners’ data.

Figure 1: ARL STEP Software MUSHRA Test Interface

2.5 Listening Panel

The listening panel of this test consisted of eleven XPERI/DTS employees (2 Female, 8 Male) all experienced in taking critical listening tests. Each listener was trained prior to the test as instructed in ITU-R BS.1534-3 Section 5.2. One listener was removed from both tests after being post-screened from Test 1a (to arrive at the same ten listeners for both tests).

| Test | Total Participants | Participants After Post-screening |
| --- | --- | --- |
| TEST 1a | 11 | 10 |
| TEST 1b | 11 | 10 |

Table 2: Listener Participation and Post-Screening

2.6 Results

All results are presented in graphs including average scores and 95% confidence intervals (t-distribution).

2.6.1 Overall Test 1 Scores

As the same listeners participated in Test 1a and Test 1b, the results from the two tests were combined for statistical analysis. The results are presented in Figure 2.

Figure 2: Results per System of Test 1a and Test 1b Combined

Observations

- The systems at 256, 384 and 512 kbps perform in the range from 85 to 95 MUSHRA points, i.e. all in the “Excellent” range

- The FOA signals are in the 50 to 60 range, with the 128 kbps compressed test signals less than 5 MUSHRA points lower than the uncompressed reference/anchor

2.6.2 Test 1a

Results for each system in Test 1a can be found below in Figure 3. Figure 4 illustrates how each system scores per test item.

Figure 3: Results per System of Test 1a Data

Figure 4: Results per System and Test Item for Test 1a Data

Note the generally similar observations as for the full Test 1:

- 256, 384 and 512 kbps in the range of 85-95, i.e. all in the “Excellent” range

- 128 kbps FOA performing within 5 MUSHRA points of the uncompressed FAO reference (both FOA signals considerably lower than the HOA reference)

- Individual test tracks show a bit larger range of scores, as expected, with only a couple of cases where 256 kbps falls under 80 (while 384 and 512 kbps all scoring above 80)

2.6.3 Test 1b

Results for each system in Test 1a can be found below in Figure 5. Figure 6 illustrates how each system scores per test item.

Figure 5: Results per System for Test 1b Data

Figure 6: Results per System and Test Item for Test 1b Data

Again, note the generally similar observations as for the full Test 1:

- 256, 384 and 512 kbps in the range of 85-95, i.e. all in the “Excellent” range

- 128 kbps FOA performing within 5 MUSHRA points of the uncompressed FAO reference (both FOA signals considerably lower than the HOA reference)

- Individual test tracks show a bit larger range of scores, as expected, with only a couple of cases where 256 kbps falls under 80 (while 384 and 512 kbps all scoring above 80)

- Test 2: Codec Quality Characterization Over Headphones

Test 2 specified in 3GPP TS 26.259 Section 7 has been marked as Optional. Due to the time constraints, we were not able to execute this test before the specified deadlines.

- Test 3: Renderer Comparison Test

Test 3 is designed to compare Audio Profile Reference Renderer (Test Signals) with CIBR (Anchors). The test methodology is specified in 3GPP TS 26.259 Section 6. The test methodology is based in Comparison Category Rating (CCR).

The Xperi Audio Profile renderer is based on the CIBR configuration (see 3GPP TS 26.259 Section 6):

Since our Audio Profile is configured with CIBR, this test reduces to comparing two perceptually-equivalent signals. Specifically:

- AHVIC-140 [6], Sec. 6.10 on Test 3, states: “Proposed Audio Profile shall be configured for an Operating Point providing transparent quality for all Test Materials”

- AHVIC-146 [7], Sec. Re doc 145, states: “There was agreement to split Test 3 in two sessions: one comparing the candidate with CIBR 1st order and another comparing the candidate with CIBR 2nd order.” (Correction: the latter should read 3rd order as per AHVIC-140 Sec 6.9 point 2: 1st order or 3rd order)

In other words:

- The compressed and uncompressed signals in the tests must be, per test requirements, perceptually equivalent

  - If not, one would have failed the requirement of configuring the Operating Point to provide transparent quality

- Test 3 split into 1st order and 3rd order which leads to

  - Comparing 1st order CIBR to 1st order CIBR, and

  - Comparing 3rd order CIBR to 3rd order CIBR

Conclusions:

- Xperi VRStream Audio Profile renderer is equivalent to the CIBR reference (the two using the same renderer configuration)

- Test 3 results in comparing two perceptually-equivalent Test Signals

- Conclusions

The presented Test 1 results show:

- Xperi VRStream Audio Profile at 256, 384 and 512 kbps performing in the range from 85 to 95 MUSHRA points, i.e. all within the “Excellent” range

- Xperi VRStream FOA Audio Profile at 128 kbps performing within 5 MUSHRA points of the uncompressed FAO reference, i.e. also “Excellent” with respect to its equitable reference

The optional Test 2 was not fully executed before the specified deadline due to time and resource constraints.

For Test 3, the Xperi VRStream Audio Profile renderer is equivalent to the CIBR reference, the two using the same renderer configuration.

- References

[1] 3GPP TS 26.259 “Subjective test methodologies for the evaluation of immersive audio systems”

[2] ITU-R BS.1534-3 “Method for the subjective assessment of intermediate quality levels of coding systems”

[3] ETSI 103 491 v1.1.1 “DTS-UHD Audio Format; Delivery of Channels, Objects and Ambisonics Sounds Fields”

[4] ETSI 103 584 v1.1.1 “DTS-UHD Point Source Renderer”

[5] AHVIC-157 “ESD Conversion Matrices for VRStream Audio Profile Characterization”

[6] AHVIC-140 “3rd Generation Partnership Project; Technical Specification Group SA WG4; Subjective Test Methodologies for the Evaluation of Immersive Audio Systems”

[7] AHVIC-146 “VIDEO-SQ SWG telco report on VRStream-LiQulmAS”