3GPP TS 26.118 V0.0.0 (2018-07)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group <TSG name>;

<Title 1;

Title 2>

(Release 15 |14 | 13	)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP..
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Logos: Use exactly one of the above lines. Delete the others.
Use the top line for documents which are specific to GERAN technology only.
Use the second line for documents which are specific to UTRAN technology only or to GERAN and UNTRAN technologies only.
Use the third line for documents which relate to LTE technology (regardless of their applicability to GERAN or to UTRAN technologies) in Releases prior to Release 10.
Use the fourth line for documents which relate to LTE technology (regardless of their applicability to GERAN or to UTRAN technologies) in Releases 10, 11 and 12.
Use the fifth line for documents which relate to LTE technology (regardless of their applicability to GERAN or to UTRAN technologies) in Release 13 onwards. 
Use the sixth line for documents which relate to 5G technology (regardless of their applicability to GERAN, UTRAN or LTE technologies) in Release 13 onwards.

Keywords

<keyword[, keyword, …]>

MCC selects keywords from stock list.

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2017, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	4

1	Scope	5

2	References	5

3	Definitions, Symbols and Abbreviations	5

3.1	Definitions	5

3.2	Symbols	5

3.3	Abbreviations	6

4	Examples for Styles	6

4.1	Heading Styles	6

4.2	Other common styles	6

"TSG <Name>" on the front page	7

Page setup parameters	7

Proforma copyright release text block	9

Abstract Test Suite (ATS) text block	10

<x1>	The TTCN Graphical form (TTCN.GR)	10

<x2>	The TTCN Machine Processable form (TTCN.MP)	10

Annex <A> (normative):	<Normative annex title>	11

Annex <B> (informative):	<Informative annex title>	11

B.1	Heading levels in an annex	11

Annex <X> (informative):	Change history	13

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

This clause is optional. If it exists, it is always the second unnumbered clause.

# 1	Scope

This document contains the information necessary to support the DTS-UHD audio format as an audio profile in VRStream. The contents of this document can be extracted for inclusion in TS 26.118.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	ETSI TS 103 491v1.1.1, “DTS-UHD Audio Format; Delivery of Channels, Objects and Ambisonic Sound Fields”

[2]	ISO/IEC 14496-12, fifth edition, “Information technology — Coding of audio-visual objects - Part 12: ISO base media file format”

[3]	ETSI TS 103 584v1.1.1, “DTS-UHD Point Source Renderer”

[4]	RFC 6381, “The 'Codecs' and 'Profiles' Parameters for "Bucket" Media Types”

[5]	3GPP TS 26.244, “3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Transparent end-to-end packet switched streaming service (PSS); 3GPP file format (3GP) (Release 15)”

[6]	3GPP TS 26.259, “Subjective Test Methodologies”

[7]	Unity OSC Headphone Tracking Reference, http://opensoundcontrol.org/

[8]	Google Resonance Audio API Reference, https://developers.google.com/resonance-audio/

[9]	Ambisonic Plugin, https://github.com/kronihias/ambix

[10]	Spatial Audio for Domestic Interactive Entertainment, https://www.york.ac.uk/sadie-project/database.html

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ACRONYM>	<Explanation>

# DTS-UHD VR Stream Codec and Renderer

## Renderer Description

The DTS-UHD VRStream Audio Profile utilizes the DTS-UHD codec, ETSI TS 103 491 [1], as a compression engine. The Profile supports content consisting of Channels, Objects, HOA, and combination thereof.

The rendering process is based on the Common Informative Binaural Renderer (CIBR) configuration described in 3GPP TS 26.259 [6] illustrated by:

Figure 1: CIBR Renderer

The CIBR-based renderer:

Rendering to virtual speakers positioned according to the Equivalent Spatial Domain representation (ESD)

VBAP renderer of channel-based and object-based content to the ESD domain

Converting HOA content to the ESD domain

Combining channel-based, object-based, and HOA-based content in the ESD domain

Sound field rotation

Converting ESD to the FOA/HOA representation

Rotation of the sound field using an AmbiX plugin [9], according to a motion sensor signal

Audio Output

Speakers

  - Converting FOA/HOA to the ESD domain

  - VBAP renderer to specified speaker locations

Headphones

  - Binauralization using Google Resonance [8]

The definition of Equivalent Spatial Domain can be found in TS 26.260 Clause 4.1.1. The VBAP implementation used for the “Documented Loudspeaker Renderer” is the DTS-UHD renderer as specified in ETSI 103 584 [3].

In our implementation of the CIBR renderer, we use the HTC Vive head-tracking devide, i.e., an actual VR head-set. Unity to utilized to pass head-tracking information to the Ambix rotator plugin for real-time implementation of the system.

## Codec Constraints

A DTS-UHD VRStream elementary stream shall conform to ETSI TS 103 491 [1], with the following additional constraints:

The Audio Chunk Type shall be 0x01.

A default Audio Presentation shall be present in the elementary stream.

The bit-rates shall be in the range of 32 to 128 kbps per full-bandwidth waveform.

While the DTS-UHD codec does not define a maximum time between sync frames, it is recommended to limit the period between sync frames to 2 seconds.

## Renderer Constraints

The VRStream Renderer constraints are dependent on the computational ability of rendering platform. No additional constraints are needed specific to the DTS-UHD solution.

## Renderer Motion to Sound Latency

The CIBR renderer Motion to Sound Latency been measured at between 27.7 ms and 27.9 ms. These measurements are based on a CIBR Max/MSP implementation and exclude audio playback buffer and motion to quaternion output latency, as reported in contribution AHVIC-142.

## Complexity of Decoder

Computational footprint of the DTS-UHD decoder has been verified on Android NEXUS 9 floating-point ARMv8, with a gcc compiled decoder executable. The measured results are in terms of MCPS (Million Cycles Per Second), with the average and peak per-frame MCPS. The average and peak MCPS vary slightly as the bit-rate increases, up to 10 MCPS on average and 14 MCPS peak per decoded waveform at the highest bit-rates.

## Default set of HRTF for Binaural Rendering

The default HRTF, used by the Google Resonance Plugin, is the Neumann KU100 binaural head from the SADIE database [10].

# DTS-UHD VRStream Interfaces

## Interfaces for File Storage and Streaming

### Overview

In this clause, operational rules for boxes defined in 14496-12 [2] as well as definitions of private extensions to those ISO file format standards are described.

An ISO media file may contain one or more audio tracks. The tracks are composed in conformity to ISO base media file format and described in 14496-12 [2], for an audio track structure. The sub-sections that follow describe the construction of an audio track containing DTS-UHD audio.

### Track Header Box

The syntax and values for the Track Box ('tkhd') and its sub-boxes shall conform to 14496-12 [2], and the following fields of each box shall be set to the following specified values.  There are some “template” fields declared to use; see the syntax of 'tkhd' in 14496-12 [2].

flags = 000007h, except for the case where the track belongs to an alternate group;

layer = 0;

volume = 0100h;

matrix = {00010000h,0,0,0, 00010000h,0,0,0, 40000000h};

width = 0;

height = 0;

### Sync Sample Box

All DTS-UHD sync frames are samples that are Sync Samples (random access points). All DTS-UHD non-sync frames are samples that are not random-access points.

Per the requirements of 14496-12 [2] the Sync Sample box shall be present if the DTS-UHD track contains samples that are not Sync Samples. More information about DTS-UHD random access points is provided in section [2].

### Handler Reference Box

The syntax and values for the Handler Reference Box ('hdlt') shall conform 14496-12 [2], and the fields of this box shall be set to the following specified values.

handler_type = ‘soun’

### Sound Media Header Box

The syntax and values for the Sound Media Header box ('smhd') shall conform to 14496-12 [2], and the fields of this box shall be set to the following specified values.

balance = 0;

### Sample Description Box

The SampleEntry box for DTS-UHD is derived from the AudioSampleEntry box defined in 14496-12 [2]. The DTS-UHD specific SampleEntry box is identified by a unique codingname value (see Table 1). It indicates the audio format used to encode the audio track and describes the configuration of the audio elementary stream.

Table 1: Defined Audio Formats

| codingname | Description |
| --- | --- |
| dtsv | DTS-UHD VR bitstream compatible with Decoder Profile = 4 |

### AudioSampleEntry Box for DTS-UHD Formats

The syntax and values of the AudioSampleEntry Box shall conform to DTSUHDSampleEntry.

The configuration of the DTS-UHD elementary stream is described in the DTSUHDSpecificBox (‘udts’), within DTSUHDSampleEntry. The syntax and semantics of the DTSUHDSpecificBox are defined in section 5.1.9.

### DTSUHDSampleEntry

DTSUHDSampleEntry extends the AudioSampleEntry box defined in 14496-12 [2]:

class DTSUHDSampleEntry (codingname) extends AudioSampleEntry (codingname) {

DTSUHDSpecificBox()	// ‘udts’ box

}

For DTSUHDSampleEntry(), the following values inherited from AudioSampleEntry are set as follows:

codingname shall be set to one of the values listed in Table 1.

channelcount shall be set to the number of decodable output channels.

samplesize shall be set to 16.

samplerate shall be set to the base sampling frequency, as indicated by the BaseSamplingFreqCode field of the DTSUHDSpecificBox.

### DTSUHDSpecificBox

The syntax and semantics of the DTSUHDSpecificBox ('udts') are shown below.

Note, many of the fields of the DTSUHDSpecificBox are identical to fields of the same name in the DTS-UHD Audio Descriptor for MPEG-2 systems and reference those sections of this document for definition.

Table 2: DTS-UHD Specific Box

| Syntax |  | Reference |
| --- | --- | --- |
| class DTSUHDSpecificBox extends Box (‘udts’) { |  |  |
| bit (6) | DecoderProfileCode; | 5.1.10.1 |
| bit (2) | FrameDurationCode; | 5.1.10.2 |
| bit (3) | MaxPayloadCode; | 5.1.10.3 |
| bit (5) | NumPresentationsCode; | 5.1.10.4 |
| unsigned int(32) | ChannelMask; | 5.1.10.5 |
| bit (1) | BaseSamplingFrequencyCode | 5.1.10.6 |
| bit (2) | SampleRateMod | 5.1.10.7 |
| bit (3) | RepresentationType; | 5.1.10.8 |
| bit (3) | StreamIndex | 5.1.10.9 |
| bit (1) | ExpansionBoxPresent; | 5.1.10.10 |
| bit (NumPresentations) | IDTagPresent[NumPresentations]; | 5.1.10.11 |
| ByteAlign [0..7] |  | 5.1.10.12 |
| for (i = 0; i < NumPresentations; i++){ |  |  |
| if (IDTagPresent[i] == TRUE) |  |  |
| unsigned char } | PresentationIDTag[16]; | 5.1.10.13 |
| if (ExpansionBoxPresent) { |  |  |
| box DTSExpansionBox []; } |  | 5.1.10.14 |
| } |  |  |

### Semantics for DTSUHDSpecificBox

#### DecoderProfileCode

For 3GPP VRStream applications, DecoderProfileCode is nominally set to 2.

#### FrameDurationCode

The frame duration code indicates FrameDuration according to Table 3. The units for FrameDuration are sample periods relative to BaseSamplingFrequency defined below.

Table 3: Frame Duration

| FrameDurationCode | FrameDuration (samples) |
| --- | --- |
| 0 | 512 |
| 1 | 1024 |
| 2 | 2048 |
| 3 | 4096 |

For VRStream applications, Frame Duration shall always equal 1024.

#### MaxPayloadCode

MaxPayloadCode indicates the maximum size of the audio payload, as indicated in Table 4. Note that Maxpayload is not the size of the largest audio frame in the presentation, but rather a "not to exceed" value for buffer configuration and digital audio interface purposes, and is inclusive of all required preambles, headers, burst spacing, etc.

Table 4: MaxPayload

| MaxPayloadCode | MaxPayload (bytes) |
| --- | --- |
| 0 | 2048 |
| 1 | 4096 |
| 2 | 8192 |
| 3 | 16384 |
| 4 | 32768 |
| 5 | 65536 |
| 6 | 131072 |
| 7 | reserved |

Note that MaxPayloadCode can be used to determine the minimum IEC sample clock frequency required to transfer the DTS-UHD stream on a digital audio interface.

For VRStream applications, MaxPayloadCode shall always be equal to 0.

#### NumPresentationsCode

This field represents the number of audio presentations encoded within DTS-UHD elementary stream. The valid range of the bitstream parameter m_ucNumAudioPres defined in TS 103 491 [1] is from 0 to 31. The relationship between NumPresentationsCode and m_ucNumAudioPres shall be as follows:

NumPresentations = m_ucNumAudioPres + 1.

#### ChannelMask

A bit mask that indicates the channel layout encoded in the default presentation of the DTS-UHD bitstream according to the bit-to-channel-label mapping described in Table 5. Note that the ChannelMask table is derived from Channel Bitmask defined in TS 103 584 [3].

Table 5: Speaker Labels for ChannelMask

| Label | Notation | ChannelMask |
| --- | --- | --- |
| C | CENTER | 0x00000001 |
| L | LEFT | 0x00000002 |
| R | RIGHT | 0x00000004 |
| Ls | SRRD_LEFT | 0x00000008 |
| Rs | SRRD_RIGHT | 0x00000010 |
| LFE1 | LFE_1 | 0x00000020 |
| Cs | SRRD_CENTER | 0x00000040 |
| Lsr | REAR_SRRD_LEFT | 0x00000080 |
| Rsr | REAR_SRRD_RIGHT | 0x00000100 |
| Lss | SIDE_SRRD_LEFT | 0x00000200 |
| Rss | SIDE_SRRD_RIGHT | 0x00000400 |
| Lc | LEFT_CENTER | 0x00000800 |
| Rc | RIGHT_CENTER | 0x00001000 |
| Lh | HIGH_LEFT | 0x00002000 |
| Ch | HIGH_CENTER | 0x00004000 |
| Rh | HIGH_RIGHT | 0x00008000 |
| LFE2 | LFE_2 | 0x00010000 |
| Lw | LEFT_WIDE | 0x00020000 |
| Rw | RIGHT_WIDE | 0x00040000 |
| Oh | TOP_CENTER_SRRD | 0x00080000 |
| Lhs | HIGH_SIDE_LEFT | 0x00100000 |
| Rhs | HIGH_SIDE_RIGHT | 0x00200000 |
| Chr | HIGH_REAR_CENTER | 0x00400000 |
| Lhr | HIGH_REAR_LEFT | 0x00800000 |
| Rhr | HIGH_REAR_RIGHT | 0x01000000 |
| Cb | LOW_FRONT_CENTER | 0x02000000 |
| Lb | LOW_FRONT_LEFT | 0x04000000 |
| Rb | LOW_FRONT_RIGHT | 0x08000000 |
| Ltf | TOP_FRONT_LEFT | 0x10000000 |
| Rtf | TOP_FRONT_RIGHT | 0x20000000 |
| Ltr | TOP_REAR_LEFT | 0x40000000 |
| Rtr | TOP_REAR_RIGHT | 0x80000000 |

#### BaseSamplingFrequencyCode

BaseSamplingFrequencyCode is derived from the parameter m_unClockRateInHz defined in TS 103 491 [1]. This field shall indicate the base sampling frequency, according to Table 6.

Table 6: Base Sampling Frequency

| BaseSamplingFrequencyCode | Clock Rate (Hz)1 |
| --- | --- |
| 0 | 44100 |
| 1 | 48000 |

#### SampleRateMod

This field indicates the sampling frequency of the audio samples stored in the bitstream. It modifies the BaseSamplingFrequency by multiplying by the corresponding value indicated in Table 7.

Table 7: SampleRateMod

| SampleRateMod | Multiplier |
| --- | --- |
| 0 | 1 |
| 1 | 2 |
| 2 | 4 |
| 3 | 8 |

For 3GPP VRStream Applications, SampleRateMod shall be set to 1.

#### RepresentationType

This value conveys general information about the type of spatial audio signals encoded in the default presentation of the DTS-UHD bit-stream according to the description in Table 8. This parameter shall match the bitstream metadata parameter ucObjRepresTypeIndex defined in TS 103 491 [1].

Allowed values for RepresentationType are affected by and may affect ChannelMask, as indicated.

Table 8: Representation Type

| RepresentationType | Description |
| --- | --- |
| 0 | Multi-channel representation in layout described by ChannelMask |
| 1 | Multi-channel representation in layout described by ChannelMask obtained by rendering 2D content, (i.e. no height channels), with spatial resolution higher than indicated by the encoded layout, (e.g. LtRt, 5.1ES) |
| 2 | Multi-channel representation in layout described by ChannelMask obtained by rendering 3D content, (i.e. includes height), with spatial resolution higher than indicated by the encoded layout, (e.g. 5.1 NeoX), |
| 3 | Binaurally processed audio (2 waveforms, L, R for headphones) |
| 4 | Ambisonic representation |
| 5 | Audio tracks with or without an associated mixing matrix to a particular channel mask based output layout |
| 6 | 3D Objects based representation |
| 7 | Combination of objects with different representation types |

When RepresentationType has a value of 0, ChannelMask shall be set to a value that directly corresponds to the encoded channels.

When RepresentationType has a value of 1 or 2, ChannelMask shall be set to a value that corresponds to the layout of the encoded channels into which higher spatial resolution components have been rendered (i.e. the corresponding channels that have been delivered to the audio compression engine).  For example, LtRt material would have a ChannelMask of 0x00000006, representing the two channels of encoded audio content that may be presented at a higher spatial resolution on, say, a 5.1-channel system.

When RepresentationType is equal to 3, ChannelMask shall be set to 0x00000006, indicating Left and Right channels are active.

When RepresentationType is greater than  3, ChannelMask shall be set to 0x00000000, indicating that the audio presentation is channel layout agnostic.

#### StreamIndex

When a given Audio Program is delivered using multiple streams, StreamIndex is the index used to indicate stream priority for prioritizing mixing metadata. The main stream shall have StreamIndex=0, while auxiliary streams shall have StreamIndex values in the range of 1-7.

If the Audio Program is delivered in a single stream, StreamIndex shall be 0.

#### ExpansionBoxPresent

This flag, if set to 1, indicates that at least one private box is present at the end of the DTSUHDSpecificBox.

#### IDTagPresent

One IDTagPresent flag exists for each presentation in the audio track, with the total number indicated by NumPresentations. The IDTagPresent flags appear in order of the increasing value of the Audio Presentation Index (ucAudPresIndex) defined in ETSI TS 103 491 [1]. If IDTagPresent is equal to 1 for a given presentation, then a 16 byte ID for that presentation (PresentationIDTag) shall be present in the descriptor.

#### ByteAlign

A variable number of bits (from 0 to 7) shall be inserted to pad to the end of the current byte boundary.

#### PresentationIDTag

PresentationIDTag for a specific audio presentation shall be present only when the corresponding IDTagPresent flag is set to 1. This 16-byte field can be used to identify the corresponding audio presentation.

#### DTSExpansionBox

If ExpansionBoxPresent is set to 1, then one or more private boxes referred to as DTSExpansionBoxes can be found at the end of the ‘udts’ box, filling the remaining space of the box. As defined in 14496-12 [2] section 4.2, boxes with an unrecognized type shall be ignored by the receiver.

Playback devices not equipped to support one or more of these private boxes shall be able to depend on the defined parameters in ‘udts’ to play the audio track with its default presentation parameters.

### Storage of DTS-UHD Elementary Streams

#### Overview

Storage of DTS-UHD elementary streams within an ISO media file shall be according to this section.

A sample, as defined in ISO/IEC 14496-12 [2] shall consist of a single DTS-UHD audio frame, including the leading syncword, as defined in TS 103 491 [1].

#### Random Access Points

DTS-UHD audio samples may be either sync frames or non-sync frames, as identified by the first four bytes of the sample, which constitute the SYNCWORD, shown in Table 9.

Table 9 : DTS-UHD SYNCWORD

| SYNCWORD | Designation |
| --- | --- |
| 0x40411BF2 | DTS-UHD Sync Frame |
| 0x71C442E8 | DTS-UHD Non-sync Frame |

For all DTS-UHD tracks containing both sync frames and non-sync frames, the first sample in the track shall be a sync frame. For fragmented track files, the first sample in each fragment shall be a sync frame.

For track fragments containing multiple mdat boxes, the first DTS-UHD frame stored in each mdat shall be a sync frame.

For DTS-UHD tracks containing non-sync frames, the methods provided in 14496-12 [2] for locating random access points such as the sync sample box ('stss'), the track fragment random access box ('tfra'), and random access information in 'trun', 'traf' and 'trex', shall be applied as required.

## Output to Headphones and Loudspeakers

The rendered output for loudspeakers shall be a multichannel 24-bit linear PCM stream with one output per rendered channel. The rendered channel configuration is an input to the renderer according to ETSI TS 103 584[3].

The rendered output for headphones shall be a stereo 24-bit linear PCM stream output from the Binaural Renderer.

## Head Tracking

Head-tracking information is passed to the Ambix rotator plugin using Unity Open Sound Control (OSC) messages [7].

## External Render API

The DTS-UHD VRStream profile is not defining an external renderer.

## API To Define HRTF

The DTS-UHD implementation does not include a specific API to define or modify HRTF. A reference HRTF renderer that can be use as an example is the Google Resonance Audio API Reference [8].

# Signalling

## Signalling for File Based Delivery

When DTS-UHD streams are are stored in .3GP files for file download delivery, the rules defined for storage of ISOBMFF tracks defined in 26.244[5] shall be applied.

## Signalling for DASH Delivery

### Introduction

This chapter describes the DTS-UHD audio track in relation to the ISO file and the constraints on the DTS-UHD audio formats.

### DASH Media Presentation Description parameters for DTS-UHD

This section details the DASH MPD attributes and elements used with DTS-UHD.

Table 10: DTS-UHD DASH Elements and Attributes

| Element or Attribute Name | Description |
| --- | --- |
| @codecs | The value of the codecs attribute shall be created according to the syntax described in RFC 6381 [4]. For DTS-UHD VR streams @codecs shall be represented as the 4CC appended with a channel count profile. Available channel count profiles are 16, 24 and 32 waveforms. For example, a typical VRStream track with up to 24 waveforms is represented as: @codecs = dtsv.24 |
| Preselection@tag | If the DTS-UHD elementary stream contains multiple Audio Presentations, this field shall map to a selectable Presentation specified within a DTS-UHD stream by value of ucAudPresIndex, as defined in TS 103 491 [1] |
| AdaptationSet@tag | This field shall list selectable Presentations specified within a DTS-UHD stream by values of ucAudPresIndex, as defined in TS 103 491 [1] |
| ContentComponent@tag | This field shall correspond to a value of ucAudPresIndex as defined in TS 103 491 [1] for the stream indicated in the tag. |
| AudioChannelConfiguration | DTS-UHD shall use the following scheme URI: tag:dts.com, 2018:uhd:audio_channel_configuration and the value shall be the ChannelMask as defined in 5.1.10.5 of this specification. |
| @audioSamplingRate | The value shall equal the maximum audio sampling frequency of the decoded audio presentation. |
| @mimeType | The MIME type to be used with DTS-UHD shall be ”audio/mp4”. |
| RandomAccess | The type to be used with DTS-UHD shall be ”closed”, i.e. the SAP type is 1. |
| Language | The language indicated should equal the language of the dialogue element. |
| Role | The Role@value should be set by the content author. |
| Label | The Label for a Representation should be set by the content author. |

Annex <X> (informative):
Change history

This is the last annex for TSs which details the change history using the following table.

This table can be used for recording progress during the WG drafting process till TSG approval of this TR.

For TRs under change control, use one line per approved Change Request

Date: use format YYYY-MM

TSG # : use format RAN#55

CR: four digits, leading zeros as necessary

Rev: blank, or number (max two digits)

Cat: use one of the letters A, B, C, D, F

Subject/Comment: for TSs under change control, include full text of the subject field of the Change Request cover

New vers: use format n[n].n[n].n[n]

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |

Change history of this template:

| 2001-07 | Copyright date changed to 2001; space character added before TTC in copyright notification; space character before first reference deleted. | 1.3.3 |
| --- | --- | --- |
| 2002-01 | Copyright date changed to 2002. | 1.3.4 |
| 2002-07 | Extra Releases added to title area. | 1.3.5 |
| 2002-12 | “TM” added to 3GPP logo | 1.3.6 |
| 2003-02 | Copyright date changed to 2003. | 1.3.7 |
| 2003-12 | Copyright date changed to 2004. Chinese OP changed from CWTS to CCSA | 14.0 |
| 2004-04 | North American OP changed from T1 to ATIS | 1.5.0 |
| 2005-11 | Stock text of clause 3 includes reference to 21.905. | 1.6.0 |
| 2005-11 | Caters for new TSG structure. Minor corrections. | 1.6.1 |
| 2006-01 | Revision marks removed. | 1.6.2 |
| 2008-11 | LTE logo line added, © date changed to 2008, guidance on keywords modified; acknowledgement of trade marks; sundry editorial corrections and cosmetic improvements | 1.7.0 |
| 2010-02 | 3GPP logo changed for cleaner version, with tag line; LTE-Advanced logo line added; © date changed to 2010; editorial change to cover page footnote text; trade marks acknowledgement text modified; additional Releases added on cover page; proforma copyright release text block modified | 1.8.0 |
| 2010-02 | Smaller 3GPP logo file used. | 1.8.1 |
| 2010-07 | Guidance note concerning use of LTE-Advanced logo added. | 1.8.2 |
| 2011-04-01 | Guidance of use of logos on cover page modified; copyright year modified. | 1.8.3 |
| 2013-05-15 | - Changed File Properties to MCC macro default - Removed R99, added Rel-12/13 - Modified Copyright year - Guidance on annex X Change history | 1.8.4 |
| 2014-10-27 | Updated Release selection on cover. In clause 3, added "3GPP" to TR 21.905. | 1.8.5 |
| 2015-01-06 | New Organizational Partner TSDSI added to copyright block. Old Releases removed. | 1.9.0 |
| 2015-12-03 | Provision for LTE Advanced Pro logo Update copyright year to 2016 | 1.10.0 |
| 2016-03-08 | Standarization of the layout of the Change History table in the last annex.(Unreleased) | 1.11.0 |
| 2016-06-15 | Minor adjustment to Change History table heading | 1.11.1 |
| 2017-03-13 | Adds option for 5G logo on cover | 1.12.0 |
| 2017-05-03 | Smaller 5G logo to reduce file size | 1.12.1 |