3GPPSA5-TM Forum Model Alignment meetings 	S5vTMFa167

# Source:	Editor (Ciena)

# Title:	3GPP/TM Forum Concrete Model Relationships and Use Cases

# Document for:	Approval

Agenda Item:

1		Decision/action requested

This V0.7 includes enhancements on S5vTMFa165 made at the Wien meeting. Ready for further discussion.

2		References

None

3		Current status

The following is a standalone document that captures the current status of and agreements related to the concrete relationships between the 3GPP and TM Forum models. The document sets out the model in the context of a number of use cases designed to satisfy the operator top ten requirements as discussed in the Joint 3GPP/TM Forum model alignment project.

[Editor’s note: Version 0.7 (S5vTMFa167) replaces version 0.6 (S5vTMFa165)]

=================

Fixed Mobile Convergence (FMC) Network Management –

3GPP/TM Forum Concrete Model Relationships and Use Cases Version 0.7

Source: Joint 3GPP/TM Forum model alignment project

Editor: Nigel Davis (Ciena)

Disclaimer: Final agreement on this document cannot occur until “Governance / Terms of Reference / Working Procedures” agreements between the involved organizations (including 3GPP & TMF) have been established.

- Contents

1	Background and Objective	2

2	References	3

3	Definitions, symbols and abbreviations	3

3.1	Definitions	3

3.2	Symbols and abbreviations	3

4	Introduction	4

5	Assurance – Fault Location Context	4

5.1	Operator Activity Context description	4

5.2	Operator requirements covered	4

5.3	Specific use cases	4

5.4	Stylized network	5

5.5	Problem statement using stylized network model examples	6

5.6	Solution proposal	9

5.7	Potential management environment solution forms	11

5.8	Resultant model	12

5.9	Implied changes for TM Forum and 3GPP	12

6	Fulfilment – Service construction context	12

7	Fulfilment – Engineering works context	12

8	Complete model proposal	13

9	Appendix A:	13

# Background and Objective

Ongoing industry convergence and pressure to reduce cost is placing ever increasing emphasis on the need to rationalize and align various network management aspects across boundaries of standards/specifications producing organizations. The cost, resulting from integration and management challenges, of the lack of a coherent treatment of the whole network is becoming increasingly apparent to the point where operators of networks are demanding action.

To drive for action the operators are developing requirements that articulate aspects of the problem to be tackled. In addition ISV and equipment vendors are also developing requirements. The intention from these two activities is to develop a consistent and complete set of requirements to cover the problem space. Analysis of these requirements led to the identification of a number of use cases that highlight detailed examples of the problems and enable solutions to these problems to be explored. It is intended that through these explorations:

- Specific relationships between 3GPP model elements and TM Forum model elements can be identified

- Overlaps of the 3GPP and TM Forum models can be highlighted

- Gaps in modelling, where neither 3GPP nor TM Forum provide suitable capability, can be plugged

It is intended that this document:

- Captures the use cases considered explaining them in the context of:

  - The requirements and scenarios that occur as a result of FMC

  - Real application needs described in terms of an operator activity context

- Develops the use cases to the form of detailed scenarios

  - Explaining a stylised network diagrams

  - Homing in on the critical area of interaction

- Provides pictorial forms for the model interaction

  - Showing the essence of the model relationships

  - Offering a familiar and penetrable form

Once the use case is confirmed as an appropriate valuable case to tackle it is intended that work continues to:

- Provide detailed formal views of the model interactions

  - These will be in  UML and/or tabular form as appropriate

- Offer one or more proposals for solutions

- Evaluate the proposals to achieve a viable recommendation

- Use the use cases and requirements to validate the recommendation

- Explore and propose work phasing and migration to achieve the support for the solution in the respective models

The work covered by this document will be carried out in conjunction with work on model harmonisation (reference) as there is interplay between these two areas. It is expected that both activities will draw from and as a feed into each other.

Through the progression described above there will be an expectation of close engagement of operator as well as the wider 3GPP and TM Forum community. This will involve engagement in the process of:

- Choosing the cases for analysis with focus on the relationship to operator requirements

- Confirming the appropriateness of the detailed scenarios especially in the context of the pictorial form of the model interactions

- Validating the solution proposals at both a high level and a detailed level especially in the context of the solution phasing

This will lead to focus on areas of the model for intense alignment and to the development of mechanisms for model interrelationship. The proposal set out in this document supports the Objectives set out in [1].

This document proposes some specific structure of model and realisation of that structure across 3GPP and TM Forum.

The proposal takes advantage of the TM Forum Information Framework (SID) [7] and the TM Forum Integration Framework (MTNM/MTOSI) [8] and 3GPP SA5 group work [14] (NRM IRPs etc).

The proposal provides an initial pragmatic solution for reduction in cost of integration and improvement of degree of integration for the purpose of End-to-End management for a specific aspect of the problem.

# References

[Editor’s note: This section is still to be updated]

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	FMC Federated Network Model (FNM) [current version is S5vTMFa145]

# Definitions, symbols and abbreviations

## Definitions

[Editor’s note: Need to reference definitions in the other documents and add any local ones here.]

## Symbols and abbreviations

[Editor’s note: Need to reference symbols and abbreviations in the other documents and add any local ones here.]

# Introduction

This document sets out a number of independent operator activity contexts where each context has been chosen to exercise one or more of the operator requirements and to lead to points of intersection of the network model work of 3GPP and TM Forum where some degree of harmonisation would consequently seem valuable.

Each context description focuses on one or more use cases that suitably describe the relevant details of that operator activity with respect to an expected area of intersection. The use cases are first explained and then depicted via one or more stylized network and model diagrams. The model diagram is then developed to a formal model structure.

Finally the set of changes proposed cumulated from all context analyses are provided as a coherent recommendation.. Respective concrete models must use this model to support the FMC use cases to claim FMC network management compliance.

# Assurance – Fault Location Context

## Operator Activity Context description

This operator activity context focuses on the location and repair of a fault. A number of fault examples are used. Each fault example is chosen at a point in the network where it will cause an effect in both the wireless and wire-line part of the network. The examples are used together to demonstrate improvements in the operations characteristics that could be achieved by interconnecting and harmonizing the TM Forum and 3GPP models.

The descriptions focus on the detection of the alerting of the presence of a fault. The descriptions assume a fault location and repair process but do not impose any particular detail on that process.

The primary focus is on the relating of reports of problems from the two network fragments (3GPP Concrete Model and TM Forum Concrete Model) rather than the rationalisation of the specific report details (covered by the other harmonisation activity [Editor’s note: add formal reference]).

## Operator requirements covered

[Editor’s note: Document and reference (as appropriate) the requirements and explain why each requirement is relevant. A key focus is quality of alarms and ease of interrelationship of those alarms (which is a sense is also quality of alarms)]

## Specific use cases

- Scenario

A converged Radio/Transport network is operated from a single Network Management System (NMS). The scenario is as follows:

- Problem occurs in the network

- Alarms are generated by systems managed via the 3GPP model and the TM Forum model

- Network Context

The OC manages all Network Elements (NE) of both the radio network and transport network. The network is operational

The radio network elements are managed using a 3GPP management approach and the transport network is managed using a TM Forum management approach. The radio network element includes transport network terminations.

- Description

- Fault location system in the  NMS receives alarms from both the radio NE and the transport NE and interrelates them using network and temporal information

- Fault location system identifies the Primary/Root Cause alarm(s)/event(s)

- Service impact system identifies degree of impact and criticality of repair

- Repair action initiated

- Alarms clear and services are restored

- Implications

In a normal network there will be several/many simultaneous problems and work activities to be balanced. The network operator will want to prioritise scarce maintenance resource to fix the problems and to choose the right sequence of actions to maximise return. Therefore knowing both the root cause and location of the fault and the degree of impact from each fault is vital.

In order to solve the root cause for the radio network problem efficiently the OC needs to get information on the alarms in the underlying transport network and radio networks in a consistent and inter-connected model form so as to be able to easily interrelate the alarms.

## Stylized network

A basic network has been chosen to simplify the consideration and a simple fault in the optical network has been chosen to highlight the case.

[Editor’s note: The diagram may need to be enhanced to better depict the failure point. This may entail further simplification of the network so as to remove clutter in the second diagram]

Fig. 1 Example network configuration – high level

[Editor’s note: Need to show position of a fault to line up the next figure with the detailed diagrams following].

Fig. 2 Example network configuration – detailed – layered view

As a result of the fault in the optical network delivery there is either:

- Degradation of service due to errors

  - Assumes that the failures are soft so as not to cause the protection schemes to activate but sufficient to cause errors and low level impact on the service sufficient to push it outside SLA

- Degradation of service due to reduced capacity

  - Where an optical system has failed and although there are other systems providing alternative feed the network is loaded at peak such that under this failure scenario there is some degree of overbooking of the network during peak usage

- Loss of resilience

  - Where the network has capacity to carry all necessary traffic even with the specific optical failure but there is now insufficient capacity to enable engineering works on other optical systems and the overall network is at risk of degradation or loss of service if there are any further failures

- Loss of service

  - Where there has been a failure in one or more optical systems as a result of shared risk or misfortune such that the capacity of the network has dropped to a point where some service is lost.

Based upon the work prioritisation consideration noted in an earlier section the operator will need to understand both the location of a fault and its impact for each of the above cases. Clearly any ambiguity especially the appearance of two separate faults where there is actually one fault and a consequence will waste valuable maintenance resource. It is therefore vital that in each case location clarity can be achieved.

## Problem statement using stylized network model examples

The position of the fault highlighted on the earlier network diagram is magnified in the following diagrams. Prior to highlighting the fault positioning it is important to show an example of the arrangement of management solution components in the mixed wireless-wire-line solution and the positioning of some of the key model entities.

Fig. 3 Example of current deployment context using unrelated concrete models

The figure above shows a deployment scenario using unrelated concrete models. The figure shows two NEs, one wireless and one wire-line being managed by two separate management solutions, one based upon 3GPP and one based upon TM Forum, both feeding to a single OSS where some form of network management is being performed that includes fault localisation. The figure shows the unrelated TM Forum and 3GPP classes that would represent aspects of behaviour of the respective NEs. There would be many other classes in the models for the NEs but only the classes shown are considered relevant in the following cases. It should be noted that the PTP/CTP combination represents the optical port, its layers/protocols and its flexible connection termination capability.

This scenario can be generalised to emphasise the model differences as below. The figure intentionally does not show the management structure and management interface as this is NOT the focus of this document.

Fig. 4 Generalised form of current deployment context using unrelated concrete models

The next two figures show the location of alarms in the models under two different fault scenarios. The first is a unidirectional fibre break and the second is a laser fail.

Fig. 5 Alarm positioning for “Fibre Break” case

Note: With respect to the red circle in the eNodB function it is recognised that under some conditions there will be no alarm either due to resilience or due to the alarm on the eNodeB function will be suppressed in the NE with wireless access by the alarm on the Transport Entities. However there are often more subtle scenarios where degradations could not be suitably apportions. In addition the indication may be necessarily exposed not suppressed to indicate a service impact.

In the fibre break case shown above, one indication of the problem is from the NE with wireless access. It should be noted that there is no standardised entity in the 3GPP NRM against which to report the fibre fail.

Fig. 6 Alarm positioning for “Laser Fail” case

In the laser fail case shown above there is an indication of a problem from the NE with wireless accesswhich is identical to the indication in the fibre break case and there is also an indication of a problem from the wire-line NE which is raised against the port represented by the PTP.

Using the current solution as depicted above there is no relationship between the alarmed IRP Transport entity (of the wireless NE) and the alarmed PTP (of the wire-line NE) and as a consequence analysis (e.g. correlation) of the alarms will prove difficult. Hence it can be argued that the quality of the alarm reporting in these scenarios can be improved from the perspective of fault isolation (a primary process of the Management Environment) by better interrelationship of the model representations of the two NEs.

## Solution proposal

The following solution proposal introduces key model interrelationships that would better enable fault isolation. This solution description incorporates further classes from the current models, the 3GPP Link and the TM Forum Topological Link, and show how they can be interrelated. The solution also show the introduction of the PTP and CTP concepts into the NE with wireless access in recognition of the fact that the wire-line protocols terminate in that NE and hence a representation of those protocol termination is required. There is currently no standardised NRM IRP model for this area of the problem domain. The TM Forum concepts can simply be used essentially as they are. There will clearly be a need for some adjustments and enchance the IRP Transport entities to accommodate these concepts in the 3GPP scope.

In both fault cases shown in the figures below the signal fail alarm on the NE with wireless access will be raised against enchanced IRP Transport entities (that have incorporated the PTP concept) and in the Laser Fail case the alarms at both ends of the fibre can be related via an understanding of the ToplogicalLink between the PTP and the enhanced IRP Transport entity.

The TopologicalLink between the two NEs may be discovered from the network if appropriate discovery protocols are enabled or may be operator provisioned. The specific approach to this is not considered further here. The adjacency information related to the TopologicalLink will be conveyed to the Management Environment using the foreign TP capability. This will not be discussed further here as it is no different from that already used in the core of the wire-line environment.

Fig. 7 Alarm positioning for “Fibre Break” case using related concrete models

Fig. 8 Alarm positioning for “Laser Fail” case using related concrete models

[Editor’s note: Further details of the case to be added to summarise the fault localisation and repair for the two situations described above. This may benefit from further division into sub sections.]

## Potential management environment solution forms

Although, as emphazied earlier, the scope of this document is information model convergence and resolution of the management environment solution is out of scope it is considered important to highlight some management environment deployment alternatives that will require further resolution.

It is clear that there are several potential management environment implementation forms that will cater for the introduction of PTP/CTP concepts to describe the wire-line ports of NEs with wireless access. Two potential forms are as follows:

- PTP/CTP/TopologicalLink model concepts built into the 3GPP NRM IRPs and exposed via 3GPP IRP Agent using external references to link the two NEs

- Two interfaces on the manager of the NE with wireless access (one MTOSI and one 3GPP) using references between the parts of the model within the NE with wireless access to join the model

The following figures depict these two potential implementation forms.

Fig. 9 Alarm flow through IRP Agent for NE with wireless access

Fig. 10 Alarm flow through IRP Agent and EMS for NE with wireless access

Clearly EMS 1 and the IRP Agent may share the same hardware platform and communications port etc as appropropriate.

## Resultant model

[Editor’s note: This section will provide model fragments from TM Forum and 3GPP that detail the relevant classes described in the NE examples above and will then bolt these models together showing the new relationships etc. The section will then emphasise the need for remotable relationships. This section will not be developed further until operator confirmation of the applicability of the cases has been received.]

## Implied changes for TM Forum and 3GPP

[Editor’s note: This section will provide a detailed list of the specific changes proposed for each model as a result of the analysis of the case and the highlighted model form. The section will discuss both additions, interdependency of the additions and potential phasing for the additions (with any recognised in-service migration challenges highlighted). As there appear to be two possible solutions both will be described both options will be described. Ideally the team will work through the two options and resolve this to a single proposal. This work will not be carried out in detail until operator confirmation of the applicability of the cases has been received.]

# Fulfilment – Service construction context

[Editor’s note: This section will provide details on another context and will be laid out in the same flow as the Assurance context. It may be considered appropriate to move this into a separate document.]

# Fulfilment – Engineering works context

[Editor’s note: This section will provide details on another context and will be laid out in the same flow as the Assurance context. It may be considered appropriate to move this into a separate document.]

.

# Complete model proposal

[Editor’s note: This section will provide the accumulation of the changes from the three contexts discussed above. If the context are moved into separate documents this section may be yet another document or may be deemed unnecessary.].

# Appendix A:

As part of a Network Operations Center (NOC)