# 3GPP TSG SA WG5 (Telecom Management) Meeting #102	S5-154195

# 24-28 August 2015, Beijing (China)	revision of S5-153abc

# Source:	Huawei

# Title:	Add usecase of fast service deployment using mobile networks that inlucde virtualized network functions

# Document for:	Approval

Agenda Item:	6.4.4.1

# 1	Decision/action requested

The group is asked to discuss and agree on the use case.

# 2	References

# 3	Rationale

This document proposes a use case of fast service deployment using mobile networks that include virtualized network functions.

# 4	Detailed proposal

### 5.4.4 LCM use cases

#### 5.4.4.1 Use Case of Fast Network Service Deployment

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To make fast service deployment using mobile networks that inlucde virtualized network functions |  |
| Actors and Roles | NMS makes decision that a new network service needs to be deployed and sends a request to NFVO. NFVO manages NS instantiation based on information included in the request. |  |
| Telecom resources | Network Management System NFVO |  |
| Assumptions |  |  |
| Pre-conditions |  |  |
| Begins when | NMS decides that a new NS is to be instantiated |  |
| Step 1 (M) | NMS decides the type and capacity of a network service, and sets NS instantiation request with proper parameters. NMS should choose a predefined NSD or design a NSD based on the real deployment scenario. The existing VNF instances could also be used by the new NS. |  |
| Step 2 (M) | NMS sends NS instantiation request to NFVO. |  |
| Step 3 (M) | NFVO instantiates the NS based on NS instantiation request. |  |
| Ends when (*) | NMS receives response from NFVO that a new NS is instantiated. |  |
| Exceptions | Virutalized resource is not enough for instantiation of the NS. |  |
| Post-conditions | A new NS is ready for service. |  |
| Traceability (*) |  |  |