# 3GPP TSG SA WG5 (Telecom Management) Meeting #111Bis	S5A-170143

# 13-17 February 2017, Munich (Germany)	revision of S5A-17xabc

# Source:	Huawei

# Title:	Adding Use Case and Requirement for changing the location of network functions within a network slice instance

# Document for:	Approval

Agenda Item:	4.2

# 1	Decision/action requested

Add this use case to TR 28.801.

# 2	References

[1]	3GPP TR 28.801: "Study on management and orchestration of network slicing for next generation network v0.4.0".

# 3	Rationale

In this contribution we propose to add a use case of migrating network functions of a network slice instance amongst different locations. We also propose to add the corresponding requirements.

# 4	Detailed proposal

It is proposed to make the following changes to TR 28.801.

Start of CHANGES

## 5.X	Use case for changing the location of network functions within a network slice instance

## 5.X.1	Pre-conditions

The operator determines that network functions of network slice instances need to be moved to a different location.

5.X.2       Description

After determining that a network function needs to be moved to a new location, the operator:

- determines the destination location of the network function;

- moves the network function to the destination location, guaranteeing that any changes in the addressing of the network function are properly handled;

- ensures that the migration of the network function from source to destination location happens with minimal disruption of the service provided by the network function.

5.X.3       Post-conditions

Network functions of network slice instances can be migrated according to the operator’s decision.

Next Changes

## 6.3	Requirements for network slice lifecycle management

REQ-NSL_LCM-CON-1	The network slice management function shall be able to create a network slice instance which includes CN part and RAN part.

REQ-NSM_LCM-CON-2	The operator shall be able to create network slice instances which may contain some functions that are common to multiple network slice instances and some that are specific to a network slice instance.

REQ-NSM_LCM-CON-3	The network slice management function shall be able to support the following capabilities when creating a new network slice instance:

- Creation of new common CN network functions.

- Creation of new slice specific CN network functions.

- Creation of new shared AN.

REQ-NSM_LCM-CON-4	The operator shall be able to modify a network slice instance

REQ-NSM_LCM-CON-5	The network slice management function shall be able to support the following capabilities for modifying a network slice instance:

- Instantiate a new CN network function.

- Update an existing CN network function.

- Add an existing CN network function into the network slice instance.

- Take out an existing CN network function from the network slice instance.

REQ-NSM_LCM-CON-6	The operator shall be able to manage the network slice instance lifecycle.

REQ-NSM_LCM-CON-7	The network slice management function shall be able to support the following capabilities:

- Creation of network slice instances.

- Activation of network slice instances.

- Modification of network slice instances.

- Deactivation of network slice instances.

- Termination of network slice instances.

REQ-NSM_LCM-CON-8	The operator shall be able to terminate a network slice instance.

REQ-NSM_LCM-CON-9 The network slice management function should be able to support the following capability when terminating a network slice instance for slice specific CN functions:

- Termination of the CN network function which is not in use any longer

- Take out the specific CN function from the slice instance.

REQ-NSM_LCM-CON-10 The network slice management function shall have the capability to determine to reuse an existing network slice instance or create a new network slice instance according to customer service requirements.

REQ-NSM_LCM-CON-11 The network slice management function shall be able to derive the CN part-related requirements and RAN part-related requirements from customer requirements.

REQ-NSM_LCM-CON-12 The network slice management function shall be able to support the following capabilities for changing capacity of a network slice instance:

- Change the capacity of an existing CN network function.

- Create a new CN network function and add into the network slice instance.

- Add an existing CN network function into the network slice instance.

- Take out an existing CN network function from the network slice instance.

REQ-NSM_LCM-CON-13 An operator shall be able to create a network slice subnet instance based on the request of another operator.

REQ-NSM_LCM-CON-13	The operator shall be able to migrate a network function of a network slice instance amongst different locations.

End of CHANGES