# 3GPP TSG SA WG5 (Telecom Management) Meeting #111Bis	S5A-170188

# 13-17 February 2017, Munich (Germany)	revision of S5A-170140

# Source:	Huawei

# Title:	Add requirement for multi-operator scenario

# Document for:	Approval

Agenda Item:	4.2

# 1	Decision/action requested

Add this use case to TR 28.801.

# 2	References

[1]	Draft TR 28.801: "Study on management and orchestration of network slicing for next generation network v0.4.0".

# 3	Rationale

The Use Case “Creating an end-to-end network slice instance across multiple operators” is specified in Section 5.19 of TR28.801. The use case proposes the following

----------Extract from TR28.801

- When requested, Operator B provides management data (e.g., performance data) to Operator A for the network slice subnet instance hosted by operator B and belonging to operator A’s end-to-end network slice instance

-----------End of Extract--------------------------

The requirement for the same is proposed to be added to TR 28.801

# 4	Detailed proposal

It is proposed to make the following changes to TR 28.801.

Start of CHANGES

## 6.8	Monitor Performance related requirements

REQ-NWS_PM-CON-01	The network slice management function shall support creation and management of measurement jobs for the network functions in a network slice subnet.

REQ-NWS_PM-CON-03	The network slice management function shall support a method to indicate the network slice instance being measured when creating and managing measurement jobs for the network functions.

REQ-NWS_PM-CON-04	The network slice management function shall support reception and handling of the performance measurement data from network slice instances

REQ-NWS_PM-CON-05	The network slice management function shall provide a method for monitoring and collection of performance measurement data from the network slice subnet instance aggregated per QoS level, aggregated per service supported by the network slice instance and aggregated per network slice instances.

REQ-NWS_PM-CON-X	An operator’s management system shall be able to report agreed upon management data (e.g., performance data) of a network slice subnet instance to another operator’s management system  that requested the creation of the network slice subnet instance.

Note: See clause 5.19 for the related use case.

End of CHANGES