# 3GPP TSG SA WG5 (Telecom Management) Meeting #116	S5-176204

# 27 November – 1 December 2017, Reno, Nevada, USA	revision of S5-17xabc

# Source:	Huawei, China Unicom

# Title:	pCR 28.531 Add use case for network slice preparation

# Document for:	Approval

Agenda Item:	6.4.4

# 1	Decision/action requested

Discuss and approve the text proposal.

# 2	References

[1]	 3GPP TS 28.530: "Telecommunication management; Management of network slicing in mobile networks; Concept, use cases and requirements v0.3.0".

[2]	3GPP TS 28.531: "Telecommunication management; Provision of network slicing for 5G network and services v0.1.0".

# 3	Rationale

In 3GPP TS 28.530 [1], the network slice provision begins with network slice preparation phase.

------------------------------------------ Extract from 3GPP TR 28.530 [1] ----------------------------------------------------------

### 4.1.1	Introduction

This clause describes management aspects of a Network Slice Instance, which can be described by the four phases shown in Figure 4.1.1.1, the phases are:

- Preparation

- Commissioning

- Operation

- Decommissioning

Figure 4.1.1.1: Management aspects of network slice instance

--------------------------------------------  End of the extract ---------------------------------------------------------------------------

Therefore, this contribution proposes to add the network slice preparation use case to 3GPP TS 28.531 [2] for completeness.

# 4	Detailed proposal

It is proposed to make the following changes to TS 28.531.

Start of changes

# 5. Specification level requirements

  - 5.1 Use cases

# 5.1.X Use case for network slice preparation

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To prepare a network slice based on the network slice request |  |
| Actors and Roles | NOP-A providing the slice NOP-B receiving the slice |  |
| Telecom resources | 3GPP management system |  |
| Assumptions | NOP-A has received a network slice provisioning request from NOP-B. |  |
| Pre-conditions | Certain information related to NSSMF capability and associated non-3GPP part capabilities are available to NSMF. 3GPP management system of NOP-A derives network requirements based on the request of NOP-B. |  |
| Begins when | NSMF of NOP-A receives network slice requirements. |  |
| Step 1 (O) | If the available information is not sufficient to perform feasibility check, NSMF request more information from NSSMFs and management entities of non-3GPP parts. |  |
| Step 2 (M) | NSMF fulfils feasibility check. NSMF prepares feasibility report, which may include solutions for modified network slice requirements. NSMF may interact with other management systems for initial network environment preparation, e.g., reservation of resources, and providing new requirements. |  |
| Step 3 (M) | The 3GPP management system of NOP-A negotiates with the NOP-B based on the feasibility report. Network slice requirements may be amended accordingly. If a feasible solution could not be agreed upon, reservations of resources due to initial network environment preparation are cancelled, and a response to NOP-B is provided with the reasons, and skip to “Exceptions”. |  |
| Step 4 (M) | 3GPP management system prepares the network environment and other necessary preparations required to be done before the creation of an NSI. |  |
| Step 5 (M) | A new NST is designed, or one of the existing NSTs in the inventory is used by assessing it based on the network requirements. This step may include preparation and creation of network slice subnets, or selection of existing NSSTs. |  |
| Step 6 (M) | NSTs and their constituents are on-boarded. An acknowledgement is provided to indicate the completion of the on-boarding phase. |  |
| Ends when | NST design, on-boarding and network environment preparation are completed. |  |
| Exceptions | One of the mandatory steps fails. |  |
| Post-conditions | The network slice is ready for creation. |  |
| Traceability | REQ-NSM_NSMF-FUN-Y, REQ-NSM_NSMF-FUN-YY |  |

End changes