3GPP TSG-SA5 Meeting #125 	S5-193217

# Newport Beach, USA,  8 – 12 April 2019										Revision of S5-19xxxx

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0080 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add measurements related to intra gNB Handover |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_SLICE_ePA |  |  |  |  |  | Date: |  |  | 2019-03-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Complement the Handover measurements so that it becomes possible to measure all types of intra RAT handovers for NR and create a KPI for Handover Success Rate |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add measurements related to intra gNB handover preparation and handover execution. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It will not be possible to monitor the intra gNB handovers and there will be no possibility to create an overall KPI to monitor the Handover success rate. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.6.x (new), 5.1.1.6.x.1 (new), 5.1.1.6.x.2 (new), 5.1.1.6.x.3 (new), 5.1.1.6.x.4 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st change

##### 5.1.1.6.x	Intra-gNB handovers

###### 5.1.1.6.x.1	Number of requested handover preparations

a)	This measurement provides the number of outgoing intra-gNB handover preparations requested by the source NRCellCU.

b)	CC.

c)	On reception of a Measurement Report from UE (see 3GPP TS 38.331[19]) indicating an intra-gNB cell fulfilling the criteria, if the decision is taken to ask target cell for resources. For split gNB deployment the measurement is triggered when gNB-CUCP is sending UE CONTEXT SETUP REQUEST message (see 3GPP TS 38.473 [6]) to gNB-DU to initiate an intra-gNB handover.

d)	A single integer value.

e)	MM.HoPrepIntraReq.

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i) 	One usage of this performance measurements is for performance assurance.

###### 5.1.1.6.x.2	Number of successful handover preparations

a)	This measurement provides the number of successful intra-gNB handover preparations received by the source NRCellCU.

b)	CC

c)	Stepped when target cell informs source cell that UE resources has been reserved following a handover preparation request. For split gNB deployment the measurement is triggered when gNB-CUCP receives UE CONTEXT SETUP RESPONSE message (see 3GPP TS 38.473 [6]) from gNB-DU to initiate a successful intra-gNB handover.

d)	A single integer value.

e)	MM.HoPrepIntraSucc.

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i) 	One usage of this performance measurements is for performance assurance.

###### 5.1.1.6.x.3	Number of requested handover executions

a)	This measurement provides the number of outgoing intra-gNB handover executions requested by the source NRCellCU.

b)	CC.

c)	On transmission of RRC ConnectionReconfiguration message to the UE triggering the handover from the source NRCellCU to the target NRCellCU, indicating the attempt of an outgoing intra-gNB handover (see 3GPP TS 38.331 [20]).

d)	A single integer value.

e)	MM.HoExeIntraReq.

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i) 	One usage of this performance measurements is for performance assurance.

###### 5.1.1.6.x.4	Number of successful handover executions

a)	This measurement provides the number of successful intra-gNB handover executions received by the source NRCellCU.

b)	CC

c)	On reception of RRC ConnectionReconfigurationComplete message from the UE to the target NRCellCU indicating a successful intra-gNB handover (see 3GPP TS 38.331 [20]).

d)	A single integer value.

e)	MM.HoExeIntraSucc.

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i) 	One usage of this performance measurements is for performance assurance.

End of changes