## 5.3	Communication service instances realized in a single or multiple network slice instance

### 5.3.1	Description

A CSP offers a portfolio of communication services to the market (CSC). CSC selects a suitable product and orders communication service by inputting some service requirements e.g. SLA requirements information.

The CSP receives a request from a customer to realize a communication service instance:

- 	The CSP checks the communication service catalogue to find communication services with the communication service description which matches the request. The CSP performs feasibility check, i.e., checking the attainable service quality from both resource and service aspects.

- 	The CSP translates the service SLA into single or multiple parameterized E2E network slice template information. In order to achieve this purpose, interactions between the CSP and NOP or some manually network design and planning process are needed. The CSP provides the parameterized NST(s) to the NOP for network resources allocation. The NOP checks the network slice inventory to find a network slice instance(s) which matches the request.

- 	The network slice instance(s) to be used will be checked whether there is sufficient capacity available to support the new communication service instance. If there is enough capacity available, the communication service instance will be assigned to the network slice instance(s).  The mapping between the communication service instance and the network slice instance(s) will be maintained by both the CSP and NOP.

The communication service instance has been realized.

### 5.3.2	Potential requirements

REQ-CSM_LCM-CON-01 The 3GPP management system shall provide a management service, which offers the capability to receive and process a request to realize a communication service instance.

REQ-CSM_LCM-CON-02 The 3GPP management system shall provide management service, which offers the capability to ensure that the list of realized communication service instances are up to date.

REQ-CSM_LCM-CON-03 The 3GPP management system shall provide a management service, which offers the capability to find a communication service description based on a request.

REQ-CSM_LCM-CON-04 The 3GPP management system shall provide a management service, which offers the capability to assign a communication service instance to an existing network slice instance.

REQ-CSM_LCM-CON-04 The 3GPP management system shall provide a management service, which offers the capability to assign a communication service instance to multiple network slice instances.