3GPP TSG-SA5 Meeting #125Adhoc 	S5-194111

# Sapporo, Japan 25-28 June 2019												Revision of S5-19xxxx

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.291 | CR | 0076 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add Offline only charging service API data model |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | OFSBI_CH |  |  |  |  |  | Date: |  |  | 2019-06-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 5G Data Connectivity Specified Data Type of data model for offline only charging service API is not addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 5G data connectivity specified data type of definitions of offline only charging service API are added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5G data connectivity specified data type of offline only charging service API is not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.5.x.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st change to TS 32.291

6.2.5.2.x	5G Data Connectivity Specified Data Type

6.2.5.2.x.1	Type ChargingDataRequest

This clause is additional attributes of the type ChargingDataRequest defined in clause 6.2.5.2.1.1 for 5G data connectivity charging described in 3GPP TS 32.255[30].

Table 6.x.5.2.2.1-1: 5G Data Connectivity Specified attribute of type ChargingDataRequest

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| pDUSessionChargingInformation | PDUSessionChargingInformation | OM | 0..1 | This field holds the 5G data connectivity specific information. |  |
| roamingQBCInformation | RoamingQBCInformation | OM | 0..1 | This field holds the 5G data connectivity specific information roaming QBC. |  |

6.x.5.2.x.2	Type ChargingDataResponse

This clause is additional attributes of the type ChargingDataResponse defined in clause 6.x.5.2.1.2 for 5G data connectivity charging described in 3GPP TS 32.255[30].

Table 6.x.5.2.2.2-1: 5G Data Connectivity Specified attribute of type ChargingDataResponse

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| pDUSessionChargingInformation | PDUSessionChargingInformation | OM | 0..1 | This field holds the 5G data connectivity specific information. |  |
| roamingQBCInformation | RoamingQBCInformation | OM | 0..1 | This field holds the 5G data connectivity specific information roaming QBC. |  |

6.2.5.2.x.3	Type MultipleUnitUsage

This clause is additional attributes of the type MultipleUnitUsage defined in clause 6.2.5.2.1.3 for 5G data connectivity charging described in 3GPP TS 32.255[30].

Table 6.x.5.2.2.3-1: 5G Data Connectivity Specified attribute of type MultipleUnitUsage

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| uPFID | NfInstanceId | OC | 0..1 | identifier of UPF |  |

6.2.5.2.x.4	Type UsedUnitContainer

This clause is additional portion of the type UsedUnitContainer defined in clause 6.2.5.2.1.4 for 5G data connectivity charging described in 3GPP TS 32.255[30].

Table 6.x.5.2.2.5-1: 5G Data Connectivity Specified portion of type UsedUnitContainer

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| pDUContainerInformation | PDUContainerInformation | OC | 0..1 | the 5G data connectivity specific information |  |

6.2.5.2.x.5	Other data types

Followings 5G data connectivity specified data types for offline only charging are same as 5G data connectivity specified data types for converged charging specified in clause 6.1.6.2.2 of this specification:

- Definition of type PDUSessionChargingInformation;

- Definition of type UserInformation;

- Definition of type PDUSessionInformation;

- Definition of type PDUContainerInformation;

- Definition of type NetworkSlicingInfo;

- Definition of type ServingNetworkFunctionID;

- Definition of type RoamingQBCInformation;

- Definition of type MultipleQFIcontainer;

- Definition of type RoamingChargingProfile;

- Definition of type QFIContainerInformation;

- Definition of type RANSecondaryRATUsageReport;

- Definition of type QosFlowsUsageReport.

End of change